3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204180r01

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Update for Solution#3: User plane location capability transfer and positioning via user plane

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3/ Rel-18

Abstract: Update solution#3 to solve ENs and cover UE mobility scenarios.

- Introduction/Discussion

1) This contribution updates solution#3 to solve following ENs:

Editor's Note: Further details of LPP transfer via User Plane are FFS.

Editor’s Note: It is for FFS how LMF determines to choose the user plan method.

This solution integrates with LMF with the user plane function and generally regards the LMF as a 5GC NF supporting user plane positioning functionality. We assume LMFs in a PLMN supporting user plane positioning are assigned with a dedicated DNN/S-NSSAI.

The transport protocol and security aspects of LPP transfer via user plane between UE and LMF can re-use the solution#1 and subject to SA3 work.

For how LMF determines to choose the user plane for positioning, more information (e,g., DNNs/S-NSSAI of established PDU session, location QoS, service type, Client Type, Deferred location type, etc.) is added as factors. Besides, a NOTE is added to clarify it is based on the local configuration of LMF to determine which positioning method will be chosen for the user plane.

2) LMF change procedure is provided to cover UE mobility during a deferred 5GC-MT-LR procedure via user plane for periodic, or triggered location events, which is similar with the existing LMF change procedures defined for control plane method.

For non-edge scenarios, with the assumption that all LMFs are accessible by any PSA UPF in the network, the PSA UPF relocation may not trigger the LMF change procedure.

For edge scenarios, with the ULCL and Local PSA relocation, the LMF change is needed when an old LMF cannot be connected by the new L-PSA. When UE finds that the user plane connection is broken, it shall report its event to AMF through control plane, then AMF initiates the LMF change procedure. AMF shall select a new LMF based on the location information of target UE. The user plane connection will be re-established with the information from new LMF.

With the mobility of UE and UPF relocation, the user plane path could be not optimal or the user plane could need re-establishment, whether and how AMF triggers LMF change procedure for path optimization or to guarantee user plane service continuity is for FFS.

3) Some texts of the solution are also revised to make it clearer.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-71.

# * * * * First change * * * *

6.3	Solution #3: User plane location capability transfer and positioning via user plane

6.3.1	Introduction

This solution addresses Key Issue #1 on positioning via user plane transmission.

In this solution, capability transmission of LPP is updated to cover 'user plane location capability' indication. If network determines to use user plane for positioning, the network should send the information (e.g. address information) of the LMF supporting user plane positioning functionality to UE to enable the user plan connection.

6.3.2	Functional Description

In this solution, we assume LMF is integrated with a user plane location function, to which UE needs to establish a user plane connection if user plane positioning is to be used.

Before a UE establishes the user plane connection, it is required to send user plane location capability to the network so that the network knows that the UE supports positioning via user plane then can determine to use user plane for positioning. Thus, the existing capability transfer should cover the 'user plane location capability' indication.

During the MO or MT positioning procedure, after the network determines to use user plane positioning, the network should indicate the UE to use user plane for positioning with the address information of LMF. With the address information, UE can establish a secure connection with the LMF, and via this secure connection, position messages can be transferred between UE and the LMF. The established user plane connection may be reused for subsequent user plane position messages transmission trigged by UE or LMF, e.g. if UE does not move to a new place, does not have new QoS requirements and uses the same positioning method via the user plane.

6.3.3	Procedures

6.3.3.1	User plane location capability transfer

During the MO-LR or MT-LR procedures, LMF may request the UE to provide the user plane location capability. Figure 6.3.3.1-1 presents the capability transfer procedure, the ProvideCapabilities message described in TS 37.355 [15] should support to carry 'user plane location capability' indication.

Figure 6.3.3.1-1: Capability transfer

1.	AMF sends Nlmf_Location_DetermineLocation Request to the LMF.

2-3. The LMF invokes the Namf_Communication_N1N2MessageTransfer service operation towards the AMF to request the transfer of a Downlink (DL) Positioning message to the UE. The Downlink Positioning message queries for the UE capabilities if the UE Positioning Capability is not received from AMF. The AMF forwards the Downlink Positioning message to the UE in a DL NAS TRANSPORT message.

4-5. UE provide its 'user plane location capability' indication in an Uplink Positioning message included in a NAS TRANSPORT message, if it supports user plane positioning method.

6.3.3.2 	5GC-MT-LR Procedure via user plane

Figure 6.3.3.2-1: 5GC-MT-LR Procedure via user plane

1~10.	Steps 1-10 of figure 6.1.2-1 in TS 23.273 [5].

11.	The AMF invokes the Nlmf_Location_DetermineLocation service operation towards the LMF to request the current location of the UE.

12-13. [Conditional] Step 2-5 of clause 6.3.3.1 for capability transfer, if LMF has not obtained the capability of UE.

14.	Based on the ‘user plane location capability’ indication of the UE, the positioning methods and modes the UE supports, the load of the control plane, service type, Client Type, deferred location type, required location QoS received in step 11, LMF decides whether to continue the position procedure via user plane. Step 15-19 is skipped if there is already user plane connection context of the target UE in LMF.

15.	[Conditional] If LMF decides to utilize user plane and there is no established PDU session supporting user plane positioning for the UE, LMF sends a user plane positioning information to AMF to indicate UE to use user plane positioning. The user plane positioning information includes the FQDN or IP address of the LMF, the dedicated S-NSSAI and DNN for the PDU session used for user plane positioning and other security related information. Also, the LMF may require the UE to report the IP address of the PDU session used for user plane positioning. If UE supports more than one positioning methods, LMF may also indicate UE which type of positioning methods should use the user plane connection, while others may still use the control plane (e.g. user plane for sensor method and control plane for NR-based positioning method). The user plane positioning information transfer can be based on NAS signalling or enhanced LPP.

NOTE: LMF can select different positioning methods (e.g., motion sensor-based method) for the user plane and it is based on implementation and local configuration to determine which positioning method to use on user plane.

Editor's note: The security mechanism and security information provided by LMF to UE is subject to SA3 WG’s work.

16.	[Conditional] When AMF receives the user plane information from LMF in step 15, AMF sends it  to UE via a DL NAS TRANSPORT message.

17.	[Conditional] The UE starts to establish a secure connection with LMF. If UE finds there is no suitable existing PDU session for user plane positioning, UE should establish a PDU session with the S-NSSAI and DNN received from the positioning information.

18.	[Conditional] If the suitable PDU session exists, UE sends an acknowledgement to LMF via a UL NAS TRANSPORT message, to confirm that user plane position is chosen, and the IP address of the PDU session may also be included in the acknowledgement. If there is no suitable existing PDU session and the PDU session establishment failed in step 17, a failure indication should be included in the acknowledgement message.

19.	[Conditional] AMF sends the acknowledgement received in step 18 to the LMF via Namf_N1messageNotify service.

20.	UE establishes a connection with LMF, and then LPP message can be transferred between UE and LMF for UE based positioning, UE assisted positioning and delivery of assistance data. If the user plane secure connection has been established, LMF directly sends LPP messages to UE. The message flow is described in clause 6.3.3.4.

21~23.	Steps 9~11 of figure 6.1.1-1 in TS 23.273 [5].

6.3.3.3	5GC-MO-LR Procedure via user plane

For MO-LR procedure, the user plane location capability indication may be included in the MO-LR request message, in this case there is no need for LMF to require UE to report its user plane capability. After receiving the Nlmf_Location_DetermineLocation message, LMF determines whether to utilize user plane for UE positioning.

Figure 6.3.3.2-2 presents the 5GC-MO-LR procedure via user plane, of which the principle is similar with 5GC-MT-LR Procedure via user plane. The difference is, in step 4, the user plane location capability indication can be carried by the Nlmf_Location_DetermineLocation message.

If the UE has received DNNs/S-NSSAIs used for user plane positioning and established corresponding PDU sessions from previous location procedure, it may directly interact with LMF via the user plane connection for MO-LR, if the requested location service type is either location estimate of the UE or location assistance data. If the MO-LR requesting location transfer to an LCS client or AF, the UE still should send the MO-LR request through AMF, but step 5~9 in figure 6.3.3.2-2 can be skipped after LMF determined to utilize user plane.

Figure 6.3.3.2-2: 5GC-MO-LR Procedure via user plane if there is no established user plane secure connection between UE and LMF

1-4. Similar with Step 1-4 in figure 6.2-1 of clause 6.2 of TS 23.273[5]. The difference is that the ‘user plane location capability’ indication is carried by the Nlmf_Location_DetermineLocation message in step 4.

4b-13. Same with step 15-23 in clause 6.3.3.1.

6.3.3.4	LMF Change Procedure with user plane positioning

Mobility of the target UE may lead to a change of LMF (e.g., with the relocation of UPF and the old LMF cannot be connected by the new UPF). When UE finds out that the previous user plane connection is broken, it reports the event through NAS Transport, then AMF can select a new LMF for it based on UE location information and LMF serving area.

Fig 6.3.3.3-3. 5GC-MO-LR Procedure via user plane

1a. The UE performs a mobility registration procedure as described in TS 23.502 clause 4.2.2.2. PSA UPF or L-UPF may has been relocated before this step or may be relocated after this step.

1b. NR-RAN performs a location reporting procedure as described in TS 23.502 clause 4.10.

2. When the UE find the user plane connection is broken, it should send a NAS Transport message include LMF1 ID, while Event Report may be included.3-5. Step 3-5 of Figure 6.4-1 in TS 23.273.

6-7. If user plane positioning is used, LMF1 should forward User Plane Positioning Context to LMF2, user plane security context, the address of UE, DNN and S-NSSAI are included in the context.

- If LMF2 also supports user plane positioning, and LMF2 decides to continue using user plane for positioning, it sends user plane positioning information to the UE. Otherwise, control plane is used and the user plane positioning information should not be included in the message.

- AMF send the NAS Transport message including the user plane positioning information to UE.

- UE and LMF performs LPP message transfer via user plane as described in clause 6.3.3.4.

Editor’s Note: With the mobility of UE and UPF relocation, the user plane path could be not optimal or the user plane could need re-establishment, whether and how AMF triggers LMF change procedure for path optimization or to guarantee user plane service continuity is for FFS.

6.3.3.4	LPP transfer via user plane

This procedure presents the LPP Message transfer between UE and LMF via user plane. The LMF interacts with the UE in order to exchange location information applicable to UE assisted and UE based position methods.

Fig 6.3.3.4-4. UE assisted and UE based positioning via user plane

1. Before the LPP message transfer, UE and LMF should has established a secure connection.

2. LMF send LPP Message to UE via DL UP LPP TRANSFER, which may request location information from the UE, provide assistance data to the UE or query for the UE capabilities.

3. After receiving the downlink LPP message, the UE may store assistance data provided by LMF and/or performs positioning measurements and/or location computation.

4. UE may return location information or returns capabilities to LMF via uplink LPP message.

Editor’s Note: The DL/UL UP LPP TRANSFER could be based on HTTPs. The user plane transport protocol and security aspects for LPP re-use solution#1 and the conclusion of SA3.

6.3.4	Impacts on services, entities, and interfaces

The solution impact the following network functions:

-	UE supports to send user plane location capability to LMF, receives user plane positioning information from LMF, support setup and release of UP connection and supports to perform positioning via user plane location connection;

-	LMF supports user plane location service and supports to transfer the user plane positioning information to UE.

# * * * * End of changes * * * *