3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204208

Elbonia, May 16 – 20, 2022

Source:	OPPO

Title:	KI#1 New solution – Application AI/ML Assistance Service Framework

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_5G_AIMLsys / Rel-18

Abstract: Propose dedicated Application AI/ML Assistance Service Framework which impacts KI#1, 2, 3, 4, 5, 6 & 7.

# 1. Introduction

In Rel-18, the Application Service Provider leverages 5G system to provide intelligent assistance for its Application AI/ML data transmission. The expected service assistance requirements have been captured by 7 key issues as described in TR 23.700-80v02.  It is observed that, many of the Application AI/ML operation may request similar assistance info and operation support from the 5G Core, the intent of this solution is to take into the considerations of the system requirements as identified by the 7 key issues as captured in clause 5, it proposes a common solution framework to assist various Application AI/ML operations which acquire similar assistance info and procedures from 5G core.  In this framework, the similar service requirements and operational behaviours are organized into various Application AI/ML Assistance (AaaML) Service Profiles where each profile defines specific AaaML Service.

The “AaaML Services” are a set of collective extensions to the existing 5G Core Services (e.g. NEF services) and the new 5G Core services which are defined specifically to assist the application layer AI/ML operation. With the support of the AaaML Service Profile information that indicates specific service requirements, an AaaML service can then be served by the existing 5G Core NF (e.g. NEF) and the new 5G Core NF, referred as AaaML NF, which is introduced by this proposal specifically to support the AaaML Service.

An AaaML Service Profile is composed of three main parts of information:

- “objective” of target AaaML operation,

  - e.g. Assuming the objective of this operation is to support “AI/ML member selection for a particular Application AI/ML operation based on per UE’s QoS performance”.  In this case, the AI/ML member selection is the main service, and the per UE’s QoS performance is the sub-service.

- “input” of provisioned service parameter(s),

  - e.g. minimum one way delay, predicted QoS performance within the next 5 minutes

- “output”

  - e.g. list of candidate UEs, event report for the group of UE’s bandwidth consumption

The configuration of an AaaML Service Profile is dependent on the corresponding Application.

Each AaaML Service Profile is identified by the AaaML Profile ID, Application ID, UE ID or Group ID or Any and optionally, the AaaML Sub-Profile ID.   Each of the IDs above has the following purposes:

    - AaaML Profile ID – It identifies the main service of AaaML Service Profile that defines:

      - what are the set of input AaaML service parameters required from the consumer and how they may be translated (e.g. GPSI to SUPI, geographical location into cell, TA, QoS requirements into QoS parameters etc.),

      - what are the target AaaML service information are to be collected from 5G Core (e.g. network analytic info for a given UE or a group or UEs, UE mobility update etc.) and

      - what are the target AaaML operation is to be conducted in 5G Core (e.g. data transmission, service provisioning, performance monitoring, etc.).

    - (Optional) AaaML Sub-Profile ID – It describes the AaaML Service Sub-profile for specifying the additional service requirement(s) for the target AaaML operation (e.g. AI/ML operation UE member selection based on QoS performance, AI/ML data transmission based on Time Of Date etc.).

    - The Application ID – It identifies the Application that require 5G Core assistance for the AI/ML operation.

- UE ID or Group ID or any - The identifier of the target UE or a group of target UE or any UE who is/are involved in the target AaaML service.

Figure 1.1-1:  5GC Application AI/ML Service Assistance Framework

AaaML NF can be co-located with NEF or standalone.   Note that, the standalone AaaML NF configuration can only be allowed when the specific AaaML Service(s) that is served by the AaaML NF as the Producer of the Consumer’s service request, and the service is not based on the extension of the existing 5G Core Service (e.g. NEF Service, NWDAF Service etc.).  Such deployment consideration is necessary in order to ensure the integrity of the existing services in 5G Core when they are extended to support the AaaML services so as not to confuse the service consumer(s) which are trying to communicate with their corresponding serving service producer(s) using the similar service based interfaces (e.g. Nnef_<x>, Nnwdaf_<y>, Npcf_<z>, etc.).  Figure 1.2 below explains the standalone configuration considerations for AaaML.

Figure-1.2: Considerations for Standalone AaaML NF when Leveraging Existing Service Based Interface to support AaaML Services

Further details for the AaaML Service framework are described in the following solution clauses.

# 2. Text Proposal

It is proposed to capture the following changes to TS 23.700-80.  This solution applies to all KIs.

# * * * * First change  * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| X | X | X | X | X | X | X | X |

# * * * * Second change (All New Text) * * * *

## 6.X	Solution #X: Application AI/ML Assistance Services

### 6.X.1	Description

This solution has impact to all KIs.

It is observed that, many of the Application AI/ML operation may request similar assistance info and operation support from the 5G Core, the intent of this solution is to take into the considerations of the system requirements as identified by the 7 key issues as captured in clause 5, it proposes a common solution framework to assist various Application AI/ML operations which acquire similar assistance info and procedures from 5G core.  In this framework, the similar service requirements and operational behaviours are organized into various Application AI/ML Assistance (AaaML) Service Profiles where each profile defines specific AaaML Service.

The “AaaML Services” are a set of collective extensions to the existing 5G Core Services (e.g. NEF services) and the new 5G Core services which are defined specifically to assist the application layer AI/ML operation. With the support of the AaaML Service Profile information that indicates specific service requirements, an AaaML service can then be served by the existing 5G Core NF (e.g. NEF) and the new 5G Core NF, referred as AaaML NF, which is introduced by this proposal specifically to support the AaaML Service.

An AaaML Service Profile is composed of three main parts of information:

- “objective” of target AaaML operation,

  - e.g. Assuming the objective of this operation is to support “AI/ML member selection for a particular Application AI/ML operation based on per UE’s QoS performance”.  In this case, the AI/ML member selection is the main service, and the per UE’s QoS performance is the sub-service.

- “input” of provisioned service parameter(s),

  - e.g. minimum one way delay, predicted QoS performance within the next 5 minutes

- “output”

  - e.g. list of candidate UEs, event report for the group of UE’s bandwidth consumption

The configuration of an AaaML Service Profile is dependent on the corresponding Application.

Each AaaML Service Profile is identified by the AaaML Profile ID, Application ID, UE ID or Group ID or Any and optionally, the AaaML Sub-Profile ID.   Each of the IDs above has the following purposes:

    - AaaML Profile ID – It identifies the main service of AaaML Service Profile that defines:

      - what are the set of input AaaML service parameters required from the consumer and how they may be translated (e.g. GPSI to SUPI, geographical location into cell, TA, QoS requirements into QoS parameters etc.),

      - what are the target AaaML service information are to be collected from 5G Core (e.g. network analytic info for a given UE or a group or UEs, UE mobility update etc.) and

      - what are the target AaaML operation is to be conducted in 5G Core (e.g. data transmission, service provisioning, performance monitoring, etc.).

    - (Optional) AaaML Sub-Profile ID – It describes the AaaML Service Sub-profile for specifying the additional service requirement(s) for the target AaaML operation (e.g. AI/ML operation UE member selection based on QoS performance, AI/ML data transmission based on Time Of Date etc.).

    - The Application ID – It identifies the Application that require 5G Core assistance for the AI/ML operation.

- UE ID or Group ID or any - The identifier of the target UE or a group of target UE or any UE who is/are involved in the target AaaML service.

Figure 6.X.1-1:  5GC Application AI/ML Service Assistance Framework

AaaML NF can be co-located with NEF or standalone.   Note that, the standalone AaaML NF configuration can only be allowed when the specific AaaML Service(s) that is served by the AaaML NF as the Producer of the Consumer’s service request, and the service is not based on the extension of the existing 5G Core Service (e.g. NEF Service, NWDAF Service etc.).  Such deployment consideration is necessary in order to ensure the integrity of the existing services in 5G Core when they are extended to support the AaaML services so as not to confuse the service consumer(s) which are trying to communicate with their corresponding serving service producer(s) using the similar service based interfaces (e.g. Nnef_<x>, Nnwdaf_<y>, Npcf_<z>, etc.).  Figure 6.X.1-2 below explains the standalone configuration considerations for AaaML.

Figure-6.X.1-2: Considerations for Standalone AaaML NF when Leveraging Existing Service Based Interface to support AaaML Services

### 6.X.2	AaaML Service Architecture

This solution introduces new network function, Application AI/ML Assistance Network Function (AaaML NF), as described in 6.X.1 above. The new service based interface that support AaaML specific services is Naaaml.  AaaML NF can be co-located with NEF or be standalone.  For standalone AaaML NF configuration, it is only allowed when AaaML NF is the producer of the first 5G Core service request that is requested by the AaaML AF of which the service is not based on the extensions of the existing 5G Core Service (e.g. NEF Service, NWDAF Service etc.).

Figure 6.X.2-1: AaaML Service Architecture

### 6.X.3	AaaML Services

The AaaML Services are a set of collective extensions to the existing 5G Core Services (e.g. NEF services) and the new 5G Core AaaML Services (i.e. AaaML NF) which are specified to assist the Application AI/ML operation.

In this clause, Table 6.X.3-1 describes the new 5G Core Services specifically defined for assisting the Application AI/ML operation of which the AaaML NF is the producer.

Table 6.X.3-2 presents the AaaML service mappings which describe the corresponding AaaML Service Profile and the corresponding existing/new 5G Core Service procedures for a specific AaaML Service as described in the appropriate clause(s).

Table 6.X.3-1: AaaML services provided by AaaML NF (Producer)

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| NaaaML_NetworkConsent | Request | Request/Response | UE, AF |
|  | Subscribe | Subscribe/Notify | UE, AF |
|  | Unsubscribe |  | UE, AF |
|  | Notify |  | UE, AF |
| NaaaML_CandidateUESelection | Request | Request/Response | AF |
|  | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
| NaaaML_NWDAFAnalyticsCollection | Subscribe | Subscribe/Notify | UE, AF |
|  | Unsubscribe |  | UE, AF |
|  | Notify |  | UE, AF |

Editor’s Note: It is FFS for more AaaML services to be defined.

Table 6.X.3-2: Application AI/ML Assistance Service Descriptions provided by NEF/AaaML NF

| Application AI/ML 5GC Assistance Operation (see Note-1) | AaaML Service Profile (see Note-2) | AaaML Service Sub-profile (Optional) (see Note-3) | Corresponding NEF/AaaML NF Services (see Note-4) | AaaML Service Descriptions (see Note-5) |
| --- | --- | --- | --- | --- |
| Network Consent | AaaML Profile ID: Network Consent | AaaML Sub-Profile ID: (see Editor’s Note-1) | TBD (see Editor’s Note-2) | Network Consent for accessing target UE or Group of UEs related information for a specific application. |
| Network Resource Monitoring | AaaML Profile ID: Network Resource Monitoring | AaaML Sub-Profile ID: (see Editor’s Note-1) | TBD (see Editor’s Note-2) | Network resource performance monitoring (e.g. network load, QoS sustainability etc.) to support application AI/ML model and data distribution, transfer and/or training |
| UE-related Network Performance Monitoring | AaaML Profile ID: UE-related Network Performance Monitoring | AaaML Sub-Profile ID: (see Editor’s Note-1) | TBD (see Editor’s Note-2) | UE-related network performance information (e.g. QoS monitoring, UE Mobility prediction etc.) to assist UE selection for Application AI/ML operation |
| 5G Core Network Exposure to UE | AaaML Profile ID: Network Exposure to UE | AaaML Sub-Profile ID: (see Editor’s Note-1) | TBD (see Editor’s Note-2) | 5GC exposes network information to the UE which is used to facilitate UE’s Application AI/ML operation (e.g. Model Training, Splitting and inference feedback etc.). |
| 5G Core Network Exposure to AI/ML AF | AaaML Profile ID: Network Exposure to AI/ML AF | AaaML Sub-Profile ID: (see Editor’s Note-1) | TBD (see Editor’s Note-2) | 5GC exposes network information to the UE which is used to facilitate Application AI/ML operation (e.g. Model download, Splitting and inference feedback etc.). |
| Candidate UE(s) Selection | AaaML Profile ID: Candidate UE(s) | AaaML Sub-Profile ID: (see Editor’s Note-1) | TBD (see Editor’s Note-2) | 5GC assistance to select candidate UE(s) to support Model Splitting, Training (either for FL or non-FL), inference feedback etc. |
| Application AI/ML Traffic Transport | AaaML Profile ID: Application AI/ML Traffic Transport | AaaML Sub-Profile ID: (see Editor’s Note-1) | TBD (see Editor’s Note-2) | The 5GC data transfer/traffic routing mechanisms to support the transmission of the Application AI/ML traffic(s) between AI/ML endpoints (i.e. UE and AF) |
| NOTE-1: This column is to describe the 5G core services that assist the application layer AI/ML operation. NOTE-2: This column is to describe the AaaML Service Profile ID that maps to the main AaaML Service. The AaaML Service Profile defines (i) what are the set of input AaaML service parameters required from the consumer and how they may be translated (e.g. GPSI to SUPI, geographical location into cell, TA, QoS requirements into QoS parameters etc.), (ii) what are the target AaaML service information are to be collected from 5G Core (e.g. network analytic info for a given UE or a group or UEs, UE mobility update etc.) and (iii) what are the target AaaML operation is to be conducted in 5G Core (e.g. data transmission, service provisioning, performance monitoring, etc.). NOTE-3: This column is to depict the optional AaaML Sub-profile ID that describes the sub-service of the AaaML Service Profile by providing the additional service requirement(s) for the target AaaML operation (e.g. AI/ML operation UE member selection based on QoS performance, AI/ML data transmission based on Time Of Date etc.). NOTE-4: This column is to capture the AaaML service mappings between the AaaML Service Profile and the corresponding existing/new 5G Core service procedures for a specific AaaML Service as described in the appropriate clause(s). NOTE-5: This column is to provide the high-level descriptions of the given AaaML service. |  |  |  |  |

Editor’s Note-1: It is FFS for the list Sub-AaaML Services to be defined for the given AaaML Service when this study is reaching conclusion. For example, the clause n.m,x TS 23.xxx describes the Group QoS Monitoring, is the Sub-AaaML Service of the Network Resource Monitoring of AaaML Service.

Editor’s Note-2: It is FFS to finalize the corresponding NF services to support the given AaaML Service when this study is reaching conclusion.  For example, the clause n.m TS 23.xxx describes the Network Resource Monitoring of AaaML Service.

### 6.X.3	Procedures for AaaML Services

#### 6.X.3.1	AaaML Service Profile support for AaaML Service Procedures

This clause explains how to retrieve the AaaML Service Profile to support the AaaML service procedures.

Figure 6.X.3.1-1: AaaML Service Profile support for AaaML Service Operation

1.& 2. If the AF has not discovered its serving NEF/AaaML NF, it queries NRF to retrieve its serving NEF/AaaML NF that supports its target AaaML Service Profile.

3.  A PDU Session between the UE and the AF may have been established.

4.  AF subscribe or requests the AaaML service by sending NEF service request or AaaML service request with the required parameters and AF Transaction ID, more specifically, AF includes the Application Identifier, UE ID or Group ID, AaaML Profile ID and optionally AaaML Sub-Profile ID.

5.  NEF verifies the authorization of the AF Request.

6. NEF or AaaML NF retrieves the AaaML service profile by providing the Application Identifier, UE ID or Group ID, AaaML Profile ID and optionally AaaML Sub-Profile ID.

NOTE: Whether the AaaML service profile is stored locally to the AaaML NF or in the UDR is to be determined.

7.  By referring to the service parameters and based on the information of the AaaML service profile, NEF or AaaML NF proceeds with the corresponding AaaML service procedures.

8.  Based on the outcome of the AaaML service procedures, NEF or AaaML NF replies to AF with the service response or notification.

### 6.X.4	Impacts on services, entities and interfaces

NRF:

- Recognize the new service parameter, AaaML Profile ID, to support the NEF and AaaML discovery

AF, NEF, AaaML NF:

- Recognizes new service parameters, AaaML Profile ID, to support the AaaML service procedure

- Support the query of the AaaML service profile based on the information of AaaML Profile ID.

# * * * * End of changes * * * *