SA WG2 Meeting #150E (E-meeting)	S2-2204222

May 16 – 20, 2022, Online	(revision of S2-220xxxx)

Source:	Sony

Title:	Solution for KI#4 and 5, Relative QoS within a PDU Set

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: Solution for KI#4 and 5, Relative QoS within a PDU Set

1. Introduction

Assuming that a PDU Set contains the same type of content/importance, then typically for XRM service, a PDU Set needs to be delivered as whole and if some pieces are not delivered in time, then the complete PDU Set should be dropped. I.e. if not all PDU´s within the PDU set are delivered, then the packets delivered successfully, are transmitted in vain.

If the PDU Set contains multiple sub-types as defined in some solutions than the same is valid for each of the sub-types.

Observation:

In order to not waste radio resources, if the initial packets in a PDU set are delivered successfully, the delivery of the remaining of the PDU´s will be increasingly important.

Solution

From a resource/bandwidth point of view the last PDU within a PDU set has higher importance compared to the first PDU within the same PDU Set of the same type.

During normal transmission then NG-RAN calculates the needed modulation or coding rate to achieve a specific BLER (Block Error Rate) to fulfill the desired QoS of the PDU. The existing solutions expect the QoS to be equal for all PDUs with the same importance within the PDU Set.

This solution improves the resource efficiency in 5GS, e.g. in NG-RAN via allowing NG-RAN to be aware of where within the PDU Set it transmit PDUs so NG-RAN could adjust the reliability level within the PDU Set so the reliability is less in the beginning compared to the end in the same PDU Set.

The prioritization/importance level etc. is not expected to be changed so it should not change how the mapping is done to for example different DRB (Data Radio Bearers) etc. The complete PDU Set should keep the same prioritization in time when it is transmitted, it is only the reliability level that should change.

Some PDU Sets may contain data that does not require the whole PDU Set to be dropped if the transmission too late, so it is proposed to define a new per PDU Set QoS parameter. This proposal does not propose to combine PDU-Set Delay Budget (PSDB) as proposed in for example solution#8 with if relative QoS should be used or not.

r01: marked like this

r02: marked like this

r03: marked like this

2. Proposal

It is proposed to agree the following solution to the 3GPP TR 23.700-60 (all new).

* * * Start of change * * *

## 6.x	Solution #X: Relative QoS within a PDU Set

### 6.X.1	Key Issue mapping

This solution is an add-on solution for KI#4 and KI#5 that only addresses the aspect of improving the resource efficiency in NG-RAN within a PDU Sets.

### 6.X.2.1	PDU-Set information

This solution provides NG-RAN with information about where within a PDU Set each PDU belongs to.

This solution utilizes the following:

- PDU Set size

  - As defined in for example solution #8 (Size of the PDU-Set) or solution #20 (PDU Set size)

- PDU-Set Sequence number

  - As defined in for example solution #8 (PDU-Set Sequence number) or solution #20 (PDU sequence number within the PDU Set)

Option#1:

In this option the PDU Set size is consistent with PDU Set maximum size as used in solution#20.

Option#2:

In this option the PDU Set contains payload data and FEC (error corrections) as described in the LS from SA4 (S2-2203658) and when increased reliability is not expected for the full PDU Set. For this option both PDU Set maximum size and PDU Set Size is needed and PDU Set Size is smaller that PDU Set maximum size. This option allows usage of increased reliability of the payload data but not all the FEC data. In this case the size of the part that should use increased reliability is defined in the PDU Set Size while the PDU Set maximum size is the total size of the PDU Set. With this the PDUs belonging to the PDU Set after the PDU Set Size will not use increased reliability.

### 6.X.2.2	New QoS parameter

Relative QoS:

NG-RAN is expected to use PDU Set size and PDU-Set Sequence number to decide a relative reliability level for each PDU within the PDU Set and adjust for example the BLER accordingly. For option#2 NG RAN is also expected to use PDU Set maximum size to decide a relative reliability level.

NOTE: Alignment with RAN is needed to evaluate if for example on/off is sufficient or if RAN prefer any additional information like pattern or how steep gradient the relative QoS is expected.

Editor’s Note: The definition of relative reliability level if FFS.

#### 6.X.3	Impacts on services, entities and interfaces

RAN:

- Perform relative QoS handling within a PDU Set.

NOTE:	The details of RAN behavior is up to RAN WGs.

Editor's note:	Other impacts are FFS.

* * * End of change * * *