3GPP WG SA WG2 Meeting #151E e-meeting	S2-2204242r07

May 16th – 20th 2022,Elbonia	(revision of S2-2xxxxxx)

Source:	China Mobile

Title:	Interim conclusions of KI#1

Document for:	Discussion / Approval

Agenda Item:	9.17

Work Item / Release:	FS_NG_RTC / Rel-18

Abstract of the contribution: This contribution proposes interim conclusions of KI#1.

- Discussion

To accelerate the conclusion of KI#1, it is proposed to discuss and try to agree some interim conclusions to build working assumptions on some aspects to help further contribution merging, evaluation and conclusions.

2.Proposal

It is proposed to capture the following interim conclusions in TR 23.700-87.

* * * * First Change * * * *

# 9	Conclusions

9.1	Interim conclusion of KI#1

The following interim conclusions are agreed for IMS DC architecture:

- The data channels shall be established in the context of an IMS MMTel session. Standalone data channels without accompanying media in an IMS MMTel session are not supported in this release.

- The signalling function and media function supporting Data Channel are logically separated from each other.

- The signalling function supporting Data Channel interacts with IMS AS for call event reports and management of data channel media. Such interaction is based on service based interface principles.

- The signalling function is not involved in SIP/SDP negotiation.

- IMS AS exclusively controls media resources for data channel purposes.

- The DCAR can be integrated in the signalling function supporting Data Channel or deployed standalone. In either case, the interface to DCAR and its functionality is out of scope of 3GPP and hence is not specified.

The following interim conclusions are agreed for IMS DC capability discovery:

- The network discovers the UE support of DC based on existing mechanism as specified in TS 26.114 [6] and TS 24.229 [8].

* * * * End of change * * * *