3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204251

Elbonia, May 16th – 20th, 2022

Source:	Samsung

Title:	KI#7: Sol#12 updates to address EN

Document for:	Approval

Agenda Item:		9.20

Work Item / Release:  FS_Ranging_SL / Rel-18

Abstract: This paper proposes updates to Sol#12 addressing one EN

# 1. Introduction/Discussion

This paper updates Sol#12 of the FS_Ranging_SL study item.

# 2. Text Proposal

It is proposed to capture the following changes into the TR 23.700-86.

# * * * * First change * * * *

### 6.12.3	Procedures

Figure 6.12.3-1: Procedure for ranging application server exposure

Figure 6.12.3-1 shows a procedure with the proposed service exposure framework towards the application server. The description of the procedure is as follows:

0.	The ProSe capabilities of the UEs are registered with the AMF beforehand. This may include the support for location reporting in addition to other ProSe capabilities. Also the Location Privacy Indication (LPI) defined in TR 38.845 [6] for ProSe based localization can be given by the UE's at this stage, again to the AMF.

1.	Ranging/sidelink positioning service request (via NEF): the ranging application server may contact GMLC directly or via NEF to provide a ranging or sidelink positioning operation request to the 5GS. 

The service operations involved in the ranging/sidelink positioning request may reuse and extend the existing NEF/GMLC services (e.g. Nnef_EventExposure_Subscribe, Ngmlc_Location_ProvideLocation) and/or new service operations may be defined if needed. The service operations may include the target and reference UEs, the required Ranging/Sidelink positioning QoS, and a number of events addressing distance and/or direction measurements between the reference and target UEs. The service operations may also indicate threshold(s) for distance and/or direction events. In addition, the service operations may include other event configuration parameter(s) that may be mandatory or optional (e.g. time to trigger, direction orientation, threshold matching direction, measurement frequency, preferred accuracy, privacy preservation).

2.	Setup of location information as per 5GC-MT-LR procedure in TS 23.273 [11], corresponding to steps 2-11 in Figure 6.1.2-1 contained in clause 6.1.2, potentially including new optional inputs as part of the existing service operations accounting for the ranging related events.

The following enhancements of the 5GC-MT-LR procedure in TS 23.273 [11] are required to support ranging/sidelink positioning:

- step 5: an AMF service is consumed by GMLC to request ranging/sidelink positioning. The service operation can be Namf_Location_ProvidePositioningInfo with the same enhancements described above for Ngmlc_Location_ProvideLocation or a new service operation could be defined if needed.

- step 10: an LMF must be selected that supports ranging/sidelink positioning.

- step 11: either the Nlmf_Location service operations should be extended to support ranging/sidelink positioning with the enhancements described for Ngmlc_Location_ProvideLocation service operation or a new service operation should be defined for that purpose.

3.	[OPTIONAL] If not done yet, the LMF checks the ProSe capabilities registered for the UEs in step 0 with the AMF. The ProSe AS or PCF may authorize and/or configure the UEs to provide location reporting as needed, and a connection is established for PC5 communication session. The reference UE (acting similar to a ProSe UE-to-Network Relay as specified in TS 23.304 [4]) may be configured or provisioned with special Relay Service Codes (RSCs) authorized for location reporting. This is provided by the PCF, already provisioned in the ME or configured in the UICC.

4.	The LMF sends a ranging/sidelink positioning request to UE 1.

NOTE 1:	UE 1 is assumed to be in coverage for this solution.

NOTE 2:	UE 1 may act as reference UE or target UE in this solution.

Editor's note:	How the reference UE and target UE are decided is FFS.

5.	UE 1 initiates the the ranging/sidelink-based positioning operation on UE 2.

6.	UE 1 sends the ranging/sidelink positioning measurements and/or results to the LMF.

7.	The LMF forwards the ranging/sidelink positioning results to the AMF.

8.	The AMF forwards the ranging/sidelink positioning results to the GMLC.

9.	The GMLC exposes the ranging/sidelink positioning results to the ranging application server via NEF with the response operation of the service employed in step 1.

### 6.12.4	Impacts on services, entities, and interfaces

GMLC

-	Support for ranging/SL positioning request with corresponding events

LMF, AMF

-	Support for ranging/SL positioning request with corresponding events

-	LMF selection considering support for ranging/SL positioning

-	LMF obtaining the UE’s ProSe capability from AMF

UE

-	Support to send/receive ranging/SL positioning request to/from LMF

-	Support to request ranging/SL positioning service from 5GS

Editor's note:	Further impacts are FFS.

# * * * * End of changes * * * *