3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204265

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI #1, Sol #1: Update of Solution 1 of 23700-47

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract: It is proposed to clarify the inactive assistant information provisioning of solution #1.

1. Discussion

Several ENs were left in SA2#150E meeting.

One EN related to the new term "transmission mode":

Editor’s Note: 	Whether and how the AF can enable or disable the transmission mode for inactive reception is FFS. Whether the new term “transmission mode” can be avoided is FFS.

In fact the purpose is to not define the new term, instead the AF provides the information to the 5GC that whether the inactive reception is allowed for specific multicast MBS service(s).

Other ENs regarding the way of storing the session priority and provisioning of session priority:

Editor’s Note: 	Whether the MBS session priority or UE session priority is stored in MBS data or UE subscription data in the UDR/UDM is FFS.

Editor's note:	How SMF gets the UE session priority within the MBS session is FFS.

The third EN to be addressed in this document is the decision of RRC_INACTIVE reception.

Editor’s Note: 	Whether the decision to receive MBS data in RRC_INACTIVE should be a RAN decision or can be UE preference is FFS.

In fact, the proposal is to let UE provide its preference and then RAN node consider such preference when deciding the RRC_INACTIVE reception.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-47.

# * * * * First change * * * *

## 6.1	Solution #1: Procedures for RRC Inactive MBS data reception

### 6.1.1	Introduction

This solution addresses Key Issue #1.

### 6.1.2	Functional description

It is assumed to reuse the current architecture defined in Rel-17 MBS work (see TS 23.247 [4]). In other words, MB-SMF is used to handle MBS session-level management while SMF performs per-UE MBS session management, e.g. authorization, multicast session information provisioning, managing 5GC Individual MBS traffic delivery.

An AF creating a multicast session should be able to influence the service quality (more UEs vs. higher reliability) talking into consideration the specific needs of the service it offers. Thus the AF can enable or disable the transmission mode for inactive reception. However, the decision whether to apply that transmission mode if it is enabled remains with the RAN nodes.

NOTE:	If the AF provides assistance information to disable reception in RRC-INACTIVE state, the number UEs in a cell that can receive the MBS session will be more limited but the transmission will be more reliable.

Editor’s Note:. Whether such behaviour is intended is FFS.

Procedures in the following clauses focus on the following functionalities:

-	5GC provisioning necessary parameters to NG-RAN node(s).

-	Switching between RRC Connected and RRC Inactive modes.

There are two levels of priority as a part of 5GC-provided parameters, namely:

-	MBS session priority: the MBS session priority denotes the priority level of an MBS session, and the priority level defines the relative importance of an MBS session. This allows the NG-RAN nodes deciding (the members of) which MBS session can be switched to RRC Inactive state, to free up resources of NG-RAN node upon e.g., congestion. NG-RAN node may also use it to decide (the members of) which MBS session can be switched to RRC Connected state, once the resources are regarded as sufficient.

The details of how the 5GC provides MBS session priority to NG-RAN node are further described in clause 6.1.3.2.

Editor’s Note: 	Whether the existing QoS parameters (e.g. ARP, 5QI) of the MBS QoS Flow(s) can be used for the MBS Session priority is FFS.

-	UE session priority: the UE session priority denotes the priority level of a certain UE within a certain MBS session, and the priority level defines the relative importance of a UE for an MBS session. This allows the NG-RAN nodes deciding if the UE of an MBS session can be switched to RRC Inactive state, to free up resources of NG-RAN node upon e.g., congestion. NG-RAN node may also use it to decide if the UE of an MBS session can be switched to RRC Connected state, once the resources are regarded as sufficient.

The details of how the 5GC provides UE session priority to NG-RAN node are further described in clause 6.1.3.2.

Editor’s Note 	Whether and how the NG-RAN use the assistant information will be determined by RAN WGs.

Whether the transmission mode for inactive reception is applied depends on multiple factors and determined by the NG-RAN node:

-	Backward compatibility with UEs supporting Rel-17 MBS but not capable of receiving 5MBS data while in RRC-inactive state. If such UEs that joined a MBS multicast session are in a cell, MBS data need to be transmitted using the Rel-17 transmission mode for RRC-connected reception.

-	UE preferences: UEs could prefer to receive MBS data in RRC inactive state to reduce their battery consumption, or in RRC connected state to increase the service quality.

NOTE:	If the UE indicates reception in RRC-INACTIVE state is not preferred and if NG-RAN follows the UE preference, the number UEs in a cell that can receive the MBS session will be more limited but the transmission will be more reliable.

Editor’s Note:. Whether such behaviour is intended is FFS.

-	MBS session priority.

Editor's note: How MBS Session priority could assist the NG-RAN in determining whether to apply transmission for inactive reception is FFS.

-	UE session priority.

Editor's note: Detailed usage of combining UE priority and MBS session priority requires more study.

-	Whether the transmission for inactive reception is allowed for specific multicast MBS service(s).

-	Ongoing session of the UEs (e.g., UE has other PDU session activated).

Editor's note:	Those assumptions need to be confirmed by RAN WGs.

AF provides the following information, and MB-SMF stores this information as a part of Multicast MBS Session context, during MBS Session Creation procedures defined in 7.1.1.2 or 7.1.1.3 in TS 23.247 [4] and clause 6.1.3.2:

-	MBS session priority.

-	Whether the transmission for inactive reception is allowed for specific multicast MBS service(s).

AF provides the following information, and UDM stores this information as a part of MBS subscription data, during External Parameter Provisioning procedures as defined in clause 6.4.2 of TS 23.247 [4]:

-	UE session priority.

### 6.1.3	Procedures

#### 6.1.3.1	General

NOTE:	The message names in the procedures below are descriptive. It is assumed that the names are updated with corresponding SBI based names where applicable during the normative phase.

#### 6.1.3.2	MBS session creation, multicast session join and session establishment procedure

Figure 6.1.3.2-1: Enhancement to current MBS procedures for session creation and join

0.	When UE registers, it indicates its capability to receive MBS multicast using the transmission mode for RRC inactive. It may also indicate a preference regarding the connectivity state in which it wishes to receive MBS session(s). If UE’s preference changes (e.g. due to UE state change such as power level etc.), then the UE performs registration update.. This information is propagated to NG-RAN via AMF.

1-2. When an AF requests the creation of a multicast MBS session, it indicates whether inactive reception of multicast shall be enabled for that session. AF also provides the MBS session priority to MB-SMF, optionally via NEF or MBSF.

Editor’s Note: 	Whether the existing QoS parameters (e.g. ARP, 5QI) of the MBS QoS Flow(s) can be used for the MBS Session priority is FFS.

3. The AF may also indicate in the service announcement towards the UE whether inactive/idle reception of multicast is enabled.

4-14 The information whether the inactive transmission mode is enabled for an MBS session is propagated from MB-SMF towards NG-RAN, via PDU session and/or via shared delivery of a multicast session.

SMF provides the UE session priority to NG-RAN node: In step 7, SMF includes UE session priority as a part of N2 SM information in Nsmf_PDUSession_UpdateSMContext response to AMF. In step 8, AMF sends the N2 SM information received from SMF to NG-RAN node during the shared tunnel establishment procedure. AF provides the UE session priority, and UDM stores this information as a part of MBS subscription data, during External Parameter Provisioning procedures as defined in clause 6.4.2 of TS 23.247 [4]. During PDU session establishment procedure, SMF fetches UE session priority from UDM.

MB-SMF provides the MBS session priority to NG-RAN node: In step 13, since MB-SMF receives the MBS session priority in step 2, MB-SMF includes MBS session priority in the N2 SM information of Nmbsmf_MBSSession_ContextUpdate response message. And AMF sends N2 MBS Session response message to NG-RAN node in step 14.

15. The NG-RAN decides the transmission mode to apply for the MBS multicast session in a cell.

#### 6.1.3.3	Moving a UE to RRC Inactive state

Figure 6.1.3.3-1: NG-RAN node moves a UE to CM-CONNECTED with RRC Inactive state

0.	5GC provides assistance information of RRC Inactive multicast MBS data reception to NG-RAN node, details see clause 6.1.3.2. UE receives multicast MBS data in CM-CONNECTED mode.

Editor's Note:	Whether/what assistance information is needed is to be coordinated with RAN WGs.  It is ffs if some UEs in the same cell can receive MBS data in RRC Inactive reception mode and other UEs can receive MBS data in Rel-17 reception mode.

1.	RAN determines to move UE in multicast MBS session to RRC Inactive state and the transmission mode to apply for the MBS multicast session in a cell taking the assistance information into consideration.

Editor's note:	Determination of switching to RRC Inactive will be confirmed by the RAN WGs.

2.	NG-RAN node releases the RRC connection and moves the UE to CM-CONNECTED with RRC Inactive state.

Editor's note:	How to release the UEs belongs to multicast MBS session will be determined by RAN WGs.

3.	UE receives multicast MBS data in CM-CONNECTED with RRC Inactive mode.

Editor's note:	RAN WGs will determine the configuration of UE receiving multicast MBS data in RRC Inactive.

#### 6.1.3.4	Moving a UE to RRC-CONNECTED from RRC Inactive state

Figure 6.1.3.4-1: NG-RAN node moves a UE to RRC-CONNECTED state

0.	5GC provides assistance information of RRC Inactive multicast MBS data reception to NG-RAN node, details see clause 6.1.3.2. UE receives multicast MBS data in CM-CONNECTED with RRC Inactive mode.

Editor's note:	Whether/what assistance information is needed is to be coordinated with RAN WGs.

1.	RAN determines to move UE in multicast MBS session to RRC-CONNECTED state based on the assistance information as described in clause 6.1.2.

Editor's note:	Determination of switching to RRC Inactive will be confirmed by the RAN WGs.

2.	NG-RAN node informs related UEs to RRC-CONNECTED state.

Editor's note:	How to inform the related UEs belonging to multicast MBS session will be determined by RAN WGs.

3.	UE receives multicast MBS data in RRC-CONNECTED mode.

Editor's note:	RAN WGs will determine the configuration of UE receiving multicast MBS data in RRC-CONNECTED mode.

### 6.1.4	Impacts on services, entities and interfaces.

Editor's note:	This clause describes impacts to existing services, entities and interfaces.

Functional entities defined in clause 5.3.2 of TS 23.247 [4] is reused.

AF:

-	The AF includes the MBS session priority and information whether the transmission for inactive reception is enabled in the message sent to MB-SMF.

-	The AF includes the UE session priority during External Parameter Provisioning procedure.

MB-SMF:

-	The MB-SMF stores MBS session priority and information whether the transmission mode for inactive reception is enabled as a part of multicast context, and provides MBS session priority and information whether the transmission mode for inactive reception is enabled to the NG-RAN node.

SMF:

-	SMF fetches the UE MBS priority and provide to the NG-RAN node during PDU Session modification/establishment procedure.

NG-RAN:

-	Determine the transmission mode for an MBS session in a cell and which UE can be switched RRC Inactive/Connected mode based on the UE MBS priority and MBS session priority from SMF/MB-SMF, respectively.

UDM:

-	Storing the UE session priority of a certain MBS session.

UE:

-	Indicate capability and preference for multicast reception in RRC inactive state.

# * * * * End of changes * * * *