SA WG2 Meeting #151E	S2-2204341r01

# May 16th – 20th, 2022; Elbonia               		  								(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.682 | CR | 0489 | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

s

| Title: | EPS roaming status for dual registration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MONTE, TEI17 |  |  |  |  |  | Date: |  |  | 2022-04-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| d |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | If the UE is registered over both 3GPP and Non-3GPP Access Type, the UDM includes two instances of Registration status event notifications. The corresponding EPS notification includes only single roaming status indication. This means that the SCS/AS / AF receives different information depending on whether the UE is dual-registered in EPS or 5GS. In addition to this, the EPS Roaming status is ambiguous as it has not been specified which one (3GPP or Non-3GPP) Roaming status should be indicated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The already existing 5GS capability to indicate two Roaming status instances is added also to EPS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Monitoring information received by the SCS/AS and AF is different depending on whether UE is registered to EPS or 5GS and it is not specified to which Access Type the indicated roaming status refers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.6.1.7	Specific Parameters for Monitoring Event: Roaming Status

This monitoring event allows the SCS/AS to query the UE's current roaming status (the serving PLMN and/or whether the UE is in its HPLMN) and to get notified when that status changes. It is based on the HSS being informed of the UE's serving PLMN by the MME according to TS 23.401 [7] and by the SGSN according to TS 23.060 [6].

1.	The SCS/AS sets Monitoring Type to "Roaming Status" prior to sending the Monitoring Request to the SCEF as in step 1 of clause 5.6.1.1. Optionally, it includes the "PLMN Information" parameter to request inclusion of the UE's Serving PLMN ID in the Monitoring Event Report.

2.	The SCEF executes step 2 of clause 5.6.1.1.

3.	The SCEF includes "PLMN Information", if sent in step 1, prior to sending Monitoring Request to the HSS as in step 3 of clause 5.6.1.1.

4. The HSS executes step 4 of clause 5.6.1.1.

5-7.	Steps 5-7 of clause 5.6.1.1 shall not be executed for this Monitoring Event.

8-9.	Steps 8-9 of clause 5.6.1.1 are executed. The Monitoring Event Report for this event is sent in the Monitoring Response message. The Monitoring Event Report indicates whether the UE is presently roaming or not. If PLMN Information was requested in step 1, and the operator policies allow, then the HSS includes the roaming status as specified in clause 5.6.3.6:

-	the HPLMN PLMN-Id if the UE is in the HPLMN and/or

-	the Visited PLMN-Id (see TS 29.272 [31]) if the UE is in the VPLMN.