3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204368r02

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Source:	Huawei, CATT, China Mobile, Futurewei, Hisilicon, Lenovo, KDDI, OPPO, Samsung, ZTE

Title:	KI#4 and #5, Baseline for PDU Set based QoS framework

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM/ Rel-18

Abstract: This paper intends to propose a merged solution for KI#4 & 5 based on the commonalities of current agreed solutions in the TR.

# 1. Introduction

This paper intends to propose a merged solution for KI#4 & 5 based on the commonalities of current agreed solutions in the TR, to save the time of evaluation of each solution independently.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60.

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| #X: Baseline for PDU Set based QoS framework |  |  |  | X | X |  |  |  |  |

# * * * * 2nd change, all new texts * * * *

## 6.X	Solution #X: Baseline for PDU Set based QoS framework

### 6.X.1	Key Issue mapping

This solution aims to resolve Key Issue #4, “PDU Set integrated packet handling” and Key Issue # 5, “Differentiated PDU Set Handling”.

### 6.X.2	Description

This solution provides a framework for the PDU Set based QoS handling. The following principles apply:

PDU Set QoS policies/rules/profiles and PDU Set QoS Flow establishment

-	Dynamic PCC rules and non-dynamic PCC rules are supported.

-	PDU Set QoS requirements provisioning by AF supported (for dynamic PCC).

-	PDU Set QoS policies are provided to SMF by PCF for dynamic PCC rules.

-	RAN receives PDU Set QoS profiles from SMF.

PDU Set identification and marking over the user plane

-	PDU Sets are detected in the UPF.

-	Between AS and UPF, multiple PDU Set marking techniques can be supported (see bullet 2 below).

-	PDU Set identification between AS and UPF can be negotiated over Control Plane (via NEF for non-trusted AF), or based on pre-configuration.

Editor’s Note: PDU Set identification for peer-to-peer scenarios (i.e. between two UEs routed via the 5G CN) is FFS.

-	UPF marks PDU Sets in GTP-U layer via GTP-U header extension. GTP-U marking is independent from and common to different PDU Set markings between AS and UPF.

The following aspects are included:

In the following list, a set of baseline parameters are identified as part of PDU Set QoS framework. Additionally, a set of potential parameters for further study (as proposed by other solutions) is described.

NOTE 1: The baseline set of parameters does not exclude additional parameters to be agreed in the future.

1.	UPF identifies the PDUs belong to a PDU Set and the following information for each PDU Set:

Info for intra-PDU Set handling (i.e. KI#4, PDU Set integrated handling)

Baseline parameters:

- 	PDU Set Sequence number (SN) (solution 7, 8, 9, 11, 12, 14, 19, 20, 21, 22)

-	Start/End PDU of the PDU Set (solution 11, 12, 15, 18, 21, 22)

- 	PDU SN within a PDU Set (solution 11, 20, 22)

- 	Number of PDUs within a PDU Set (solution 9, 20)

Editor’s Note: Whether PDU Set Sequence number can also convey Start/End PDU of a PDU Set instance is FFS.

Info for inter-PDU Set handling (i.e. KI#5, PDU Set differentiated handling)

Baseline parameters:

-	PDU Set importance (solution 7, 11, 14, 15, 18, 19, 22, 24)

Parameters for further study:

-	PDU Set dependency (solution 11, 14, 19, 22, 24)

2.	UPF identifies the above information (listed in bullet #1) based on the following mechanism(s):

Parameters for further study:

Option 1: by matching RTP/SRTP header and payload (RFC 3550/6184/draft-ietf-avtext-framemarking)

(solution 7, 12, 14, 15, 16, 17, 18, 23, 24)

Option 2: new RTP extension header (solution 8)

Option 3: by information provided by the AS in N6 encapsulation header e.g. GTP-U (solution 9, 22)

Option 4: by detection based on traffic characteristics (solution 12, 17, 18, 24)

Option 5: by non-standardized mechanisms UPF implementation (solution 20, 25)

Editor’s Note: What information can be achieved by each option need further discussion.

3.	UPF provides the above PDU Set related information (listed in bullet #1) to the RAN.

For PDU Set importance:

Options for further study:

Option 1: UPF classifies the DL traffics into different QoS Flows based on PDU Set importance (solution 10, 14, 24, 26)

Option 2: UPF classifies the DL traffics into different sub-QoS Flows based on PDU Set importance (solution 17, 18).

Option 3: UPF adds PDU Set importance into GTP-U header (solution 7, 11, 14, 15,17, 18, 19, 22)

For other PDU Set related info (listed in bullet #1):

UPF adds them into GTP-U Header (solution 7, 11, 14, 15, 19, 22)

4.	New QoS parameters for  PDU Set level QoS handling in 5GS:

Baseline parameters:

-	PDU Set Delay Budget (PSDB) (solution 8, 9, 12, 14, 25, 26)

-	PDU Set Error Rate (PSER) (solution 8, 12, 25, 26)

Editor’s Note: The definition of the PDU Set Error Rate is FFS.

-	Whether all PDUs are needed for the usage of PDU Set by application layer (solution 22, 23, 25)

Parameters for further study:

-	Whether to drop a PDU Set in case PSDB is exceeded (solution 8, 22, 24, 25)

5. Information from AF

Baseline parameters on PDU Set QoS requirements:

-	PDU Set handling indication (solution 8, 15, 25) (indicating if PDU Set based handling should be activated to a flow)

NOTE 2: This indication may be implicitly indicated via other PDU Set related information provided by the AF.

-	Whether all PDUs are needed for the usage of PDU Set by application layer (solution 22, 23, 25)

Editor’s Note: Whether more detailed indicators are needed for different PDU Set QoS handling features (e.g. PDU Set integrated handling, QoS handling based on PDU Set importance, PDU Set dropping due to delivery failure of dependent PDU Sets) is FFS.

-	PDU Set Delay Budget (solution 12, 25, 26)

-	PDU Set Error Rate (solution 12, 25, 26)

Baseline parameters on frame identification:

-	Burst periodicity (solution 8, 12, 24, 26)

NOTE 3: The details of RAN behaviour is up to RAN WG.

Editor’s Note: The PDU Set QoS handling for Uplink media services is FFS.

### 6.X.3	Procedures

#### 6.X.3.1 PDU Set based QoS handling

Figure 6.X.3.1-1: High-level Procedure of PDU Set based QoS handling

The PDU Set based QoS control may happens during PDU Session establishment or modification procedures. The process includes the following steps:

1a.	Steps 1-7a of PDU Session Establishment procedure (defined in clause 4.3.2.2.1 of TS 23.502 [3) are performed. A network slice type for XR service can be used for such a PDU Session.

1b.	The AF may send the information described in above bullet#5 of clause 6.X.2 to PCF via Nnef_AFsessionWithQoS_Create request as defined in clause 4.15.6.6 of TS 23.502. AF may also provide these information to the 5GS prior to PDU Session Establishment.

2. 	The PCF generates appropriate PCC rules, which may include PDU Set related QoS parameters listed in bullet #4. The PCF sends the PCC rules to SMF.

NOTE 4: This step is done in step 7b in PDU Session Establishment procedure (defined in clause 4.3.2.2.1 of TS 23.502[3]) or Step 1b in PDU Session Modification procedure (defined in clause 4.3.3.2 of TS 23.502[3]).

If this step is triggered by step 1b, the PCF generates the PCC rules considering the information provided by the AF.

3.	The SMF generates the QoS profiles and N4 rules based on the PCC rules from PCF. SMF sends the N4 rules to UPF and sends the QoS profiles to the RAN node via AMF.

NOTE 5: This step is done via steps 8-15 in PDU Session Establishment procedure (defined in clause 4.3.2.2.1 of TS 23.502[3]) or steps 2-7 in PDU Session Modification procedure (defined in clause 4.3.3.2 of TS 23.502[3]).

Editor’s Note: The extensions of N4 rule and QoS profile to support PDU Set related handling are FFS.

4. The leftover steps of PDU Session Establishment or Modification procedures are performed.

5.	Based on received N4 rules or locally configuration on the UPF, the UPF identifies the info in above bullet #1 via the methods described in bullet#2 of clause 6.X.2, and performs PDU Set based QoS handling according to N4 rule instruction.

6.	UPF sends the PDU Set info to RAN according to above bullet #3.

7.	Based on the PDU Set related info in GTP-U header, RAN performs PDU Set based QoS handling.

NOTE 6: The details of RAN behaviour is up to RAN WG.

### 6.X.4	Impacts on services, entities and interfaces

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

TBD

# * * * * End of changes * * * *