SA WG2 Meeting #151E	S2-2204374r01

# May 16th – 20th, 2022; Elbonia               		          (revision of S2-220)

Source: 	Samsung

Title: 	Subscription of UPF Event Exposure Service

Document for: 	Approval

Agenda Item: 	9.25

Work Item / Release:	FS_UPEAS / Rel-18

Abstract of the contribution: This proposes the solution of Key Issue #2

1	Discussion

It proposes the subscription of UPF Event Exposure Service served by the UPF handling a specific IP Flow over the PDU session. In this solution, we provide how to discover the specific UPF with UE IP address.

2 Proposal

It is proposed the solution for KI#2.

6.X	Solution #X: Subscription of UPF Event Exposure Service

6.X.1	Key Issue mapping

This solution is for KI#2.

6.X.2	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Sub - clause(s) may be added to capture details.

The solution introduces for the AF to subscribe to UPF Event Exposure served by the UPF handling a specific IP Flow over the PDU session. In this solution, we provide how to discover the specific UPF with UE IP address.

6.X.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

#### 6.X.3.1	UPF Event Exposure using NEF

##### 6.x.3.1.1	Procedure of UPF service operations information flow

The procedure is used by the AF/NEF to subscribe event notification to the UPF handling a specific PDU session. Cancelling is done by Nupf_EventExposure_Unsubscribe request identifying the subscription to cancel.

Figure 6.x.3.1.1: Procedure of UPF service operations information flow

1.	In order to subscribe to Event Exposure service on a specific PDU session, the AF sends a request to the NEF with UE IP address.

2.	The NEF sends the request to the BSF.

3.	Based on UE IP address, the BSF can identify the specific UPF and sends the request to it.

4-6. Acknowledgements for each request

##### 6.x.3.1.2	Procedure of UPF information in BSF

The procedure is used for storing UPF information in BSF.

Figure 6.x.3.1.1: Procedure of UPF information in BSF

0.	The UE requests PDU session establishment.

1.	During the PUD session establishment, the SMF sends Npcf_SMPolicyControl_Update Request message to the PCF with UE IP address and UPF ID information.

2.	The PCF sends Nbsf_Management_Register Request message to the BSF with UE IP address and UPF ID information.

### 6.X.4	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.