SA WG2 Meeting #151E	S2-2204401r02

# 16 – 20 May 2022, e-meeting

Source:	Lenovo

Title:	KI#1: New solution to support of DetNet aware nodes in 5GC

Document for:	Approval

Agenda Item:	9.6

Work Item / Release:	FS_Detnet / Rel-18

Abstract of the contribution:

# 1		Discussion

# 2		Proposal

The following solution is proposed.

******************************** First change (all new text)  *******************************

## 6.x	Solution #x: Supporting QoS configuration for DetNet flows in 5GC

### 6.x.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s) which should be explicitly stated. (Sub) clause(s) may be added to capture details.

This solution is related with KI#1.

Given that Detnet aware nodes in 5GS are out of scope of this study one possible deployment option to support integration of Detnet in the 3GPP system as shown in the Figure below.

Figure 6.x.1-1: Support of Detnet in 5GS

In order to support the architecture proposed in Figure 1 the following need to be supported:

-	A UE establishing a PDU session for carrying DetNet traffic.

-	Reporting to a Detnet aware AF, the addresses of the UE and UPF

-	DetNet aware AF receiving the DetNet flow configuration information from DetNet controller based on the Detnet yang profile. DetNet controller takes into account 5GS node information provided by the Detnet aware AF (out of scope of 3GPP).

-	Map the DetNet flow configuration information obtained from DetNet controller into 5GS QoS information (e.g. 5QI, TSC Assistance Information) for a QoS Flow in a corresponding PDU Session established by a UE

### 6.x.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

The UE establish a PDU session for relaying DetNet flows as shown below.

Figure 6.x.2-1: Establishing a PDU session for relaying DetNet flows

The steps are as follows:

1.	The UE is triggered (based on configuration) to establish a PDU session for relaying DetNet flows. .

2.	The UE includes in a PDU session establishment request towards the SMF a requested S-NSSAI, DNN.

3.	The AMF receives the request and selects SMF based on the S-NSSAI/DNN requested

4.	The AMF sends a Create Session Establishment request to the SMF including the SM container received in step 2.

5.	The SMF selects a PCF and establishes an SM Policy Association

6.	The SMF selects a DetNet capable UPF (i.e. a UPF supporting relaying of DetNet flows) based on the S-NSSAI/DNN.

7.	The SMF acknowledges the request to the AMF

8.	SMF initiates an N4 session establishment including rules for the PDU session. During this procedure the UPF may provide DetNet capability information in a transparent container that includes the following information:

-	IP address of the UPF

9.- 10	SMF sends a PDU session accept to the UE.

11.	Based on a subscription from the PCF (via the AF) the SMF provides the IP addresses of the UE and UPF to the PCF

12.	The PCF forwards the information to the DetNet aware AF by invoking an Npcf_PolicyAuthorization_Notify.

13.	The DetNet aware AF learns the IP addresses of the UE and UPF.

The procedure is to configure the 5GS system with appropriate QoS characteristics is shown below.

Figure 6.x.2-2: Configuring 5GS with DetNet QoS characteristics

The procedure is as follows:

1.	The UE establishes a PDU session for DetNet flows as shown in Figure 6.x.2-1

2.	The DetNet controller subscribes via the AF to be informed of 5GS node supporting Detnet traffic in 3GPP system.

Editor’s note: it is FFS what protocol is used for such subscription

3.	The DetNet controller discovers the DetNet end systems that interface with the 5GS For example, the DetNet controller can use a neighbour discovery protocol as per IETF RFC 4861 to discover DetNet end systems located behind the UE and the UPF.

Editor's Note: It is FFS how the DetNet controller (which is a remote entity) can use a neighbour discovery protocol as per IETF RFC 4861 to discover DetNet end systems located behind the UE and the UPF

4.	The DetNet controller provides to a DetNet aware AF the configuration of the DetNet flow by providing the rules in the form of a DetNet yang model as described in draft-ietf-detnet-yang-16 [x]. This procedure is out of scope of 3GPP

5.	The DetNet aware AF derives DetNet specific assistance information including the QoS charactertics from the DetNet configuration information provided by the DetNet controller based on the IETF DetNet Yang profile.

Editor’s note: Based on the agreed assumptions, the DetNet aware AF corresponds to the TSCTSF.

6.	The DetNet aware AF sends a request to an NEF establish an AF session with required QoS including the UE IP address and Detnet assistance information.

Editor’s note: the need for NEF is FFS.

7.	The NEF discovers the PCF serving the UE and sends an Npcf_PolicyAuthorizationRequest

8.	The PCF determines PCC rules based on the TSC assistance information

9.	The PCF acknowledges the Npcf_PolicyAuthorization request

10.	The NEF acknowledges the AF session with QoS request

11. The PCF installs updated PCC rules to the SMF

12.	The SMF installs N4 rules to the UPF.

### 6.x.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

-	AF translating Detnet configuration information into assistance information for the 3GPP network to establish appropriate QoS flows.

-	SMF, PCF need to relay UPF IP addreses

******************************** End of change *******************************