3GPP TSG-SA/WG2 Meeting #151e	S2-2204414r02

# Elbonia, May, 2022

Source:	CATT

Title:	KI #1, Sol #3: Update to remove ENs

Document for:	Approval

Agenda Item: 	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract of the contribution: This contribution proposes to update solution#3 for resolving the EN and add exposure of timing synchronization capability of UPF to AF.

- Discussion

1) As described in the current solution, it is allowed TSCTSF collects timing synchronization capability of UPF and creates timing synchronization configuration on the UPF upon reception of AF request. Another possibility is to allow exposure of timing synchronization capability of UPF to AF, and then AF request the timing synchronization requirement correspondingly. The procedure for exposure of capability of time sync service defined in clause 4.15.9.2 of TS 23.502[3] to can be enhanced to support exposure of timing synchronization capability of UPF.

2) To address the following EN:

Editor's note:  Reporting of 5GS network timing synchronization status to UE is FFS.

It is proposed the 5GS network time synchronization status is reported to UEs when UPF detects a change of time synchronization status (e.g. degradation or recovery of original timing source), then UPF updates the PTP clock class and clock accuracy for the (g)PTP messages for the UEs based on the UPF detected information or UMIC configured by TSCTSF (if the status reporting has received by TSCTSF).

2	Proposal

It is proposed to agree the following changes in clause 6.3 of TR 23.700-25.

******* start of 1st change ********

6.3	Solution #3: Timing synchronization resiliency and status reporting

6.3.1	Introduction

This solution aims to address Key issue #1: 5GS network timing synchronization status and reporting. When 5GS provides timing resiliency service e.g. in the smart grid or financial sector, the timing synchronization status (such as divergence from UTC, timing source degradation) needs to be able to inform UEs (e.g. application running in the UE), devices attached to the UE (i.e. that receive time information from 5GS) and AFs. The AF can configure 5GS with the requirements about the timing resiliency service to meet when timing synchronization event happens.

This solution makes the following assumptions:

-	UEs are consuming (g)PTP timing synchronization service from UPF/NW-TT that acts as grand-master based on IEEE Std 802.1AS [7] or IEEE Std 1588 [8];

-	5G GM may have different sources of time/frequency like GNSS signal, Synchronous Ethernet (SyncE), PTP transport network, PPS input, etc. It is assumed that the 5G GM is collocated with UPF or UPF is synchronized with 5G GM by PTP compatible transport, etc.

-	When UPF detects 5GS networking timing synchronization status for original timing source, it sends the reporting to 5GC. Additionally, the UPF activates the timing synchronization configuration for PTP GM functionality as configured by TSCTSF in case of degradation for original timing source, so as to provide seamless time synchronization service for the UEs; or the UPF de-activates the timing synchronization configuration for PTP GM functionality as configured by TSCTSF in case of recovery for original timing source.

Editor's note:	It is FFS how to address the case where time synchronization service is offered based on 5G Access Stratum timing distribution.

6.3.2	Functional Description

This solution uses the following principles:

-	TSCTSF collects timing synchronization capability of UPF. The TSCTSF retrieves the timing synchronization capability via PMIC or UMIC, and then TSCTSF sends the collected information to AF if it subscribes:

-	The timing synchronization capability can indicate holdover time or traceability capability information. For example, clock class, synchronization accuracy, holdover time parameters for the smart grid scenario, or UTC divergence scale and synchronization accuracy parameters for financial scenario.

-	TSCTSF creates timing synchronization configuration on the UPF upon reception of AF request:

-	TSCTSF receives timing synchronization requirement from AFs. The timing synchronization requirement includes timing resiliency service KPIs that 5GS needs to meet, e.g. AF requests for holdover time for 1h for ms sync accuracy for the smart grid scenario, or AF requests for 1ms maximum divergence from UTC for financial scenario.

Editor's note:	It is FFS whether and what additional information is needed in AF request for time synchronization service and subscription for 5GS networking timing synchronization status.

-	TSCTSF confirms that the 5GS can meet the requirement based on UPF reported timing synchronization capability.

-	TSCTSF creates a timing synchronization configuration on UPF.

-	With the timing synchronization configuration, the UPF detects and reports the networking timing synchronization status, and (de)activates the timing synchronization configuration for PTP GM functionality accordingly.

-	UEs are consuming (g)PTP timing synchronization service from UPF/NW-TT that acts as grand-master based on IEEE Std 802.1AS [7] or IEEE Std 1588 [8].

-	When UPF detects networking timing synchronization status for original timing source, it sends the status reporting to TSCTSF, and the TSCTSF forwards the reporting to AFs:

-	When UPF detects degradation or unavailable for original timing source, the UPF activates the timing synchronization configuration for PTP GM functionality as configured by TSCTSF, and the status reporting contains original timing source disabled status: 5GS timing synchronization enabled status, the timing source clock class, time validity.

-	When UPF detects recovery for original timing source, the UPF de-activates the timing synchronization configuration for PTP GM functionality, and status reporting contains original timing source enabled status: 5GS timing synchronization disabled status, the timing source clock class, time validity.

6.3.3	Procedures

Existing PDU Session establishment procedures are reused in timing synchronization capability configuration. Enhanced the procedure for exposure of capability of time sync service defined in clause 4.15.9.2 of TS 23.502[3] to support exposure of timing synchronization capability. Existing timing distribution procedures are reused for UEs using (g)PTP messages.

6.3.4	Impacts on services, entities and interfaces

UPF:

-	Support of signalling the timing synchronization capability to TSCTSF and AF.

-	Support of reporting time synchronization status (e.g. holdover time or UTC divergence status) to TSCTSF.

TSCTSF:

-	Support of receiving timing synchronization requirement from AFs.

-	Support of timing synchronization capability configuration.

-	Support of timing synchronization status report to AFs.

AF:

-	Support of providing timing synchronization requirement (e.g. holdover time and UTC divergence requirement) to TSCTSF, and receiving status reporting from TSCTSF.

-  Support of subscribing timing synchronization capability.

******* End of 1nd change ********