SA WG2 Meeting #151E	S2-2204430r01

# 16 – 20 May 2022, e-meeting

Source:	Lenovo

Title:	KI#4: Update of solution 12 to address editor's notes

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_ph3 / Rel-18

Abstract of the contribution:

# 1		Discussion

The updates made are the following:

-	Clarifying further the applicability of the option where the Data Producer for Location Accuracy Data is the LMF is for the case where Analytics are requested for a target area for Any UE.

-	Adding an additional option to describe the scenario where the Data Producer for providing location information to the NWDAF is the GMLC.

# 2		Proposal

The following solution is proposed.

******************************** First change (all new text) *******************************

## 6.12	Solution #12: Supporting analytics for location accuracy

### 6.12.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s) which should be explicitly stated. Clause(s) may be added to capture details.

As part of the location services requirements defined in TS 22.071 [18], one of the main requirements for the LCS service is the support of Location Accuracy.

The location accuracy is provided in terms of horizontal, vertical accuracy and response time. In general the accuracy of the location depends on the positioning technology used, the varying radio environments (e.g. considering multipath propagation), the type of environment where the location is measured (dense urban area or rural area).

The location accuracy required by each service varies and can vary from location accuracy up to 200km for weather services to location accuracy of up to 50m for Asset Location, route guidance, navigation. Further information is provided in Table 4.1 of TS 22.071 [18].

When a location client requires a location accuracy the location client may include in the request to the GMLC:

-	A response time (no delay, low delay, delay tolerant).

-	Horizontal and/or vertical accuracy requirements (in meters).

-	LCS QoS Class (best effort, assured or multiple QoS corresponding to a priority of QoS accuracy).

When the LMF determines the location the LMF also determines the accuracy of the location and also whether the location requirements are met which are sent to the GMLC. The GMLC responds to the client the accuracy of the location measurement can be met.

Analytics from the NWDAF can be enhanced to allow a location client to be aware if a location accuracy can be met. Example consumers of such analytics is as follows:

-	LMF uses the analytics to identify the best positioning method to use to determine a location in a specific area, taking into account UE capabilities, QoS requirements and operator policies.

-	The location client can determine to upgrade/downgrade the requested LCS QoS taking into account the analytics of location accuracy.

-	The location client can adapt the application service accordingly. For example, a navigation service can take into account areas where location accuracy cannot be met when providing route guidance.

### 6.12.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

A consumer (e.g. a location client) requests from the NWDAF analytics for "Location Accuracy Sustainability". The request may also include the following as Analytic Filters:

-	A target area.

-	A target UE (not applicable for option 1), group of UEs (not applicable for option 1) or any UE.

-	Time of Day.

-	An LCS QoS Class or Horizontal/Vertical Accuracy requirement (denotes the requested LCS QoS or Horizontal/Vertical accuracy requirement.

-	Optionally a positioning method used.

-	A location accuracy threshold, i.e. report accuracy analytics only if the location accuracy exceed an horizontal or a vertical accuracy threshold (in meters).

Two options are available from the NWDAF to collect input data for Location Measurement Accuracy

-	Option 1: Data Producer is the LMF (applicable when an analytics consumer requests Location Accuracy Analytics for any UE for a specific Target Area)

-	Option 2: Data Producer is the GMLC

Data collection for Option 1:

The NWDAF finds the LMF that servers a target area. The NWDAF then invokes a new SBI (e.g. Nlmf_Event_Exposure) and requests data from the LMF identified by a new Event ID, e.g. "Location Measurement Accuracy request". The LMF based on the Event ID requested provides information on accuracy of a measurement of a location when a location measurement is made by the LMF.

Editor's note:	Whether the LMF can provide measurement information to NWDAF for location accuracy is FFS.

i thi

The LMF provides measurements reports to the NWDAF as follows:

The LMF provides measurements report for any location request received by the LMF by a location client (the request is received via the GMLC)

-	If the request includes a requested LCS QoS class or horizontal/vertical accuracy the LMF provides a measurements report when the LMF receives a location request from a location client with the requested accuracy

-	If the request includes a positioning method, the LMF provides measurements report when the LMF determines the location using the position method used.

The procedure is shown below:

Figure 6.12.2-1: Procedure to derive analytics for location accuracy sustainability (Option 1)

1.	A Consumer requests analytics for location accuracy sustainability from the NWDAF. The request may include a target area, a time of day, a target UE (or group of UEs or any UE). The request may also include, to report analytics for a specific LCS QoS requirement or horizontal/vertical accuracy.

2.	The NWDAF finds the LMF serving the targer area (Option 1) by querying the NRF .

3.	The NWDAF sends a request to the LMF to provide location measurement accuracy data (the request may be a subscription or a one-time request). The NWDAF includes in the request to the LMF information to satisfy the requirements of the consumer in step 1.

4a-4c.	The LMF is triggered to provide a location  from a location client as per procedure in TS 23.273 [5]. The LMF derives the location of the UE.

5.	The LMF determines that the location measured is in an area where the NWDAF has subscribed to report location measurements. The LMF reports the location accuracy to the NWDAF.

6.	The data are provided to the NWDAF

7.	The NWDAF derives analytics for location accuracy .

8.	The NWDAF provides analytics to the consumer.

Data collection for Option 2:

The NWDAF subscribes from GMLC to obtain location data for a UE or any UE. The NWDAF discovers GMLC from NRF and invokes (via a new SBI) to retrieve data about the location measurement accuracy (optionally per positioning method).

The GMLC provides measurements report to the NWDAF as follows:

-	If the request target any UE the GMLC provides measurements report for any location request received by a location client

-	If the request target a specific UE, the GMLC provides measurements report when the GMLC receives a location request from a location client that targets the specific UE.

-	If the request includes a requested LCS QoS class or horizontal/vertical accuracy the GMLC provides measurements report when the GMLC receives a location request from a location client with the requested accuracy

-	If the request includes a positioning method, the GMLC provides measurements report when the GMLC receives information (from the AMF) on the positioning method used by the LMF to determine the location of the UE.

The procedure is shown below:

Figure 6.12.2-2: Procedure to derive analytics for location accuracy sustainability (Option 2)

1.	A Consumer requests analytics for location accuracy sustainability from the NWDAF. The request may include a target area, a time of day, a target UE (or group of UEs or any UE). The request may also include, to report analytics for a specific LCS QoS requirement or horizontal/vertical accuracy.

2.	The NWDAF finds the GMLC from the NRF

3.	The NWDAF sends a request to the GMLC to provide location measurement accuracy data (the request may be a subscription or a one-time request). The NWDAF includes in the request to the GMLC information to satisfy the requirements of the consumer in step 1.

4a-4b.	The GMLC is triggered to provide a location  from a location client as per procedure in TS 23.273 [5]. The GMLC invokes a location request to the AMF/LMF as per 3GPP TS 23.273 [5]. The GMLC receives location information from the AMF/LMF including information indicating if the location accuracy requested has been met and the positioning method used.

5.	The GMLC determines to report the location information to the NWDAF.

6.	The data are provided to the NWDAF

7.	The NWDAF derives analytics for location accuracy .

8.	The NWDAF provides analytics to the consumer.

The following input data are required by the NWDAF to measure location measure accuracy.

Table 6.12.2-1: Location measurement accuracy data collected by NWDAF

| Information | Source | Description |
| --- | --- | --- |
| Location Measurement accuracy data | LMF or GMLC | Includes information on the Position Method used, the requested LCS QoS or horizontal/accuracy and whether the location accuracy request was satisfied |
| Timestamp | LMF or GMLC | The time where the location was measured by the LMF |
| UE Location | AMF | The location area of the UE when the location measurement was made |

The analytics output to the consumer may include one of the following:

Table 6.12.2-2: "Location Accuracy Sustainability" statistics

| Information | Description |
| --- | --- |
| List of Accuracy Analytics (1..max) |  |
| >Applicable Area | A list of TAIs or Cell IDs within the Location information that the analytics applies to. |
| >Applicable Time Period | The time period within the Analytics target period that the analytics applies to. |
| >The requested LCS QoS | The analytics provide a delta against the request measurement accuracy level from a consumer |
| > List of SUPIs | A list of SUPIs experiencing the same analytics accuracy |
| > Positioning Method | The positioning method used to measure location |
| > Percentage of analytics accuracy above/below requested thresholds | A percentage indicating if the requested analytics accuracy is met |

Table 6.12.2-3: "Location Accuracy Sustainability" predictions

| Information | Description |
| --- | --- |
| List of Accuracy sustainability Analytics (1..max) |  |
| >Applicable Area | A list of TAIs or Cell IDs within the Location information that the analytics applies to. |
| >Applicable Time Period | The time period within the Analytics target period that the analytics applies to. |
| > The requested LCS QoS | The analytics provide a delta against the request measurement accuracy level from a consumer |
| > List of SUPIs | A list of SUPIs experiencing the same analytics accuracy |
| > Positioning Method | The positioning method used to measure location |
| > Percentage of analytics accuracy above/below requested thresholds | A percentage prediction indicating if the requested analytics accuracy is met |
| >Confidence | Confidence of the prediction. |

### 6.12.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

-	Applicable to Option 1: New Nlmf event notification service to allow the NWDAF to retrieve location measurement reports from the LMF.

-	Applicable to Option 2: NWDAF support Ngmlc service based interfaces to retrieve location information for a UE.

******************************** End of change *******************************