3GPP TSG-SA WG2 Meeting #151e	S2-2204433r01

16- 20 May 2022, e-Meeting

Source:	Lenovo

Title:	KI#4 - New solution for path switching of remote UE(s) having an indirect connection via a L2 UE-NW Relay

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract of the contribution:.

# Introduction

This solution addresses Key Issue 4 .

A solution is proposed to address the scenario where a Layer 2 based UE-NW Relay that has one or more Remote UE(s) having an active indirect connection via this relay and the network determines that the UE-NW relay needs to handover. When the handover takes place one or more of the Remote UE(s) may not be allowed to access the target RAN node selected for this UE-NW relay due to mobility restrictions. In order to ensure service continuity the source RAN node can effectively trigger path switch to a direct connection for such Remote UE(s).

The details of this solution are presented below,

# Proposal

It is proposed to include the following in TR 23.700-33 v0.2.0

********************************START OF CHANGES (all new text)*******************************

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | X |  |  |  |  |  |  |
| 2 | X |  |  |  |  |  |  |
| 3 | X |  |  |  |  |  |  |
| 4 | X |  |  |  |  |  |  |
| 5 | X |  |  |  |  |  |  |
| 6 | X |  |  |  |  |  |  |
| 7 | X |  |  |  |  |  |  |
| 8 | X |  |  |  |  | X |  |
| 9 | X |  |  |  |  |  |  |
| 10 | X |  |  |  |  |  |  |
| 11 | X |  |  |  |  |  |  |
| 12 | X |  |  |  |  |  |  |
| 13 | X |  |  |  |  |  |  |
| 14 |  | X |  |  |  |  |  |
| 15 |  | X |  |  |  |  |  |
| 16 |  |  | X |  |  | X |  |
| 17 |  |  | X |  |  |  |  |
| 18 |  |  | X |  |  |  |  |
| 19 |  |  | X |  |  |  |  |
| 20 |  |  | X |  |  |  |  |
| 21 |  |  | X |  |  |  |  |
| 22 |  |  | X |  |  |  |  |
| 23 |  |  |  | X |  |  |  |
| 24 |  |  |  | X |  |  |  |
| 25 |  |  |  |  | X | X |  |
| 26 |  |  |  |  | X |  |  |
| 27 |  |  |  |  | X |  |  |
| 28 |  |  |  |  | X |  |  |
| 29 |  |  |  |  | X |  |  |
| X |  |  |  | X |  |  |  |

## 6.X	Solution #X: Path switching of remote UE(s) having an indirect connection via a L2 UE-NW Relay

### 6.X.1	Description

This solution addresses Key Issue 4 .

A solution is proposed to address the scenario where a Layer 2 based UE-NW Relay that has one or more Remote UE(s) having an active indirect connection via this relay and the network determines that the UE-NW relay needs to handover. When the handover takes place one or more of the Remote UE(s) may not be allowed to access the target RAN node selected for this UE-NW relay due to mobility restrictions. In order to ensure service continuity the source RAN node can effectively trigger path switch to a direct connection for such Remote UE(s).

This solution requires the following functionality to be supported:

-	NG-RAN node is aware of the Remote UE(s) that have an indirect 3GPP connection via a Layer-2 based UE-NW relay

-	NG-RAN node is aware of the Mobility Restriction List of the L2 UE-NW Relay and Remote UE(s) having an active indirect connection via the UE-NW Relay

-	Remote UE(s) provide Measurement Reports to the NG-RAN node via the Layer-2 based UE-NW relay

-	NG-RAN node handles the handover signalling of the L2 UE-NW Relay and Remote UE(s) (that have an indirect 3GPP connection via the L2 UE-NW relay) separately

### 6.X.2	Procedures

The procedure is described in the Figure below.

Figure 6.x.2-1 - Path switch procedure

0a.	A Remote UE has already selected a UE-NW Relay and has an active indirect connection to the 3GPP network via the L2 UE-NW Relay. Both Remote UE and UE-NW Relay are in Connected state

0b.	The Source RAN node is aware of which Remote UE(s) have an active indirect connection via a UE-NW Relay

0c.	The Source RAN node is also aware of the Mobility Restriction List of both the UE-NW Relay and the Remote UE

1a.	The L2 UE-NW Relay provides RRC Measurement Reports to the Source RAN node

1b.	The Remote UE provides RRC Measurements Reports to the Source RAN node via the L2 UE-NW Relay

2.	Based on the measurement reports of the L2 UE-NW Relay the Source RAN node decides to handover the L2 UE-NW Relay and selects a Target RAN node. The Target RAN node selected should support both L2 UE-NW Relay and Remote UEs.

Editor's Note: Step 2 has RAN dependency and should be evaluated by RAN WGs

3.	The Source RAN node may decide to switch the Remote UE connection to a direct 3GPP connection and selects a Target RAN node for the Remote UE taking into account the Measurement Reports provided by the Remote UE in step 1b.

4.	Source RAN node starts the Handover Preparation phase for the Remote UE as described in clause 4.9.1.3.2 of 3GPP TS 23.502 [x]. The Source RAN node includes in the handover signalling information related to the Remote UE (e.g. QoS Flows/DRB information and the PDU sessions related to the Remote UE).

5.	The AMF of the Remote UE starts the Execution Phase of the handover sending a Handover Command indication to the Remote UE.

6.	The Remote UE determines a handover to a direct connection.

7.	The Remote UE synchronises to the target cell

8.	The AMF of the Remote UE sends a PDU session update to the SMF informing that the handover is complete (as per clause 4.9.1.3.3 of 3GPP TS 23.502 [x]).

9.	The Remote UE may release the direct PC5 connection to the UE-NW relay

10.	Source RAN node starts the Handover Preparation phase for the L2 UE-NW Relay as described in clause 4.9.1.3.2 of 3GPP TS 23.502 [x]. The Source RAN node includes in the handover signalling information related to the L2 UE-NW Relay (e.g. QoS Flows/DRB information and the PDU sessions related to the UE/NW Relay).

11.	The AMF of the L2 UE-NW Relay starts the Execution Phase of the handover sending a Handover Command indication to the L2 UE-NW Relay.

12.	The L2 UE-NW Relay synchronises to the target cell

13.	The AMF of the L2 UE-NW Relay sends a PDU session update to the SMF informing that the handover is complete (as per clause 4.9.1.3.3 of 3GPP TS 23.502 [x]).

### 6.x.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

-	NG-RAN triggers handover of remote UE(s) during handover of relay UE.

******************************** END OF CHANGES **************************************