SA WG2 Meeting #151E	S2-2204435r03

# 16 – 20 May 2022, e-meeting

Source:	Lenovo

Title:	KI#2: Update Solution 7: Clarification of AF requesting SFC policies.

Document for:	Approval

Agenda Item:	9.1

Work Item / Release:	FS_SFC / Rel-18

Abstract of the contribution:

1		Discussion

During discussion of the SFC topic in SA2#150e two main use cases for supporting SFC in 5GS have been discussed:

- 	Scenario 1: Operator owns/deploys SFC functions that are hosted in the 5GC network. AF indicates to the 5GC what traffic is required to be routed via one or more SFC functions for one or more UEs in the 5GC network

-	Scenario 2: SFCs functions are hosted by 3rd party network. 5GC is configured (via an AF) to route traffic to the SFC function(s) hosted by the 3rd party.

Scenario 1 assumes that the 5GC operator has already configured traffic steering policies for routing traffic over to the SFC network and the 3rd party AF is aware (e.g. via SLA) of the available traffic steering policies that can be used. Each traffic steering policy

Scnenario 2 requires that AF indicates to the 5GC network how the traffic should be "tagged" with SFC related information when user plane traffic is routed to a 3rd party SFC network. The UPF is configured to add the SFC ID(s) and metadata required by the 3rd party SFC network to handle the user plane traffic accordingly.

Based on the aforementioned scenarios this contributions addresses the following questions:

- 	For scenario 1: Does a 3rd party AF need to discover available SFC functions hosted in a 5GC in order to indicate to the 5GC network what SFC services should be enabled for user plane traffic of a UE or group of UEs or is it assumed that all configuration is done via SLA agreements?

-	The option for configuration based on SLA agreement is already captured in the solution. However, it is also proposed to capture the procedure for negotiation of SFC policies to allow a more dynamic method of a 3rd party AF to indicate what SFC services are needed. In such procedure, the 3rd party AFs identifies all available SFC functions hosted in the 5GC network and indicates to the network what SFC functions (and in what order) user plane traffic should be handled.

-	It is proposed that as part of the conclusion phase it will be determined whether a more dynamic method is required for SFC configuration.

- 	For scenario 2: How does a 3rd party AF indicate to the 5GC how to "tag" user plane traffic?

-	The dynamic configuration of SFC policies can be also re-used for this purpose with the difference that the traffic steering policies include some additional information to assist the UPF to tag the user plane traffic when the traffic is routed to the 3rd party SFC network.

In addition the following editor's note is addressed:

Editor's note:	It is FFS how the 5GC selects the UPF with the appropriate predefined SFPs corresponding to the AF request of the 3rd party. The operator can have SLAs with multiple 3rd parties and the predefined SFPs for distinct 3rd parties can be deployed in distinct UPFs.

The assumption with this editor's note is that it mainly addresses scenario 2 where a 3rd party AF requires specific handling of SFC when user plane traffic is routed to a 3rd party SFC via a local UPF.

To support such requirement it is proposed the 3rd party AF to include in the traffic influence request, DNAI information assisting the SMF to select a UPF that interfaces with a specific 3rd party SFC.

Further details are provided in the solution description.

2		Proposal

The following solution is proposed.

******************************** First change  *******************************

## 6.7	Solution #7: Enhancing Application Function influence on traffic routing to an N6-LAN based on pre-defined SFC policies

### 6.7.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s) which should be explicitly stated. (Sub) clause(s) may be added to capture details.

The present solution addresses the following:

-	How an AF is aware of the available pre-defined SFC policies in a 3GPP network.

-	How an AF influences traffic steering for application traffic to an N6-LAN.

The solution supports both scenarios where network operator owns/deploys SFCs that are hosted in the 5GC network (Scenario 1) and SFCs hosted by 3rd party network. 5GC is configured (via an AF) to route traffic to the SFC function(s) owned  by the 3rd party (Scenario 2).

Editor's note:	This SFC function or SF need be further clarified.

Two options are proposed:

Option 1: AF is aware via SLA agreements the SFC policy identifier(s) that corresponds to a specific Service Function Chaining

This option assumes that AF is aware of the SFC Policy identifier(s) based on SLA agreements

Option 2: AF requesting an SFC policy based on the available Service Function Services offered by the 5GS network

It is first assumed that the 3rd party AF is aware of the available Service Function services (SF service) (e.g. voice transcoding, TCP optimisation) offered by the 3GPP network via SLA agreements where each service is identified by an SF service identifier. What is required is the ability of the 3rd party AF to request to the network to discover SFC policy based on the available SF services and enable steering of traffic according to the SFC policy identifier provided by the network for service data flows for one or more UEs.

The AF discovers the available SF services and requests to discover an SFC policy from the 3GPP network via a new SBI interface. The AF request to the 3GPP network (via NEF) can include the following information:

-	An ASP identifier.

-	Optionally a UE identifier (or Group identifier).

-	A list of SF services where each service identified by a SF service identifier

The 3GPP network authorises the AF request and provides the SFC policies to the AF where each SFC policy is identified by a specific SFC policy identifier.

When the AF requires to enable SFC policies for AF traffic the AF can re-use the Nnef_TrafficInfluence SBI including in the request the target UEs, service data flow information of the traffic to be routed an SFC policy identifier. To address scenario 2 the AF additionally includes:

-	DNAI information allowing the SMF to select a UPF capable to interface with a 3rd party SFC.

-	Metadata information allowing a UPF to identify how to tag user plane traffic sent to an external 3rd party SFC

### 6.7.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

#### 6.7.2.1	AF requesting SFC policies in a 3GPP network

The procedure for requesting N6-LAN services is as follows:

Figure 6.7.2.1-2: AF requesting pre-defined SFC policies

The procedure is as follows:

0.	OPTION 1: The AF is aware of the available SFCs and corresponding SFC policy identifier(s) in a network . This can be based on SLA agreements.

OPTION 2: AF requesting SFC policies based on pre-defined SF services available either in the 5GC or hosted by a 3rd party.

1.	An AF is triggered to discover the available SF services.

2.	The AF requests the available SF services by sending an Nnef_SF_Services_Get Request including an ASP identifier

3.	The NEF provides the available SF services based on the ASP identifier. How the NEF is aware of this information is based on configuration (e.g. via OAM configuration).

Editor's note:	It is FFS whether it need standardized the SF service name.

4.	The AF is triggered to discover an SFC policy. For example, enable voice transcoding.

5.	The AF invokes an Nnef_SFC_Policy_Get_Request including an identifier of the ASP, optionally external identifiers (GPSI) and a list of SF service(s) identified by a list of SF service ID(s)

6.	The NEF forwards the request to the PCF invoking an Npcf_SFC_Policy_Create request.

7.	The PCF may check in the UDR if there are available SFC policies for the ASP.

8.	The PCF makes a policy decision and determines the SFC policies for the ASP. If there in no SFC policy available for the requested service the PCF rejects the request.

9.	The PCF includes the SFC policy according to the AF request identified by an SFC policy identifier.

10.	The NEF forwards the response to the AF.

Editor's note:	Whether Option 1 or Option 2 will be used for configuration of SFC policies when the SFC is hosted by the 5GC operator is to be determined at conclusions phase.

#### 6.7.2.2	AF influence on traffic routing by including pre-defined SFC policies

When the AF is triggered to enable an N6-LAN service the AF invokes an Nnef_TrafficInfluence_Create that includes the UE ID or Group ID of the affected UEs and N6-LAN steering information.

The N6-LAN steering information includes:

-	Application ID or Traffic Filters of the affected application.

-	SFC policy identifier according to the N6-LAN service requested by the AF as described in Figure 6.7.2.1-1.

-	Applicable S-NSSAI/DNN information.

If S-NSSAI/DNN is included the policy is applicable for all UEs that have established a PDU session at this DNN and/or S-NSSAI.

The NEF determines the internal identifier of the UE(s) (i.e. SUPI) and update the subscription in the UDR by invoking an Nnef_DM_Update request service operation. The NEF updates the subscription data in the UDR.

The UDR notifies the PCF of the updated subscription information. The PCF determines the affected UEs, PDU sessions and application traffic and determines new or updated PCC rules where the PCC rule includes traffic descriptors of the traffic that needs to be routed to the operator's N6-LAN network and corresponding traffic steering policy identifier of the associated SFC policy (identified by the SFC policy identifier) . The PCF then provides updated PCC rules to the SMF. The SMF configures the UPF with the traffic steering policy identifier corresponding to the SFC policy identifier that allows the UPF to route application traffic to the N6-LAN network.

The detailed procedure is illustrated below:

Figure 6.7.2.2-1: AF requesting to enable an N6-LAN service for a UE

1.	The AF is triggered to enable anSFC policy for a UE or group of UEs.

2.	The AF invokes an Nnef_TrafficInfluence Create including in the request the external identifier of the UE (or Group of UEs) and in a container SFC steering information that includes, an application id or traffic filters of the application andan SFC policy identifier. The AF may include a DNAI and metadata information for the case where SFC are owned by a 3rd party.

3.	The NEF determines the SUPI of the UE by interfacing with the UDM. The NEF may also determine if the ASP is authorized to use the SFC policy by interfacing with the UDR.

4.	The NEF updates the subscription data in the UDR by invoking an Nudr_DM_Update service operation including SFC steering information as a data subset under the Application Data Set and adding the SUPI of the affected UEs as a Data Key.

5.	The UDR notifies the PCFs that have subscribed to be notified of the changes in the subscription profile

6.	The UDR invokes an Nudr_DM_Notify service operation including the updated subscription information.

7.	The PCF determines the affected UEs, traffic and PDU sessions where the SFC policy needs to be included/updated.

8.	The PCF provides updated PCC rules including the corresponding pre-defined traffic steering policy identifier for N6-LAN to the SMF. If metadata information is provided in step 2 the PCF forwards the information to the SMF.

9.	The SMF selects a UPF based on existing mechanisms and configures the UPF according to the PCC rules. The SMF sends over N4 a Forward Action Rule request including in the request traffic filters and the traffic steering policy identifier. The SMF includes the metadata if provided by the PCF in step 8.

NOTE:	SMF selects a UPF based on DNAI if provided by the AF in step 2.

10.	The UPF routes traffic corresponding to the traffic filters to N6-LAN. If metadata information is provided by the SMF, the UPF classifies the packet, e.g. by adding VxLAN information according to the metadata information, before routing the packet to the SFC function.

### 6.7.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

-	New SBI supporting an AF to discover the available SFC policies.

-	Enhancing Nnef_TrafficInfluence SBI supporting steering of traffic to N6-LAN.

-	If metadata is supported:

AF including metadata information in traffic influence request

-	NEF, PCF, SMF forwarding metadata information to UPF

-	UPF tagging the packet according to metadata information before routing the packet to the N6-LAN network.

******************************** End of change *******************************