SA WG2 Meeting #150E	S2-2204438

# 6 – 12 April 2022, e-meeting

Source:	Lenovo

Title:	KI#9: New solution supporting UE mobility analytics with finer granularity than TA/cell

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_ph3 / Rel-18

Abstract of the contribution:

# 1		Discussion

If the NWDAF requires to provide UE mobility analytics with finer granularity than TA/cell then the NWDAF can interface with the GMLC/AMF to obtain location information using the LCS procedures. In such scenario the NWDAF interface with a GMLC to obtain location information for a UE. Another approach is that the NWDAF acts as a GMLC client and obtain location information directly from the AMF.

The UE mobily analytics output include as additional analytic output the UE location in geographical coordinates and/or local coordinates expressed as a shape as defined in TS 23.032.

In addition, as part of the support of Location Services work, specified in 3GPP TS 23.273, a consumer may request a deferred location request requesting to be notified of a location of a UE when specific events associated to a target UE take place. The following types of events are defined:

a)	UE availability: Any event in which the 5GCN has established a contact with the UE. This event is considered to be applicable when the UE is temporarily unavailable due to inaction by the user, or for temporarily loss of radio connectivity or IMSI detach and so on

b)	Area: An event where the UE enters, leaves or remains within a pre-defined geographical area.

c)	Periodic Location: An event where a defined periodic timer expires in the UE and activates a location report.

d)	Motion: An event where the UE moves by more than some predefined straight line distance from a previous location. The motion event may be reported one time only, or multiple times.

The UE mobility analytics provided by the NWDAF can be additionally enhanced to collect location information for one or more UEs (or any UE) using the deferred location events supported by the eLCS architecture. Such scenario is useful if the consumer requires to be aware of the UE mobility trends in a target area. For example, a consumer may require to be aware of mobility analytics of UEs that are in motion in a target area.

Further details are include in the main solution.

# 2		Proposal

The following solution is proposed.

******************************** First change (all new text) *******************************

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 9 |  |  |  |
| X |  |  |  |  |

## 6.X	Solution #X: Supporting UE mobility analytics with finer granularity than TA/cell

### 6.X.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s) which should be explicitly stated. Clause(s) may be added to capture details.

If the NWDAF requires to provide UE mobility analytics with finer granularity than TA/cell then the NWDAF can interface with the GMLC/AMF to obtain location information using the LCS procedures. In such scenario the NWDAF interface with a GMLC to obtain location information for a UE. Another approach is that the NWDAF acts as a GMLC client and obtain location information directly from the AMF.

The UE mobily analytics output include as additional analytic output the UE location in geographical coordinates and/or local coordinates expressed as a shape as defined in TS 23.032 [y].

In addition, as part of the support of Location Services work, specified in 3GPP TS 23.273 [x], a consumer may request a deferred location request requesting to be notified of a location of a UE when specific events associated to a target UE take place. The following types of events are defined:

a)	UE availability: Any event in which the 5GCN has established a contact with the UE. This event is considered to be applicable when the UE is temporarily unavailable due to inaction by the user, or for temporarily loss of radio connectivity or IMSI detach and so on

b)	Area: An event where the UE enters, leaves or remains within a pre-defined geographical area.

c)	Periodic Location: An event where a defined periodic timer expires in the UE and activates a location report.

d)	Motion: An event where the UE moves by more than some predefined straight line distance from a previous location. The motion event may be reported one time only, or multiple times.

The UE mobility analytics provided by the NWDAF can be additionally enhanced to collect location information for one or more UEs (or any UE) using the deferred location events supported by the eLCS architecture. Such scenario is useful if the consumer requires to be aware of the UE mobility trends in a target area. For example, a consumer may require to be aware of mobility analytics of UEs that are in motion in a target area.

### 6.X.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

If a consumer requires UE mobility analytics with finer granularity than TA/cell then the following apply.

Figure 6.x.2-1: Supporting UE mobility analytics taking into account mobility trends.

1.	A Consumer requires UE mobility analytics taking into account UE mobility trends or specific pre-defined geographical area of a UE or group of UEs or UEs in a target area.

2.	Consumer sends a request for analytic including the analytic ID and as analytic filters.

-	A target area.

-	Motion Event Criteria specifying a threshold linear distance that the UE must make before the UE provides a location.

3.	The NWDAF requests location reports from the GMLC/AMF using eLCS location procedures as described in 3GPP TS 23.273 [x].

4.	NWDAF derives UE mobility analytics

6.	Analytics are reported to the consumer NF.

The following input data are required by the NWDAF to measure UE mobility analytics for specific UE mobility trends in addition to the input data defined in clause 6.7.2.2 of 3GPP TS 23.288 [x].

Table 6.x.2-1: Location measurement from LMF (via AMF)

| Information | Source | Description |
| --- | --- | --- |
| UE location estimate | GMLC/AMF | The estimated location of the UE determined by the LMF |
| Indication of motion event occurence | GMLC/AMF | Provides an indication that the UE has moved a requested linear threshold distance |
| Timestamp | GMLC/AMF | Time stamp of location estimation |

The UE mobility analytics output to a consumer include the following, in addition to the analytic output data defined in clause 6.7.2.3 of 3GPP TS 23.288 [x].

Table 6.x.2-2: UE mobility statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [x] clause 5.9.7, SUPI. |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot start within the Analytics target period |
| > Duration | Duration of the time slot |
| > UE location (1..max) | Observed location statistics |
| >> UE location | TA or cells or geographical/local coordinates as described in 3GPP TS 23.032 [y] which the UE stays |
| …..>> UE mobility trends | Distance travelled within reported location(s) |
| >> Ratio | Percentage of UEs in the group (in the case of a UE group) |

Table 6.x.2-3: UE mobility predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [x] clause 5.9.7, or SUPI. |
| Time slot entry (1..max) | List of predicted time slots |
| >Time slot start | Time slot start time within the Analytics target period |
| > Duration | Duration of the time slot |
| > UE location (1..max) | Predicted location prediction during the Analytics target period |
| >> UE location | TA or cells or geographical/local coordinates as described in 3GPP TS 23.032 [y] where the UE may move into |
| … >> UE mobility trends | Distance travelled within reported location(s) |
| >> Confidence | Confidence of this prediction |
| >> Ratio | Percentage of UEs in the group (in the case of a UE group) |

Editor's note:	UE location needs to be in the form of some area to enable related analytics. It is ffs how areas can be defined with finer granularity than TA/cell level.

### 6.X.3	Impacts on services, entities and interfaces

Editor's note: This clause captures impacts on existing 3GPP nodes and functional elements.

-	NWDAF supporting eLCS SBI to retrieve UE location information from GMLC/AMF

******************************** End of change *******************************