SA WG2 Meeting #151e	S2-2204461r01

# May 16th – 20th, 2022; Elbonia               			(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	23.700-85: Updates to Solution #1; Usage of application guidance for URSP rules determination mechanisms

Document for: 	Approval

Agenda Item: 	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract of the contribution:

# 1 Discussion

In solution #1 it is proposed that when the HPLMN provides URSP rule for a VPLMN, it may include a validity condition in the URSP rule to indicate to the UE that the URSP rule is valid only for a specific PLMN.

"These URSP may have a validity condition corresponding to the VPLMN. As part of the location validity condition the AF from VPLMN may provide the list of TA(s) or Cell Id(s) of the VPLMN."

However, there were questions raised regarding whether a separate validity condition is needed or can we use the PLMN ID indication at the PSI level to identify for which PLMN the URSP rule is valid. Accordingly, below Editor's Note was added in the solution #10 in the previous SA2 meeting (i.e. SA2 #150e) to address this issue in subsequent meetings.

Editor's note:	It is FFS whether a PLMN validity condition should be included in the URSP rule (e.g. in the RSD) or should the PLMN ID indication at the PSI level be used to identify for which PLMN the rule apply.

Below information can be noted in the TS 23.503:

=============

The PLMN ID is provided to the UE together with UE policy information and it is used to indicate which PLMN a Policy Section list belongs to.

The UE provides the list of stored PSIs which identify the Policy Sections associated to the home PLMN and the visited PLMN (if the UE is roaming) that are currently stored in the UE.

When the PCF (i.e. the (H-)PCF as well as the V-PCF) receives a list of PSIs associated to the PLMN of the PCF from the UE, the PCF compares the list of PSIs provided by the UE and the list of PSIs retrieved from the UDR.

In the roaming scenario, the V-PCF shall also forward any UE provided PSIs that are associated to the home PLMN to the H-PCF.

When the PCF (i.e. the (H-)PCF as well as the V-PCF) receives a list of PSIs associated to the PLMN of the PCF from the UE, the PCF compares the list of PSIs provided by the UE and the list of PSIs retrieved from the UDR.

=============

From the above description in the current specification, it is clear that the PLMN ID in the policy section is used to identify which PLMN owns the policy section. A PCF can provide UE policies in policy sections (identified by PSI) that are associated with the PLMN that the PCF belongs to. So, a PCF from HPLMN cannot provide UE policies for a Policy Section list that belongs to a VPLMN. Hence, PLMN ID indication at the PSI level cannot be used for this solution.

Proposal:	It is proposed to clarify in the Solution #10 that PLMN ID in the PSI cannot be used. And it is proposed to use the existing Route Selection Validation Criteria condition in the URSP rule.

# 2 Proposal

It is proposed to update 23.700-85 as follows

FIRST CHANGE

## 6.1	Solution #1: Usage of application guidance for URSP rules determination mechanisms

### 6.1.1	Description

Currently when a UE is served by a network other than its home operator (i.e. VPLMN) the serving network cannot influence the determination of URSP. It would be useful for the serving network to be able to influence the determination of URSP for example in LBO case where the VPLMN may support dedicated DNN(s) for Edge applications.

A solution is to leverage, clause 6.6 of TS 23.548 [5] "Support of AF Guidance to PCF Determination of Proper URSP Rules" and clause 4.15.6.10 of TS 23.502 [3] "Application guidance for URSP rules determination mechanisms" as defined for Rel-17.

An AF from the VPLMN provides the VPLMN specific URSP information (e.g. DNN and S-NSSAI to be used for an application traffic etc.) (according to clause 6.6 of TS 23.548 [5] and clause 4.15.6.10 of TS 23.502 [3]); this indication can be leveraged by the PCF of the HPLMN to (if authorized by HPLMN) provide dedicated URSP rules applicable to relevant users of the HPLMN when they are roaming in that VPLMN. These URSP may have a validity condition (e.g. Time Window and/or Location Criteria) corresponding to the VPLMN. The location validity condition may be provided by the AF from VPLMN that contains the list of TA(s) or Cell Id(s) of the VPLMN.

NOTE 1:	The PLMN ID in the the UE Policy Section Identifier (UPSI) is used to determine which PLMN owns the policy section according to TS 29.525[xx]. A PCF can provide UE policies for policy sections that are associated with the PLMN that the PCF belongs to i.e. a PCF from HPLMN cannot provide UE policies for a Policy Section list that belongs to a VPLMN.

The H-PCF may, based on local policies, determine whether to always send the URSP rules immediately to the UE or to send these rules only when the UE is served by the corresponding serving PLMN, e.g. waiting to be triggered from a V-PCF of that PLMN in step 3 of clause 4.6.11 of TS 23.502 [3].

The PCF of the HPLMN when it sends such URSP update to a UE, needs, as for Rel-17, to ensure that the DNN, S-NSSAI provided in the URSP rules sent to the UE is allowed in UDM subscription even when the UE is roaming in that VPLMN.

NOTE 2:	This solution could also apply in FS_EDGE_Ph2.

NOTE 3:	This solution is applicable only when HPLMN supports dynamic PCC and application guidance for URSP rules determination mechanisms.

### 6.1.2	Procedures

The solution reuses existing procedures.

### 6.1.3	Impacts on Existing Nodes and Functionality

-	The VPLMN and the HPLMN as part of the roaming agreement need to ensure that the VPLMN AF can reach the NEF of the HPLMN to use the API defined in clause 4.15.6.10 of TS 23.502 [3] "Application guidance for URSP rules determination mechanisms"

-	UDM to include the DNN, S-NSSAI of the VPLMN in the list of Subscribed S-NSSAIs and to allow the DNN, S-NSSAI to work in LBO mode in the user subscription.

END OF CHANGES