3GPP TSG SA2 Meeting #151 (e-meeting) 	S2-2204465

16 – 20 May, 2022, Elbonia

# Source:	China Mobile

# Title:	New WID: BSF discovery for a MAC address in distributed BSF deployment

# Document for: Approval

Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	BSF discovery for a MAC address in distributed BSF deployment

Acronym:	TEI18_ BSF4MAC

Unique identifier:

Potential target Release:	Rel-18

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

3	Justification

For distributed BSF deployment scenraio, there is a problem for Ethernet PDU session that the AF can not find specific BSF through NRF with using specific MAC address, since the MAC address(es) can not be managed as IP address range. So the AF needs to request every BSF within a S-NSSAI+DNN, which is not sufficient when there are multiple BSFs deloyed in the network.

Another consideration is that for an ethernet PDU session, the UE may operate in bridge mode with regard to a LAN it is connecting to the 5GS, thus different MAC addresses may be used as source address of different frames sent UL over a single PDU Session (and destination MAC address of different frames sent DL over the same PDU Session). During UE mobility, the UE operate in bridge mode may change from one PDU session to another PDU session.

And for current TS 23.503:

In clause 6.1.3.5

The UE MAC address change trigger shall trigger a SMF interaction with the PCF if a new UE MAC address is detected or a used UE MAC address is inactive for a specific period during the lifetime of the Ethernet type PDU Session. The SMF reports that the UE MAC address change trigger was met and the new or released UE MAC address.

NOTE 4:	The SMF instructs the UPF to detect new UE MAC addresses or used UE MAC address is inactive for a specific period as described in TS 23.501 [2].

In clause 6.1.1.2.2

-	The PCF registers, updates and removes the stored information in the BSF using the Nbsf management service operations defined in TS 23.502 [3]:

-	For a PDU Session, the PCF ensures that it is updated each time an IP address is allocated or de-allocated to the PDU Session or, for Ethernet PDU Sessions supporting binding of AF request based on MAC address, each time it has been detected that a MAC address is used or no more used by the UE in the PDU Session.

The observation for the above information: For ethernet PDU session, when a UE operate in bridge mode, more than one MAC addresses may associated with one ethernet PDU session dynamically, and SMF will report to PCF and PCF will update BSF. But the AF doesn't knowBSF such information to help the subsequent AF request.

Therefore, we would like to propose TEI18 work item on distributed BSF support for PCF discovery via MAC address discovery.

4	Objective

The objective of this work item is to:

-	In a distributed BSF deployment how to optimise the procedure for AF discovering the BSF associated with a specific MAC address of an Ethernet PDU session

This work item will require 0. 75 TU(s) to discuss and agree on the corresponding CRs.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.502 | Service operation enhancement to support the MAC address discovery | SA#98 (Dec. 2022) |  |
| TS 23.503 | MAC address discovery for session binding | SA#98 (Dec. 2022) |  |

6	Work item Rapporteur(s)

Dan Wang (China Mobile), wangdanyjy@chinamobile.com

7	Work item leadership

SA2

8	Aspects that involve other WGs

None

9	Supporting Individual Members

Supporting IM name

China Mobile