SA WG2 Meeting #S2-151e(e-meeting)	S2-2204470

16 - 20 May 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	CATT

Title:	KI #5, New Sol: Support Rejected S-NSSAI request in specific TA in RA

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3

Abstract of the contribution: Introduce a new solution to key issue#5 in TR 23.700-41.

1. Discussion

This pCR is proposed to add a new solution to key issue#5: Improved support of RAs including TAs supporting Rejected S-NSSAIs.

In order to allow the UE to initiate a registration for an S-NSSAI which was rejected for the RA when the UE enters a TA that is part of the RA and the TA supports this S-NSSAI, in this solution, the AMF provides a TA list for the rejected S-NSSAI. When the UE is in the TA list, the UE can include the rejected S-NSSAI in the Requested NSSAI and initiate the Registration Procedure. When the UE moves out of the TA list, the PDU Session can be released or deactivated which is triggered by UE or AMF.

In order to reduce the signalling cost, when sending the rejected S-NSSAI and the TA list to the UE, the AMF may also provide an indication to indicate whether Registration is required when the UE is in the TA list and wants to use the S-NSSAI. The AMF decides the indication based on, e.g. whether NSSAA is required for the rejected S-NSSAI, whether NSAC is required for the rejected S-NSSAI and operator policy.

2. Proposal

It is proposed to add a new solution for KI#5: Improved support of RAs including TAs supporting Rejected S-NSSAIs.

* First change *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| Solution #1: Additional S-NSSAI associated with the PDU session | X |  |  |  |  |  |
| Solution #2: Slice Re-mapping Capabilities for Network Slice Service Continuity | X |  |  |  |  |  |
| Solution #3: Support of Network Slice Service continuity using SSC mode 3 | X |  |  |  |  |  |
| Solution #4: PDU Session on compatible network slice | X |  |  |  |  |  |
| Solution #5: PDU session handover to a target CN with an alternative S-NSSAI support | X |  |  |  |  |  |
| Solution #6: Extended SoR VPLMN Slice Information transfer to UEs |  | X |  |  |  |  |
| Solution #7: Enabling awareness of Network Slice availability in VPLMNs |  | X |  |  |  |  |
| Solution #8: Gracefully network slice termination |  |  | X |  |  |  |
| Solution #9: Support of a Network Slice with an AoS not matching existing TA boundaries |  |  | X |  |  |  |
| Solution #10: Associating a validity timer with a temporary slice |  |  | X |  |  |  |
| Solution #11: Enabling UEs to Request S-NSSAIs not uniformly available |  |  | X |  | X |  |
| Solution #12: Solution for Centralized Counting for Multiple Service Areas and 5GS-EPS Interworking |  |  |  | X |  |  |
| Solution #13: Hierarchical NSACF Architecture for Maximum UE/PDU Session number control |  |  |  | X |  |  |
| Solution #14: Maximum Number Distribution in multiple NSACFs |  |  |  | X |  |  |
| Solution #X: Support Rejected S-NSSAI request in specific TA in RA |  |  |  |  | X |  |

* Second change *

6.X	Solution #X: Support Rejected S-NSSAI request in specific TA in RA

6.X.1	Introduction

This solution addresses KI#5: Improved support of RAs including TAs supporting Rejected S-NSSAIs.

6.X.2	Functional Description

In order to allow the UE to initiate a registration for an S-NSSAI which was rejected for the RA when the UE enters a TA that is part of the RA and the TA supports this S-NSSAI, in this solution, the AMF provides TA(s) for the rejected S-NSSAI. When the UE is in the TA(s), the UE can include the rejected S-NSSAI in the Requested NSSAI and initiate the Registration Procedure. When the UE moves out of the TA(s), the PDU Session can be released or deactivated which is triggered by UE or AMF.

In order to reduce the signalling cost, when sending the rejected S-NSSAI and TA(s) to the UE, the AMF may also provide an indication to indicate whether Registration is required when the UE is in the TA(s) and wants to use the S-NSSAI. The AMF decides the indication based on, e.g. whether NSSAA is required for the rejected S-NSSAI, whether NSAC is required for the rejected S-NSSAI and operator policy. With this indication, the enhancements to AMF and UE are as follows:

-	If the indication indicates that the Registration is not required:

-	The AMF includes the rejected S-NSSAI and TA(s) to PCF. The PCF decides the NSSP rule for the rejected S-NSSAI. The AMF provides the rejected S-NSSAI, TA(s), indication indicating that the Registration is not required, NSSP rule to the UE.

-	When the UE is in the TA(s), the UE can uses the rejected S-NSSAI based on the NSSP rule without initiating the Registration Procedure.

-	When the UE moves out of the TA(s), the PDU Session can be released or deactivated which is triggered by UE or AMF.

-	If the indication indicates that the Registration is required:

-	The AMF provides the rejected S-NSSAI, TA(s), indication indicating that the Registration is required to the UE.

-	When the UE is in the TA(s), the UE can include the rejected S-NSSAI in the Requested NSSAI and initiate the Registration Procedure.

-	When the UE moves out of the TA(s), the PDU Session can be released or deactivated which is triggered by UE or AMF.

The AMF can update the TA(s) and/or indication above by initiating the UCU procedure. If the indication is changed from Registraion not required to Registration required, the AMF also requests the PCF to update the NSSP rule and provides the update NSSP rule to UE.

Editor's note:	It is FFS how to support roaming.

6.X.3	Procedures

6.X.3.1	Enhancements to Registration procedure

In order to allow the UE to initiate a registration for an S-NSSAI which was rejected for the RA when the UE enters a TA that is part of the RA and the TA supports this S-NSSAI, enhancements to the Registration procedure are as follows:

-	Step 21: AMF includes the TA(s) associated with the rejected S-NSSAI, may also include an indication indicating whether Registration is required when the UE is in the TA(s) and wants to use the rejected S-NSSAI.

6.X.3.2	Enhancements to UE Configuration Update procedure

In order to allow the UE to initiate a registration for an S-NSSAI which was rejected for the RA when the UE enters a TA that is part of the RA and the TA supports this S-NSSAI, enhancements to the UE Configuration Update procedure are as follows:

-	Step 1: AMF includes the rejected S-NSSAI, the TA(s), may also include an indication indicating whether Registration is required when the UE is in the TA(s) and wants to use the rejected S-NSSAI.

6.X.3.3	Enhancements to PDU Session Release Procedure

Enhancements to the PDU Session Release Procedure: add a new trigger for UE initiated procedure and AMF initiated procedure, i.e. the S-NSSAI associated with the PDU Session is the rejected S-NSSAI only available in TA(s) and the UE moves out of the TA(s).

NOTE 1:	The AMF may also decide to deactivate the PDU Session, in this case, the existing procedure in clause 4.3.7 in TS 23.502 can be re-used.

6.X.4	Impacts on services, entities and interfaces

AMF:

-	Provide the rejected S-NSSAI and TA(s) to UE.

-	Decide an indication to indicate whether Registration is required when the UE is in the TA(s) and wants to use the S-NSSAI and provide the indication to UE.

UE:

-	Receive the rejected S-NSSAI and TA(s) from AMF, may also receive an indication to indicate whether Registration is required when the UE is in the TA(s) and wants to use the S-NSSAI.

-	Initiates the Registration Request to request the rejected S-NSSAI when UE enters the TA(s) or requests PDU Session related to the rejected S-NSSAI when UE enters the TA(s).

* End of changes *