SA WG2 Meeting #S2-151e(e-meeting)	S2-2204479r02

16 - 20 May 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	CATT

Title:	KI #5, Sol #8: Update to remove ENs

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR

Abstract of the contribution: Update solution #8 to solve FFS.

1. Discussion

1.1	Background

The integrated access and backhaul feature is introduced in Rel-16, when UE access via IAB-node, the UE positioning is supported as follows:

-	The positioning procedures defined in TS 23.273 are re-used.

-	The UE positioning procedure defined in RAN WG is enhanced to add the interaction between gNB-DU and gNB-CU which is defined in clause 8.13 in TS 38.473.

In Rel-16, the location of IAB-node is static. The LMF knows the detailed information (e.g. TRP position) for TRPs hosted by the gNB-DU (i.e. IAB-node) via the TRP Information Exchange operation and calculates UE location using the static TRP position.

In Rel-18, when UE accesses via mobile base station relay which is a mobile IAB-node, the main difference with Rel-16 feature is that the location of the IAB-node may change. So the LMF has to obtain the IAB-node location which is used to calculate UE location. This part is not clearly described in solution #8, thus there are some FFS in solution #8. This pCR is proposed to update solution#8 to solve FFS.

1.2	Proposal

In order to realize that the LMF obtains mobile base station relay location to calculate UE location, it is proposed that:

-	For UL positioning, the mobile base station relay provides its location to LMF via NRPPa message. When the location is received, the LMF decides that the UE is accessing the network via a mobile base station relay and uses the location to calculate UE location. In this case, the mobile base station relay initiates the 5GC-MO-LR procedure to obtain its location.

-	For DL positioning, the mobile base station relay provides UE ID (i.e. GPSI) in the NRPPa message to LMF. The LMF acts as LCS Client to request the location of mobile base station relay from GMLC.

2. Proposal

It is proposed to update solution #8 to solve FFS.

* First change *

6.8	Solution #8: Support of positioning for UE accessing via mobile base station relay

6.8.1	General

This solution addresses the KI#5: Support of location services for UEs accessing via a mobile base station relay.

6.8.2	Functional Descriptions

For UEs accessing the network via a mobile base station relay (MBSR), when UE positioning is performed, in order to improve the accuracy of the UE location, the LMF obtains the location of the mobile base station relay which is used to calculate the UE location as follows:

-	For UL positioning, the mobile base station relay provides its location to LMF via NRPPa message. When the location is received, the LMF decides that the UE is accessing the network via a mobile base station relay and uses the location provided by the mobile base station relay to calculate UE location. In this case, the mobile base station relay initiates the 5GC-MO-LR procedure to obtain its location.

-	For DL positioning, the mobile base station relay provides UE ID (i.e. GPSI) in the NRPPa message to LMF. The LMF acts as LCS Client to request the location of mobile base station relay from GMLC.

6.8.3	Positioning procedure for UE accessing via mobile base station relay

Figure 6.8.3-1: Positioning procedure for UE accessing via mobile base station relay

0.	Triggers for UE positioning happen, e.g. the AMF receives Namf_Location_ProvidePositioningInfo Request from GMLC or receives MO-LR Request from UE.

1.	The AMF invokes a Nlmf_Location_DetermineLocation Request service operation towards the LMF.

The steps in box (A) (i.e. steps 2-8) are performed when UL positioning is used to obtain UE location.

2.	The LMF sends NRPPa message to NG-RAN.

3.	The NG-RAN sends NRPPa message to the mobile base station relay connected by UE.

4.	The mobile base station relay initiates the 5GC-MO-LR procedure in clause 6.2 in TS 23.273 to obtain its location.

5.	The mobile base station relay obtains measurements.

6.	The mobile base station relay sends NRPPa message including mobile base station relay location to NG-RAN.

7.	The NG-RAN sends NRPPa message including mobile base station relay location to LMF.

8.	The LMF decides that the UE connects to mobile base station relay based on the location in the received NRPPa message and uses the location to calculate UE location.

The steps in box (B) (i.e. steps 9-16) are performed when DL positioning is used to obtain UE location.

9.	The LMF sends NRPPa message to NG-RAN.

10.	The NG-RAN sends NRPPa message to mobile base station relay.

11.	The mobile base station relay returns the NRPPa message including GPSI of the mobile base station relay to NG-RAN.

12.	The NG-RAN sends NRPPa message including GPSI of the mobile base station relay to LMF.

13.	The LMF decides that the UE connects to mobile base station relay based on the GPSI in the NRPPa message. The LMF sends the Ngmlc_Location_ProvideLocation Request including the GPSI to GMLC to obtain the location of the mobile base station relay. The LMF also requests UE velocity.

NOTE 1:	If the serving LMF of UE selects DL positioning, in order to reduce the time difference error, it is recommended that the serving LMF of UE defers the UE positioning procedure (i.e. step 15) some time later after the step 13 is performed. The duration deferred depends on the implementation.

13a.	[Conditional] If MBSR is roaming in VPLMN, the (V)GMLC sends Ngmlc_Location_ProvideLocation Request including the GPSI to HGMLC to obtain the location of the mobile base station relay.

14.	The steps 2-24b in 5GC-MT-LR procedure in clause 6.1.2 in TS 23.273 are performed. After this step, the serving LMF of UE obtains the MBSR location, MBSR velocity and MBSR location time.

NOTE 2:	The step 14 reuses the existing procedure in TS 23.273, no enhancement is introduced.

15.	The LPP procedure is performed for LMF to obtain measurements from UE.

16.	The LMF calculate UE location using the location of the mobile base station relay obtained in step 14 and the measurements obtained in step 15.

NOTE 3:	Assume that the positioning measurements of UE are obtained at time T1, the MBSR location time at step 14 is time T2. In order to reduce the time difference error, it is recommended that the serving LMF of UE calculates MBSR location at the time T1 based on the MBSR location, MBSR velocity and time T2 firstly, and calculates UE location based on the positioning measurements of UE at time T1 and the MBSR location at time T1.

17.	The LMF sends the Nlmf_Location_DetermineLocation Response including UE location to AMF.

6.8.4	Impacts on services, entities, and interfaces

LMF:

-	obtain location of the mobile base station relay and use the location to calculate UE location.

-	obtain UE ID of the mobile base station relay, requests location of the mobile base station relay by sending location request including the UE ID to GMLC and use the location to calculate UE location..

Mobile base station relay: provides its location or UE ID to LMF.

* End of change *