SA WG2 Meeting #151E	S2-2204497r02

May 16 – 20, 2022, Elbonia	(revision of S2-220xxxx)

Source:	China Telecom

Title:	KI#2, New Sol: 5GC assistance to UE on performing AI/ML task

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract of the contribution: This contribution proposes a solution to the KI#2 & KI#4 for 5GC assistance to UE on performing AI/ML task.

# Discussion

This paper proposes a solution for KI#2 on 5GC information exposure to UE and KI#4 on enhancing External Parameter Provisioning.

The KI#2 is to study:

-	Whether and for which purpose there is a need for 5GC to expose information or data analytics to the UE, and what are the information or data analytics that can be provided by 5GC to the UE to assist the Application AI/ML operation at the UE side.

-	How the 5GC exposes such information or data analytics to the UE.

The KI#4 is to study:

-	Whether and what additional parameters can be provisioned to 5GC by an external party for the assistance to Application AI/ML operation.

-	How additional parameters are provisioned to 5GC by an external party.

-	How provisioned parameters are retrieved or maintained or used by 5GC NFs (e.g. UDM, AMF or SMF).

NOTE:	Privacy aspects will be handled by SA WG3.

# Proposal

The following text is proposed to be added to TR 23.700-80.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | X |  |  |  |  | X |  |
| 2 |  | X |  |  |  |  |  |
| 3 |  | X |  |  |  |  |  |
| 4 |  | X |  |  |  |  |  |
| 5 |  | X |  |  |  |  |  |
| 6 |  |  | X |  |  |  | X |
| 7 |  |  | X |  |  | X |  |
| 8 |  | X | X |  |  |  |  |
| 9 |  |  |  | X |  |  |  |
| 10 |  |  |  |  | X |  |  |
| 11 | X |  |  |  | X |  |  |
| 12 |  |  |  |  | X |  |  |
| 13 |  |  |  |  | X |  |  |
| 14 |  |  |  |  |  | X | X |
| 15 |  |  |  | X |  | X |  |
| 16 | X |  |  |  | X | X | X |
| 17 |  |  |  |  |  |  | X |
| 18 |  |  |  |  |  |  | X |
| 19 |  |  |  |  |  |  | X |
| 20 |  |  |  |  |  |  | X |
| 21 |  |  |  |  |  |  | X |
| 22 |  |  |  |  |  |  | X |
| 23 |  |  |  |  |  |  | X |
| 24 |  |  |  |  |  |  | X |
| 25 |  |  |  |  |  |  | X |
| 26 |  |  |  |  |  | X | X |
| 27 |  |  |  |  |  |  | X |
| #XX |  | X |  | X |  |  |  |

# * * * * Next change (All new text)* * * *

## 6.X	Solution #X: < 5GC assistance to UE on performing AI/ML task >

## 6.X.1	Description

This solution is proposed to address Key Issue #2: 5GC information exposure to UE and Key Issue #4: Enhancing External Parameter Provisioning.

In some scenarios, AI/ML based services need stable network quality to ensure service sustainability, for example, as described in TS 22.261 [2], UE may upload or download AI/ML models or learning data from application servers (i.e. online model distribution is needed). If UE can get information about when the network can provide sufficient network resource, it can make better decisions on AI/ML operations (e.g. whether begin to perform an AI/ML service). It is helpful to expose some information to UE.

However, network information exposure directly to UE may cause security issues and some network information is not useful for UE to make decisions on AI/ML operations since how to allocate the network resource is determined by network rather than UE. Thus, it is important to study what information is valuable to be exposed to UE.

On the other hand, network tends to provide service for each UE according to the QoS requirement. However, when many AI/ML applications are running over 5G mobile network, these applications may generate large data traffic and give heavy load to network.

To reduce the impact on other services in the network and guarantee the quality of AI/ML based service, this solution proposes that the 5GC exposes a recommended service transmission time duration for UE to suggest when to perform AI/ML tasks, which means the network is more likely to provide sufficient network resources in this time duration. 5GC will consider AI/ML task related information and network resource to select a relatively idle time duration of both network and UE as recommended service transmission time duration, so that UE can perform these AI/ML tasks in this time duration.

In this solution, the two main functionalities are proposed to take into consideration:

- UE and AI/ML application servers negotiate AI/ML task in application layer and provision following AI/ML task related information to 5GC (UDM):

  - AI/ML application identifier: identifies the AI/ML application.

  - AI/ML task identifier: identifies AI/ML task to be performed by UE. It is AI/ML related traffic which needs to be transferred over 5GS and used to distinguish different tasks for the same application.

  - AI/ML task duration: defines the estimated time duration how long the task will take.

  - AI/ML task valid time: defines the time period during which the task is valid.

- In addition to above information, 5GC (SMF) considers network performance by requesting analytics from NWDAF to determines recommended service transmission time duration. SMF exposes recommended service transmission time duration to UE. The analytics ID can be requested from NWDAF includes:

  - Network Performance

  - NF load information

  - Service Experience

  - UE communication

### 6.X.2	Procedures

#### 6.X.2.1 Procedure for AF provisioning information to 5GC

Figure 6.X.2-1 illustrates the procedure for AF provisioning information to 5GC.

Figure 6.X.2-1: Procedures for AF provisioning information to 5GC

- UE and AI/ML application servers negotiate the AI/ML task to be performed and the task related parameters in 6.X.1. AF provides AI/ML task related parameters (UE ID or UE group ID, AI/ML application identifier, AI/ML task identifier, AI/ML task duration, AI/ML task valid time) to be created or updated in a Nnef_ParameterProvision_Create or Nnef_ParameterProvision_Update Request to the NEF.

- If the AF is authorised by the NEF to provision the parameters, the NEF requests to create, update and store provisioned parameters as part of the subscriber data via Nudm_ParameterProvision_Create, Nudm_ParameterProvision_Update or Nudm_ParameterProvision_Delete Request message.

If the AF is not authorised to provision the parameters, then the NEF continues in step 4 indicating the reason to failure in Nnef_ParameterProvision_Create/Update Response message.

- UDM responds the request with Nudm_ParameterProvision_Create/Update Response. If the procedure failed, the cause value indicates the reason.

- NEF responds the request with Nnef_ParameterProvision_Create/Update Response. If the procedure failed, the cause value indicates the reason.

#### 6.X.2.2 Procedure for 5GC exposing information to UE

Figure 6.X.2-2 illustrates the procedure for 5GC determining recommended service transmission time duration and exposing it to UE.

Figure 6.X.2-2: Procedures for 5GC exposing information to UE

- The PDU session between UE and AI/ML application server has been established and SMF subscribes to be notified when the subscriber data for the UE is changed from UDM. If UE has ability to perform AI/ML service, it will indicate to request AI/ML service notification.

- When the subscriber data is changed by the procedure as 6.X.2.1, the UDM notify SMF in a Nudm_SDM_Notification including AI/ML task related parameters (UE ID or UE group ID, AI/ML application identifier, AI/ML task identifier, AI/ML task duration, AI/ML task valid time).

- The SMF considers AI/ML task related parameters and requests analytics (Network Performance, NF load information, Service Experience, UE communication) from NWDAF by setting time when analytics information is needed within the AI/ML task valid time.

- The NWDAF collects input data and derives analytics output as TS23.288[6].

- The NWDAF sends the output to the SMF.

- The SMF considers the output of analytics, AI/ML task duration, AI/ML task valid time and operator configuration to determine recommended service transmission time duration.

- The SMF sends the AI/ML service notification including AI/ML application identifier, AI/ML task identifier and recommended service transmission time duration to UE by NAS layer.

- UE may perform AI/ML task during the recommended service transmission time duration by using existing PDU session or establishing a new PDU session.

### 6.X.3	Impacts on Existing Nodes and Functionality

AF:

- Support on provisioning AI/ML based service information to 5GC.

UDM:

- Support on modifying AI/ML related subscriber data by AF.

- Support on notifying AI/ML related subscriber data to SMF.

SMF:

- Support on determining recommended service transmission time duration by getting notification form UDM and requesting analytics from NWDAF.

- Support on sending AI/ML service notification to UE.

UE:

- Support on receiving AI/ML service notification from 5GC via NAS.

# * * * * End of changes * * * *