SA WG2 Meeting #150e	S2-2204512r01

May 16th – 21th, 2022	  (revision of S2-220xxxx)

Source: 	China Mobile

Title: 	KI #2, New Sol: Support QoS parameters exposure by UPF

Document for: 	Approval

Agenda Item: 	9.25

Work Item / Release:	FS_UPEAS/ Rel-18

Abstract of the contribution: Solution proposal for UPF exposing QoS parameters.

1 Discussion

The solution proposal presented here addresses the Key Issue #2, which reads:

-	How and what specific information the UPF can expose to NEF/Local NEF/trusted AF, e.g. the information which can be exposed in Rel-18 SA WG2 studies such as FS_EDGE_Ph2 and FS_XRM, or information which has been justified for exposure.

This paper proposes to support the UPF exposes QoS parameters to NEF/local NEF/trusted AF directly to facilitate the local AF requesting information with low latency.

2 Proposal

It is proposed to update TR 23.700-62 v0.2.0 as follows.

****************************First change****************************

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> |
| #1: UPF event exposure service framework enhancements to support registration, deregistration and discovery via NRF | X |  |
| #2: UPF event exposure service for TSC management |  | X |
| #3: using the proper subscription mechanism depending on the event targeted by the UPF event consumer |  | X |
| #4: upgrading N4 to pass necessary event filtering information to the UPF |  | X |
| #5: registering UPF(s) serving a PDU session at UDM |  | X |
| #6: Determining the UPF(s) that serve a UE address |  | X |
| #7: Support to existing (R16-R17) data analytics with PDU Session Data Usage Events |  | X |
| #8: Support to existing (R16-R17) data analytics with QoS Flow level measurements |  | X |
| #9: NWDAF collects information from UPF by event exposure |  | X |
| #10: UPF event exposure service to NWDAF |  | X |
| #11: UPF event exposure service to NWDAF |  | X |
| #X: QoS parameters exposure by UPF |  | X |

****************************Second change (all text is new) ****************************

6.X	Solution #X: QoS parameters exposure by UPF

6.X.1	Key Issue mapping

This is a solution for KI#2.

6.X.2	Description

The QoS parameters of a QoS flow is determined by UPF eventually. Thus, the UPF should have the ability to handle some QoS flow by itself, e.g., providing the QoS parameters configured locally in UPF.

The UPF may be instructed to report information about a QoS flow directly to the NEF, i.e. by passing the SMF and the PCF. This reporting may target a third-party AF that wants to know the QoS parameters (i.e., 5QI, Maximum Bitrate, Guaranteed Bitrate, packet rate, Usage report) used by UPF to guarantee the QoS flow. By exposing this information directly from UPF to NEF, the AF can know the QoS parameters with flexibility and low latency.

6.X.3	Procedures

Figure 6.X.3-1: QoS parameters exposure by UPF

1.	The AF issues an Nnef_EventExposure_Subscribe (Application ID, GPSI, IP address) service operation to request QoS parameters from the L-NEF.

2.	The L-NEF finds the appropriate UPF(s) by using the UPF selection method targeting the PDU sessions of a certain UE with information of IP address, as described in clause 6.1.2.5.1.

3.	The L-NEF sends the Nupf_EventExposure_Subscribe request to the UPF to request QoS parameters.

4.	The UPF responds the QoS parameters to L-NEF over the Nupf_EventExposure_Notify service operation.

5.	The L-NEF responds the QoS parameters to the AF over the Nnef_EventExposure_Subscribe service operation.

6.X.4	Impacts on services, entities and interfaces

UPF:

-	Newly introduced UPF Service Operations to support AF/Local NEF/NEF to subscribe the UPF event exposure service directly.

-	Expose QoS parameter related information to AF/Local NEF/NEF directly.

**End of change**