3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204527r02

Elbonia, 16th – 20th May, 2022	(revision of S2-220xxxx)

Source:	ZTE

Title:	FS_eNS_Ph3 KI#1 Update on Solution 1

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: This paper updates solution 1 to resolve FFS

1. Introduction/Discussion

This paper propose solution update to resolve FFS in solution 1.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-41 V0.2.0.

# * * * * First change * * * *

6.1	Solution #1: Additional S-NSSAI associated with the PDU session

6.1.1	Introduction

This solution aims to address the key issues#1: Support of Network Slice Service continuity, particularly it reuses the concept that the PDU session can be associate with two S-NSSAIs (based on values provided in mapping of Allowed NSSAI in case of roaming). The scenario covers the case where NFs (e.g. AMF, SMF, UPF...) in a network slice instance support multiple S-NSSAIs. How to split the resource depends on implementation. It is possible that the resource of one S-NSSAI is getting congested, while other S-NSSAI is not (e.g. there is a fixed allocation of resources to each S-NSSAI in the NF instances). This solution proposes that when one S-NSSAI is congested, the network can associate the PDU session with additional S-NSSAI in order to provide service continuity during mobility scenario (scenario 2d), so the resource allocated in SMF and/or UPF for one S-NSSAI can be shared by other S-NSSAI if other S-NSSAI is getting congested. This solution can also be used for PDU Session establishment procedure in scenario 1b.

NOTE:	This solution may be extended to cover no mobility scenarios 1b and 1c described in clause 5.1.1

Editor's note:	It is FFS whether and how the PDU Session can be associated with more than one S-NSSAI within a PLMN

6.1.2	Functional Description

In this solution it is assumed that a network slice instance may be associated with multiple S-NSSAIs in the PLMN, and each S-NSSAI may use different network resources. The AMF, SMF and UPF is configured with which S-NSSAIs are associated with the network slice instance. When the UE establishes PDU session it determines the requested S-NSSAI based on the URSP rules. The PDU Session is initially associated with this requested S-NSSAI. When this requested S-NSSAI become congested in the area of interest, the network may associate the PDU session with an additional S-NSSAI which can be associated with the network slice instance and is not congested in this area. The network then notifies the NG-RAN to allocate radio resource of this additional S-NSSAI for the PDU session, instead of original requested S-NSSAI. After the S-NSSAI congestion is mitigated, the network may remove the additional S-NSSAI of the PDU Session.

The AMF may determine the S-NSSAI is congested in the area of interest and the impacted PDU sessions based on the load level information or service experience for a Network Slice or network slice instance provided by the NWDAF.

When the AMF determines to associate the PDU session with an additional S-NSSAI, and if there is an I-SMF, the AMF provides the additional S-NSSAI to the I-SMF and the I-SMF does not provides the additional S-NSSAI to the SMF. If there is no I-SMF, the AMF  provides the additional S-NSSAI to SMF. The I-SMF/SMF uses this additional S-NSSAI in the N2 information and send it to NG-RAN. The I-SMF/SMF may also send the additional S-NSSAI to the UPF.

This additional S-NSSAI is not provided to the UE, so from UE perspective the PDU session is still associated with the original requested S-NSSAI and therefore no impact on the UE side. Since the PDU session is still associated with the original requested S-NSSAI, the anchor SMF/UPF and the IP address is not changed so service continuity is ensured.

NOTE 1:	From RAN perspective, the PDU session is only associated with the additional S-NSSAI

For PDU Session with home routed scenario, the AMF may determine to associate this PDU session with additional S-NSSAI in the VPLMN. The AMF doesn't update the slice mapping between VPLMN and HPLMN in the UE.

NOTE 2:	When the AMF detects that the original requested S-NSSAI is not congestd, the AMF may remove the additional S-NSSAI associated with the PDU session.

During PDU Session establishment procedure, if the requested S-NSSAI is congested in this area, the AMF may also determine to associate the additional S-NSSAI with the PDU session. During the N2 based handover, the target AMF may reselect a different AMF supporting the additional S-NSSAI and forward the handover message to this AMF.

6.1.3	Procedures

6.1.3.1	Additional S-NSSAI handling during Handover Procedure

The target AMF is configured that both S-NSSAI#1 and S-NSSAI#2 are associated with one network slice instance.

Figure 6.1.3.1-1: Additional S-NSSAI handling during Handover Procedure

1.	The UE establishes a PDU session with the requested S-NSSAI#1 via S-RAN.

2.	S-RAN performs a UE measurement and determines that a handover to the T-RAN is needed. The S-RAN sends a handover required message(T-RAN node information, a source to target transparent container, an SM N2 information list, PDU session ID(s)) to the AMF via the N2 interface.

3.	The source AMF selects a target AMF based on the T-RAN node information and create UE context in the target AMF.

4.	If the target AMF is notified that the S-NSSAI#1 is under congestion in the CN and the service continuity of the PDU session is required, the target AMF checks if the S-NSSAI#2 is supported in the T-RAN node. If it is supported the target AMF determines to associate the PDU session with additional S-NSSAI#2.

5.	The target AMF sends an Nsmf_PDUSession_UpdateSMContext request message (the PDU session ID, the S-NSSAI#2, N2 SM Information) to the SMF. The SMF associates the S-NSSAI#2 with the PDU session.

6.	The SMF may modify the N4 session to update the N4 session information in the UPF.

7.	The SMF sends a Nsmf_PDUSession_UpdateSMContext response message (PDU session ID, N2 SM Information) to the target AMF. The N2 SM information includes the S-NSSAI#2 received from the AMF.

8.	The rest steps in N2 based handover procedure are performed.

6.1.3.2	Additional S-NSSAI handling during PDU Session Establishment Procedure

The target AMF is configured that both S-NSSAI#1 and S-NSSAI#2 are associated with one network slice instance.

Figure 6.1.3.2-1: Additional S-NSSAI handling during PDU Session establishment Procedure

1.	The UE establishes a PDU session with the requested S-NSSAI#1 via RAN.

2.	The AMF is notified that the S-NSSAI#1 is under congestion in the CN, the AMF checks if the S-NSSAI#2 is supported in the RAN node. If it is supported the AMF determines to associate the PDU session with additional S-NSSAI#2.

3.	The AMF sends an Nsmf_PDUSession_CreateSMContext request message (the PDU session ID, requested S-NSSAI#1, S-NSSAI#2, SM NAS container) to the SMF. The SMF determines that the PDU session is associated with both S-NSSAI#1 and S-NSSAI#2.

4.	The SMF creates the N4 session to update the N4 session information in the UPF.

5.	The SMF sends a Nsmf_PDUSession_CreateSMContext response message to AMF.

6.	The SMF sends Namf_Communication_N1N2MessageTransfer(PDU session ID, N2 SM Information) to the AMF. The N2 SM information includes the S-NSSAI#2 received from the AMF.

7.	The AMF sends the N2 message to the RAN node, including the N2 SM information.

8.	The rest steps in PDU Session establishment procedure are performed.

6.1.4	Impacts on existing entities and interfaces

AMF:

-	determines to associate additional S-NSSAI with the PDU Session

SMF/I-SMF/V-SMF:

-	associate additional S-NSSAI with the PDU Session

UPF: None

NG-RAN: None

UE: None

# * * * * End of changes * * * *