3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204530r03

Elbonia, 16th – 20th May, 2022	(revision of S2-220xxxx)

Source:	ZTE

Title:	FS_eLCS_Ph3 KI#2 Update on Solution 4

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3 / Rel-18

Abstract: This paper updates solution 4 to resolve FFS

1. Introduction/Discussion

This paper propose solution update to resolve FFS in solution 4.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-71 V0.2.0.

# * * * * First change * * * *

6.4	Solution #4: Direct communication between LMF and RAN node

6.4.1	Introduction

This solution aims to address the key issues#2: enhanced positioning architecture for NPN deployment. Particularly this solution address the following questions:

-	How to realize low latency positioning procedure under NPN deployment.

-	How to achieve reliable and secure location result delivery and exposure, e.g. UE location not exposed to the public network.

6.4.2	Functional Description

The following is the architecture figure in this solution.

Figure 6.4.2: Reference architecture for Location Services

In this solution, the LMF communicates  with the RAN node via a local AMF. . The LMF use local AMF  to exchange the Network Positioning Message between the RAN and LMF for network assisted or network based positioning procedure, and obtaining Network Positioning Message for Non-UE Associated Network Assistance Data procedure, without going through the AMF. The LMF directly communicates with the GMLC to exposure the UE location to GMLC directly without going through the AMF.

The TNL association between the RAN node and the local AMF is pre-established via configuration. For UE associated Network Positioning Message the AMF provides the RAN NGAP UE ID to LMF. The Local AMF allocates Local AMF NGAP UE ID and establishes NGAP association between the RAN node and Local AMF.

Editor’s note: Whether “local AMF” in this solution is same as AMF defined in TS 23.501 and TS 23.502 is FFS.

The Local AMF supports the management of N2 interface, i.e. the TNL association and the NGAP association between the RAN node and Local AMF. The weight factor of TNL association with the Local AMF shall be set to zero so the RAN node shall not select the Local AMF for initial N2

When the UE moves to a new RAN node, the AMF provides the new RAN NGAP UE ID to LMF. The Local AMF allocates a new local AMF NGAP UE ID and establish a new NGAP association towards the new RAN node.

The GMLC notification address is provided from AMF to LMF. The LMF notifies the UE location estimation towards the GMLC directly.

NOTE 2:	This solution assumes that the LMF/Local AMF and the RAN node are in the same trusted domain.

6.4.3	Procedures

6.4.3.1	Network Assisted Positioning Procedure

Figure 6.4.3-1: Mobile Terminated Network Assisted Positioning Procedure

1.	Based on configuration the TNL association may be pre-established between the RAN node and the Local AMF/LMF.

2.	The GMLC invokes the Namf_Location_ProvidePositioningInfo service operation towards the AMF to request the current location of the UE. The service operation includes the SUPI, and client type and may include the required QoS and Supported GAD shapes.

3.	If the UE is in CM IDLE state, the AMF initiates a network triggered Service Request procedure as defined in clause 4.2.3.3 of TS 23.502 [3] to establish a signalling connection with the UE.

4.	The AMF selects the LMF and invokes the Nlmf_Location_DetermineLocation service operation towards the LMF to request the current location of the UE. This service operation may include the RAN node ID, and the RAN NGAP UE ID. This service operation may also include the GMLC notification address to receive the UE location.

5.	The LMF response Nlmf_Location_DetermineLocation response.

6.	The LMF/Local AMF sends a Network Positioning message in the N2Transport message via the TNL association to the RAN node to request position related information from the RAN node. The local AMF allocates an Local AMF NGAP UE ID and includes both the RAN NGAP UE ID and the Local AMF NGAP UE ID in the N2 Transport message

7.	The target RAN node returns any position related information to the LMF/Local AMF in a Network Positioning message included in an N2 Transport message. The N2 Transport message includes the Local AMF NGAP UE ID.

8.	The LMF invokes the Nlmf_Location_EventNotify service operation towards the GMLC notification address received from AMF in step 4.

6.4.3.2	Obtaining Non-UE Associated Network Assistance Data

Figure 6.4.3.2: Non-UE Associated Network Assistance Data procedure

1.	Based on configuration the TNL association may be pre-established between the RAN node and the LMF/Local AMF .

2.	The LMF/Local AMF sends a Network Positioning message in the Non UE associated N2 Transport message via the TNL association to the RAN node to request position related information from the RAN node.

3.	The target RAN node returns any position related information to the LMF in a Network Positioning message included in an Non UE associated N2 Transport message.

6.4.4	Impacts on services, entities, and interfaces

LMF:

-	Exchange the Network Positioning message with RAN node via Local AMF

-	Send the UE location to the GMLC.

AMF:

-	Provides RAN node ID  and RAN NGAP UE ID to the LMF.

Local AMF:

-	Manage the N2 interface with the RAN node.

RAN node:

-	Support NGAP association with the AMF and NGAP association with the Local AMF

-	Exchange the Network Positioning message with LMF via Local AMF.

# * * * * End of changes * * * *