3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204542r01

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI #8, New solution: LCS session continuity due to UE mobility

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	5G_eLCS_Ph3 / Rel-18

Abstract: this is the new solution for LCS session continuity

1. Introduction/Discussion

This pCR proposes a new solution to address key issue 8.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-71.

# * * * * First change (all texts new)* * * *

## 6.X	Solution #X: LCS session continuity due to UE mobility

### 6.X.1	Key Issue mapping

This solution address key issue#8: LCS session continuity.

### 6.X.2	Description

LCS interworking between EPS and 5GS architecture is illustrated as follows:

Figure 6.x.2-1 architecture of the solution

Solution principle for LCS interworking between EPS and 5GS:

- When N26 interface is supported, and UE performs interworking procedure from 5GS to EPS, AMF will send the UE location context to MME.

- MME, based on the UE location context, determines an E-SMLC which handles the deferred location request for the UE. E-SMLC is obtained of the LCS correlation ID retrieved from the AMF, and sends an LPP message to UE with the same correlation ID.

### 6.X.3	Procedures

### 6.X.3.1	LDR Continuity Procedure when UE moves from 5G to 4G

Target UE initially resides in the 5G network, and UE is associated with an activated LDR request from an external AF, when UE moves from 5G to 4G, the corresponding LDR request information shall be sent from 5G to 4G simultaneously.

Figure 6.X.3.1-1: LDR Continuity Procedure when UE moves from 5G to 4G

- The AMF sends a Forward Relocation Request as in Step 2 in clause 5.5.1.2.2 (S1-based handover, normal) in TS 23.401. And this request shall include the LDR request information for target UE.

- The AMF sends a LocationEventNotify request to GMLC, and this request shall contain the indication that UE has moved to 4G, the SUPI that identifies the target UE, and also the serving MME address.

- Once 4G network has the corresponding LDR request information, 4G core network can continue to provide the LDR service to external AF via typical Deferred EPC-MT-LR procedure for Periodic and Triggered Location described in clause 9.1.19.1 in TS 23.271.

### 6.x.3.2	LDR Continuity Procedure when UE moves from 4G to 5G

Target UE initially resides in the 4G network, and UE is associated with an activated LDR request from an external AF, when UE moves from 4G to 5G, the corresponding LDR request information shall be sent from 4G to 5G simultaneously.

Figure 6.x.3.2-1: LDR Continuity Procedure when UE moves from 5G to 4G

- The MME sends a Forward Relocation Request as in Step 2 in clause 5.5.1.2.2 (S1-based handover, normal) in TS 23.401. And this request shall include the LDR request information for target UE.

- The MME sends a LocationEventNotify request to GMLC, and this request shall contain the indication that UE has moved to 5G core network, the IMSI that identifies the target UE, and also the serving AMF address.

- Once 5G network has the corresponding LDR request information, 5G core network can continue to provide the LDR service to external AF via Deferred 5GC-MT-LR Procedure for Periodic, Triggered and UE Available Location Events described in clause 9.1.19.1.

6.x.3.3	LDR Continuity Procedure when UE moves between NG-RAN

Figure 6.X.3.3-1 Support of service continuity when UE changes serving cell

- Same as step 1-13 in clause 6.3.1.

- LMF sends Namf_EventExposure subscribe (LCS correlation ID, event type=UE location (cell ID)) to AMF.

- AMF sends location reporting control message to NG-RAN, to request for report of change of UE cell ID, as defined in TS 38.413[X].

- NG-RAN detects the UE changes its serving cell.

- NG-RAN reports the latest cell ID of the serving cell to AMF.

- AMF sends Namf_EventExposure Notify message (cell ID, LCS correlation ID) to LMF.

- If UE handovers to a target NG-RAN.

- AMF obtains the cell ID of the serving cell in the target NG-RAN.

- AMF sends Namf_EventExposure Notify (cell ID, LCS correlation ID) to LMF.

### 6.X.4	Impacts on Services, Entities, and Interfaces

Editor's note:	This clause is FFS.

# * * * * Second change * * * *

6.0	Mapping Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |  |  |  |
| 1 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| 18 |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |
| x |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |

# * * * * End of changes * * * *