3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204543r01

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI #2, Sol #3: Update to remove ENs

Document for:	Approval

Agenda Item:	9.5

Work Item / Release:	FS_UAS_Ph2 / Rel-18

Abstract: this is the solution update for MBS-based UAV remote ID broadcasting

# 1. Introduction/Discussion

This pCR proposes updates on solution#3.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-58.

# * * * * First change * * * *

## 6.3	Solution #3: MBS-based UAV remote ID broadcast

### 6.3.1	Key Issue mapping

This solution address key issue#2: Support of Broadcast Remote ID

### 6.3.2	Description

When UAV UE registers to the network, based on TS 23.256, the AMF is aware of the UAV subscription, and interacts with the USS for authentication an authorization.

NOTE 1: this solution applies only to UAVs that have a 3GPP subscription, have Uu capability, are in coverage, and are registered to the 3GPP system.

NOTE 2: this solution applies only to receivers of remote ID broadcast that have a 3GPP subscription, have Uu capability, are in coverage and are registered to the 3GPp system

Overview of the solution:

-	USS/UTM knows the UAV location based on UAV location reporting, defined in clause 5.3.2, TS 23.256. In detail, USS/UTM triggers the deferred location request indicating a location event, to subscribing for notification when the UAV moves in/out of a geographic area.

-	USS/UTM to broadcast remote identification via MBS system in specific area. USS/UTM may establish a single broadcast session to deliver all the broadcast remote ID(s).

-	One or multiple Broadcast Session can be established by the USS/UTM. If multiple Broadcast Sessions are established, the content for broadcast in each session may be different. The difference of content is determined by the USS/UTM, e.g. only broadcasting the content for a certain geographic area.

-	USS/UTM triggers location request to the 5G network for the UE location reporting. This is used by the USS/UTM to n

-	The broadcast session establishment may take place by the USS/UTM(server) at any time, or triggered by a UAV registration (after UUAA procedure)

-	UAV UE also provides it paired UAV-C ID to the USS/UTM, in order to allow the USS/UTM to combine the pairing information for broadcasting.

Editor’s Note: what UAV-C ID is used and provided to the USS/UTM is FFS, since the remote ID and the CAA-Level UAV ID apply only to the UAV.

Editor’s Note: how the solution applies when the UAV-C is not a 3GPP UE is FFS.

### 6.3.3	Procedures

Figure 6.3.3-1 MBS-based UAV remote ID broadcast

Step 0 broadcast session is established using mechanism defined in TS 23.247. The USS/UTM (server) firstly requests to the NEF/UAS NF for the broadcast session establishment.

Step 1 UAV UE registers to the network and establishes user plane connection with USS/UTM (server).

Step 2 UAV UE reports its remote ID to the server, and paired UAV-C ID, via user plane. Optionally, the report message includes its speed, heading, etc.

Step 3 if the broadcast session is not yet established, triggered by step 2, USS/UTM requests the 5G network to establish a broadcast session using mechanism defined in TS 23.247. USS/UTM (server) firstly requests to the NEF/UAS NF for the broadcast session establishment.

After the broadcast session is established, USS/UTM (server) starts to broadcast the remote ID of UAV.

### 6.3.4	Impacts on Services, Entities, and Interfaces

Editor's note: This clause captures impacts on existing 3GPP services, entities, and interfaces.

# * * * * End of changes * * * *