SA WG2 Meeting #150E	S2-2204610r03

16 – 20 May 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#3, New Sol: ECN for scalable congestion control for XR/media services

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: This contribution proposes a solution to use L4S ECN for scalable congestion control.

# 1. Introduction

This paper proposes a solution for KI#3:

“This Key Issue will study mechanisms that enable codec/rate adaptation to meet requirements for services.

-	Study the use cases and whether enhancements to the exposure framework are needed for such use cases;

-	What 5GS information needs to be exposed to enable application codec/rate adaptation;

-	How to expose 5GS information for application codec/rate adaptation.

NOTE 1:	Parameters for exposure may be coordinated with RAN and SA WG4.

NOTE 2:	Potential overlap with enhancements done in FS_EDGE_Ph2/FS_UPEAS should be considered.

”

# 2. Proposal

This paper proposes the following updates to TR 23.700-60 clause 6.

* Start of change *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[6]	3GPP TR 22.847: "Study on supporting tactile and multi-modality communication services; Stage 1".

[7]	RFC 8311

[8]	Low Latency, Low Loss, Scalable Throughput (L4S) Internet Service: Architecture, Briscoe et al, Internet-draft, IETF, Nov 2020, https://datatracker.ietf.org/doc/draft-ietf-tsvwg-l4s-arch/

[9] 	Explicit Congestion Notification (ECN) Protocol for Ultra-Low Queuing Delay (L4S) , Briscoe et. al, Internet draft, IETF, Mar 2021, https://tools.ietf.org/wg/tsvwg/draft-ietf-tsvwg-ecn-l4s-id/

* Next change * (all new text)

## 6.X	Solution #X: Use of ECN for scalable congestion control and meet requirements for services

### 6.X.1	Key Issue mapping

Editor's Note:	This clause lists the key issue(s) addressed by this solution.

### 6.X.2	Description

Editor's Note: This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Sub-clause(s) may be added to capture details.

This solution proposes that the 5G System uses ECN marking for the purpose of “Low Latency, Low Loss and Scalable Throughput” (L4S [9]) when congestion is detected for uplink or downlink so that the application layer can trigger real-time and gradual rate adaptation of the real-time video encoder based on L4S feedback.

NOTE: This solution is applicable only in case UE and Application Server support ECN mechanism defined in RFC 8311[7] and use protocols that support relaxed ECN feedback between UE and application server.

### 6.X.2.1 Use of ECN for Uplink

When the 5G System detects congestion in the uplink, it inserts ECN marking in the IP header that facilitates the application layer to provide feedback and do rate adaptation in the uplink (illustrated in figure 6.X.2.1-1). It is preferrable to perform the marking in the entity where congestion is detected.

UE

I

P

f

l

o

w

s

QoS Flow

IP flow

Inserts L4S marking

Inserts L4S marking

Figure 6.X.2.1-1 ECN marking for Uplink – either in the UE or gNB

The congestion in the wireless uplink between UE and gNB can be detected and remedied via ECN marking by either the UE or the RAN. Either the gNB or the UE should be responsible for congestion detection. Otherwise, they could conflict. After this initial choice, several options are possible to bring the congestion information into the PDU IP header where it is further handled by the end-host to control the application’s sending rate.

L1

L2

L1

L2

Application

L1

L2

IP

UDP

GTP-U

IP/…

UPF

gNB

UE

PHY

MAC

RLC

PDCP

SDAP

IP/…

Application

L1

L2

IP

UDP

GTP-U

PHY

MAC

RLC

PDCP

SDAP

IP/…

ECN

ECN

ECN

cong info

Uplink
congestion
detected

N3

N6

NR-Uu

DN

IP/…

3

2

1

2

3

Figure 6.X.2.1-2 Congestion for uplink detected in the gNB

gNB is well positioned to have the overall view of congestion in the cell and overall queues that are building up in the UEs are visible in the gNB via buffer status reports. Following options are possible when the gNB handles congestion in the uplink (illustrated in figure 6.X.2.1-2):

- gNB translates congestion into ECN marks, sets immediately in the PDU IP header directly. This option does not impact the UE and the UPF.

NOTE: The criteria that RAN uses to perform the marking is up to RAN implementation.

Editor’s Note: Whether RAN can support such a marking need to be coordinated with RAN WG.

- gNB transmits congestion information in the GTP-U header, which is transmitted over N3 to UPF, that translates congestion into ECN marks, which is finally set in the IP PDU header. This option will impact the UPF in addition.

- gNB translates congestion into ECN marks, sets in the outer IP header of the GTP-U tunnel, which is transmitted over N3 to UPF, which is finally transferring the ECN marks over to the IP PDU header towards N6. This option also impacts the UPF in addition.

L1

L2

L1

L2

Application

L1

L2

IP

UDP

GTP-U

IP/…

UPF

gNB

UE

PHY

MAC

RLC

PDCP

SDAP

IP/…

Application

L1

L2

IP

UDP

GTP-U

PHY

MAC

RLC

PDCP

SDAP

IP/…

ECN

ECN

ECN

Cong info

Uplink
congestion
detected

N3

N6

NR-Uu

DN

IP/…

1

6

5

ECN

ECN

ECN

3

2

4

5

6

Figure 6.X.2.1-3 Congestion for uplink detected in the UE

UE can also monitor congestion in the uplink radio bearers. This option can cause interactions between scheduler policies that use the buffer status reports which will be influenced (especially if kept “too” low) by the congestion control in the UE. This option impacts additionally the UE, and detection of uplink congestion is limited to the scope of the UE. Three options are possible for inserting ECN marking when the UE handles congestion in the uplink while two of them require gNB assistance (and potentially other nodes), which has further 3 options to carry the ECN marks into the final IP PDU header. (illustrated in figure 6.X.2.1-3):

- UE translates congestion into ECN marks, sets immediately in the PDU IP header.

- UE translates congestion into ECN marks, sets in the PDCP header, which is transmitted over NR-Uu to the gNB, and carried further over one of the three options 4, 5 or 6 for the gNB to forward the ECN marks over to the IP PDU header.

- UE translates congestion into ECN marks, sets in the RLC header, which is transmitted over NR-Uu to the gNB, and carried further over one of the three options 4, 5 or 6 for the gNB to forward the ECN marks over to the IP PDU header.

If UE is transmitting the ECN marks in one of the lower layer headers (like options 2 and 3 above) additional network support is needed as in one of the following 3 options:

- gNB transfers the ECN marks, sets immediately in the PDU IP header.

- gNB transmits the ECN marks as congestion information in the GTP-U header, which is transmitted over N3 to UPF, which translates congestion into ECN marks, set finally in the IP PDU header.

- gNB transfers the ECN marks, set in the outer IP header of the GTP-U tunnel, which is transmitted over N3 to UPF, which finally transfers the ECN marks over to the IP PDU header towards N6.

In general, if functions are not marking packets directly in the PDU IP header, guarantees are needed that another function will do so before the intermediate header information is discarded and before the PDU IP packet is processed by the end system.

### 6.X.2.2 Use of ECN for Downlink

When the 5G System detects congestion in the downlink, it inserts ECN marking in the IP header that facilitates the application layer to provide feedback and do rate adaptation in the downlink (illustrated in figure 6.X.2.2-1). It is preferrable to perform the marking in the entity where congestion is detected.

IP flows

UE

IP flow

QoS flow

Inserts L4S marking

L4S feedback

Figure 6.X.2.2-1 ECN marking for Downlink

There are multiple options to do this (as illustrated in figure 6.X.2.2-2 below):

- gNB translates congestion into ECN marks, sets immediately in the PDU IP header. It should be noted that the gNB already has visibility to the PDU IP header for ROHC compression. This option does not impact the UE at all.

Editor’s Note: Whether RAN can support such a marking need to be coordinated with RAN WG.

- gNB translates congestion into ECN marks, sets in the PDCP header, which is transmitted over NR-Uu to the UE, which finally copies the ECN marks over to the IP PDU header. This option supports a direct correlation between PDCP and IP packets.

- gNB translates congestion into ECN marks, sets in the RLC header, which is transmitted over NR-Uu to the UE, which finally copies the ECN marks over to the IP PDU header. With this option, congestion can be reported immediately where it occurs.

- gNB transmits congestion information as part of the uplink traffic in the GTP-U header, which is transmitted over N3 to UPF. The UPF translates congestion into ECN marks, set finally in the IP PDU header of subsequent downlink packets. This option impacts also UPF and will cause extra latency going backwards and forward again. This can cause an extra delay in congestion response of twice the link latency between gNB and UPF.

NOTE: The criteria that RAN uses to perform the marking is up to RAN implementation.

In general, if functions are not marking packets directly in the PDU IP header, guarantees are needed that another function will do so before the intermediate header information is discarded and before the PDU IP packet is processed by the end system.

L1

L2

L1

L2

Application

L1

L2

IP

UDP

GTP-U

IP/…

UPF

gNB

UE

PHY

MAC

RLC

PDCP

SDAP

IP/…

Application

L1

L2

IP

UDP

GTP-U

PHY

MAC

RLC

PDCP

SDAP

IP/…

ECN

ECN

ECN

cong info

Downlink
congestion
detected

N3

N6

NR-Uu

DN

IP/…

1

3

4

ECN

ECN

2

3

2

4

Figure 6.X.2.2-2 Solution options for ECN marking over Downlink

### 6.X.2.3 Enabling use of ECN

To enable ECN, an existing or a separate QoS flow can be used. At the time of QoS Flow establishment, it is determined whether use of ECN is enabled or not.

Editor’s note: Whether use of ECN is enabled based on use of specific 5QI or other means is FFS.

### 6.X.3	Procedures

Editor's Note: This clause describes high-level procedures and information flows for the solution.

### 6.X.4	Impacts on services, entities and interfaces

Editor's Note: This clause captures impacts on existing 3GPP nodes and functional elements.

Depending on the solution option chosen for uplink and downlink, support for ECN marking (for the purpose of L4S) of payload packets as specified in RFC 8311[7] needs to be supported either in the UE, NG-RAN. In some cases, additional congestion notification is supported in the NG-RAN along with support of ECN (for the purpose of L4S) marking in the UPF for payload packets as specified in RFC 8311[7].

* End of changes *