3GPP TSG-WG SA2 Meeting #151 e-meeting 	S2-2204620r01

May 16th – 20th, 2022; Elbonia

Source:	China Telecom

Title:	KI #5, New Sol: Improve communiction reliability with two-path transmission

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract: It is a revision of postponed S2-2202703r02, to clarify the issues raised in last meeting.

# 1. Introduction/Discussion

It is valuable to offer end to end communication reliability including both Core network side and RAN side. In this solution the Core network can provide NG-RAN with information to assist the NG-RAN to identify the two PDU session from the remote UE and L-3 relay UE. Then the NG-RAN can realize communication reliability for these two PDU session by using exixting redundancy handing mechanism in URLLC, e.g. allocating different, redundant user plane resources in NG-RAN for each PDU session.

Since it has RAN dependancy, confirmation of feasibility on RAN impacts by RAN WGs is required before concluding this solution.

# 2. Text Proposal

It is proposed to add the following changes in 3GPP TR 23.700-33.

# * * * * First change * * * *

## 6.0 Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | X |  |  |  |  |  |  |
| 2 | X |  |  |  |  |  |  |
| 3 | X |  |  |  |  |  |  |
| 4 | X |  |  |  |  |  |  |
| 5 | X |  |  |  |  |  |  |
| 6 | X |  |  |  |  |  |  |
| 7 | X |  |  |  |  |  |  |
| 8 | X |  |  |  |  | X |  |
| 9 | X |  |  |  |  |  |  |
| 10 | X |  |  |  |  |  |  |
| 11 | X |  |  |  |  |  |  |
| 12 | X |  |  |  |  |  |  |
| 13 | X |  |  |  |  |  |  |
| 14 |  | X |  |  |  |  |  |
| 15 |  | X |  |  |  |  |  |
| 16 |  |  | X |  |  | X |  |
| 17 |  |  | X |  |  |  |  |
| 18 |  |  | X |  |  |  |  |
| 19 |  |  | X |  |  |  |  |
| 20 |  |  | X |  |  |  |  |
| 21 |  |  | X |  |  |  |  |
| 22 |  |  | X |  |  |  |  |
| 23 |  |  |  | X |  |  |  |
| 24 |  |  |  | X |  |  |  |
| 25 |  |  |  |  | X | X |  |
| 26 |  |  |  |  | X |  |  |
| 27 |  |  |  |  | X |  |  |
| 28 |  |  |  |  | X |  |  |
| 29 |  |  |  |  | X |  |  |
| X |  |  |  |  | X |  |  |

# * * * * Second change  All text is new * * * *

## 6.X	Solution #X: Improve communiction reliability with two-path transmission

### 6.X.1	Description

This solution addresses the Key Issue 5 on Multi-path transmission using U2N Relay to improve the comunication reliability. It applies for scenario when the indirect network communication path is through L-3 U2N relay.

This solution assumes the following mechanism to support comunication reliability:

-	In case of L-3 U2N relay without N3IWF, it is assumed that the Remote UE uses its own PDU session as the direct comunication path and the PDU session of the relay UE as the indirect network communication path to realize comunication reliability. These two redundant PDU sessions use two different UPFs.

-  In case of L-3 U2N relay with N3IWF, it is assumed that the Remote UE uses its own PDU session as the direct comunication path and the Non-3GPP PDU session established through N3IWF as the indirect communication path to realize comunication reliability, The Non-3GPP PDU session established through N3IWF is over the PDU session of L-3 U2N relay. These two redundant PDU sessions use two different UPFs.

-  In case of L-3 U2N relay with N3IWF, it is assumed that the Remote UE can use ATSSS mechanism to setup two redundant User Plane paths in a MA PDU session to realize comunication reliability. One path is over 3GPP access as the direct comunication path, and the other one is over Non-3GPP access as the indirect communication path which is through N3IWF and the path of PDU session of the L-3 U2N relay. The traffic distributed across the two redundant User Plane paths may apply cooresponding Streering Mode, e.g. Active-Standby or Duplication.

-  In the above 3 cases, the two redundant User Plane paths may go through two different NG-RANs or same NG-RAN, depending on the scenarios. In case of two different NG-RANs, communication reliability at RAN side is natrually supported. In case of a same NG-RAN, the NG-RAN needs to internally realize the communication reliability, e.g. by allocate different, redundant user plane resources for the each User Plane path.

Editor's note: How the NG-RAN knows that two PDU Sessions are redundant for L3 relaying without N3IWF, and L3 relaying with N3IWF is FFS.

-  It is assmumed NG-RAN is provided with the ID of the communication path in the PDU session of L-3 U2N relay. This ID is generated by L-3 U2N relay. The NG-RAN is also provided with ID of redundant communication path in the PDU session established by Remote UE over 3GPP acces. This ID is determined by Remote UE, and the Remote UE get this ID information from L-3 U2N relay. In case within a NG-RAN there are two PDU session with the same ID for the communication path and redundant communication path, this NG-RAN should realize communication reliability for these two PDU session.

-  The Remote UE and L-3 U2N relay should support the above redundancy mechanism. The Remote UE and L-3 U2N relay need to be authorized for the additional service of redundancy communication when performing authorization and provision for L-3 U2N Relay. In the discovery procedure, the Remote UE selects a L-3 U2N relay which supports redundancy communication.

### 6.X.2	Procedures

Editor's note:	This clause describes services and related procedures for the solution.

#### 6.X.2.1 Procedures for comunication reliability

Editor's note: What steps apply to which solution (e.g. L3 relaying with or without N3IWF, one PDU Session or two PDU Sessions) are to be clarified.

Editor's note: How the ID of redundant communication path is handled in different NFs within 5GC is FFS.

Figure 6.X.3-1 describes the general procedures of the solution

Figure 6.X.2.1-1: General procedures for the solution

- Redundancy service Authorization and Provisioning procedures are performed for L-3 U2N Relay and Remote UE.

Editor's note: It is assumed to follow the procedures in R17 with the enhancement for redundancy service on relay.

- The 5G ProSe L-3 U2N Relay may establish a PDU Session for relay that supports redundancy. The L-3 U2N Relay may generate the ID of the communication path and sent to its SMF. The SMF sends the ID of the communication path to NG-RAN in the PDU session establish procedures, e.g. in N2 SM information.

NOTE 1: The The L-3U2N Relay UE can randomly generate the ID of the communication path to ensure it is unique across all Relay UEs in a RAN node.

- The Remote UE performs discovery of a ProSe L-3 U2N Relay with redundancy capability and service. This may be triggered by UE implementation, when some application needs reliable communication.

- The Remote UE establishes a connection to L-3 U2N Relay. If there is no PDU Session associated with the Relay Service Code or a new PDU Session for relaying is needed, the 5G ProSe L-3 U2N Relay initiates a new PDU Session establishment procedure for relaying as described in step 2 before completing the PC5 connection establishment. The L-3 U2N Relay send the ID of the communication path to Remote UE, then the Remote UE treats it as ID of the redundancy communication path.

Editor's note: It is FFS on how the L-3 U2N Relay send ID of the communication path to Remote UE

- Subsequent steps for different L-3 U2N communication mode, after Remote UE connected to L-3 U2N Relay

5a. In case of L-3 U2N relay without N3IWF, for IP PDU Session Type and IP traffic over PC5 reference point, IPv6 prefix or IPv4 address (including NAT case) is allocated for the Remote UE. The UE use the IP address for communication.

5b. In case of L-3 U2N relay with N3IWF, the Remote UE registers in 5GC through N3IWF via the communication path of L-3 U2N Relay. The Remote UE establishs PDU session or MA PDU session at Non-3GPP side.

Editor's note: It is FFS how to select between two PDU sessions or MA PDU session for Remote UE.

- If it is following step 5a in which the Remote UE uses the PDU session of L-3 U2N relay for communication, or in step 5b the Remote UE establishes PDU session at at Non-3GPP side, the Remote UE establishs a new PDU session at 3GPP access side. These two redundant PDU sessions use two different UPFs which could be achieved by operator configuration.

If in step 5b the Remote UE established MA PDU session at Non-3GPP side, the Remote UE establishs another access leg of the MA PDUsession at 3GPP access side.

In the above PDU session establish procedures, the Remote UE may send the ID of redundant communication path to its SMF. The SMF sends this ID to NG-RAN in the PDU session establish procedures, e.g. in N2 SM information.

Editor's note: It is FFS whether the NG-RAN needs to know the UE that the ID for PDU Session is provided is L3 Remote UE. If so, how?

After step 6, two redundant communication paths are established, then they can be used for redundant traffic delivery end-to-end following the methods defined in URLLC. If it is a MA PDU session, then the traffic distributed across the two redundant paths may apply cooresponding Streering Mode for ATSSS, e.g. Active-Standby or Duplication.

Editor's note: How to support redundant handling for ATSSS will be studied in R18 ATSSS_Ph3.

- If the PDU session of L-3 U2N Relay and PDU session of Remote UE at 3GPP access are in a same NG-RAN node, this NG-RAN should realize communication reliability for these two PDU session, e.g. allocating different, redundant user plane resources for each PDU session. The NG-RAN can identify the two PDU sessions by comparing the ID of the communication path in a PDU session of Relay with the ID of the redundant communication path in a PDU session of Remote UE. If the two PDU sessions are separately at different NG-RAN nodes, then each of the NG-RAN can not and dose not need to identify the two sessions since they are naturaly redundant at RAN side. When multiple Remote UEs share a same Relay in a NG-RAN, the NG-RAN should realize communication reliability for PDU session of Relay and PDU sessions of multiple Remote UEs, e.g. the allocated user plane resources for PDU session of Relay is different from the allocated user plane resources for PDU sessions of multiple Remote UEs. It is assumed that the NG-RAN could identify the two PDU sessions by finding out a PDU session of Remote UE whose ID of redundant communication path has the same value with the ID of communication path of the PDU sessions of Relay, when there is new PDU session of Relay in the NG-RAN, e.g. due to Relay mobility. The NG-RAN could identify the two PDU sessions by finding out a PDU session of Relay whose ID of communication path has the same value with the ID of redundant communication path of the PDU sessions of Remote UE when there is new PDU session of Remote UE in the NG-RAN.

Editor's note: It is FFS whether the NG-RAN needs to act as Master Node and Secondary Node, or a single Node, for same NG-RAN case.

Editor's note: Confirmation of feasibility on RAN impacts by RAN WGs is required before concluding this solution.

Editor's note: It is up to RAN decision on how to identify the paired PDU session for redundancy in a NG-RAN.

### 6.X.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

The following impacts are foreseen by this solution:

Remote UE:

-	Authorization of ProSe service of L3 Relay supporting communication redundancy

-  Selection of a L-3 U2N relay support communication redundancy mechanism in ProSe discovery procedure

-  Provision of ID of redundant communication path

L-3 U2N relay:

-	Authorization of ProSe service of L3 Relay supporting communication redundancy

-  Provision of ID of communication path

5G DDNMF:

- Authorization and discovery for ProSe service of L3 Relay supporting communication redundancy

SMF:

- Provision to NG-RAN with ID of communication path, ID of redundant communication path

NG-RAN

- Redundancy handing between the two PDU sessions identified by same ID of communication path

# * * * * End of changes * * * *