SA WG2 Meeting #S2-151E	S2-2204628r16

16 - 20 May, 2022, Electronic meeting

Title:	[DRAFT] Follow-up LS on QoS support with Media Unit granularity

Response to:	S2-2203658 / S4-220505

Release:	Release 18

Work Item:	FS_XRM

Source:	SA2

To:	SA4

Cc:	-

Contact Person:

#### Name:	Sašo Stojanovski

E-mail Address:	saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 thanks SA4 on and the detailed information provided in their reply LS (S2-2203658 / S4-220505).

Based on the following answer from SA4:

3. Clarify what, if any, dependency there is between the IP packets that make up PDU Sets, e.g. a frame/”slice”.

Again, based on the response to question 1, no single comprehensive answer can be provided. In some cases there are no dependencies, in other cases dependencies exist. The examples above provide some insight into possible dependencies, from none to a dependency that the information unit is prefix-dependent to the case that if any piece is lost of an information unit, the entire unit is useless. We believe that different application media layer mappings and receiver implementations can be addressed by the PDU Set concept and the media/application layer should be able to configure the appropriate handling. Further coordination between SA2 and SA4 is encouraged.

SA2 understands that transmitting the remaining PDUs of a PDU Set following the loss of a PDU from that same PDU Set may still be useful in some cases, while being useless in other cases (yellow text).

The green text indicates that the media/application layer should be able to provide information for appropriate handling i.e. “should deliver remaining PDUs” vs “can drop remaining PDUs”.

Based on TR 23.700-60 SA2 would like to ask SA4 the following:

Q1: For the green text above about appropriate handling i.e. “should deliver remaining PDUs” vs “can drop remaining PDUs”, is there a suitable case whereby different PDU Sets in the same service data flow could have different handling requirements, i.e. for a specific service data flow, some PDU Sets have the handling of “should deliver remaining PDUs” while the other PDU Sets of the same service data flow have the handling of “can drop remaining PDUs”?

Q2: Can HTTPS also be used as transport protocol for low latency XR? How does the concept of PDU Sets apply for it and is there a difference in that sense compared to RTP or SRTP used as transport protocols?

2. Actions:

To SA4 group.

ACTION: 	SA2 respectfully asks SA4 to provide feedback on the questions above.

3. Date of Next TSG-SA WG2 Meetings:

3GPP SA2#152E			17 - 26 August 2022			e-meeting