SA WG2 Meeting #151e	S2-2204658r02

# May 16th – 20th, 2022; Elbonia                  	       (revision of S2-22xxxxx)

Source: 	Philips International B.V.

Title: 	KI#2 and KI#4 clarifications on multiple UEs

Document for: 	Approval

Agenda Item: 	9.20

Work Item / Release:	FS_Ranging / Rel-18

Abstract of the contribution: This contribution proposes to add some clarifications on multiple UEs being involved in ranging..

# 1	Discussion

Currently KI#2 mentions the use of an Assistant UE to assist in ranging between two devices. However, there are also other cases to be considered when making use of multiple devices for ranging, such using multiple anchor UEs to improve accuracy of position estimation of the target UE or the Assistant UE.

This pCR proposes to update KI#2 and KI#4 to address some of the other cases involving multiple UEs as well.

# 2 Proposal

It is proposed to update TR 23.700-86 by updating Key Issue #2 and Key Issue #4 as follows.

************************** START OF CHANGE ***********************

## 5.2	Key Issue #2: Ranging service operation procedure with the assistance of another UE

### 5.2.1	General description

The ranging service operation between two UEs may need the assistance of an one or more other UEs (e.g., assistant UE) if the direct ranging/sidelink positioning between a reference UE and a target UE cannot be supported, or if the ranging/sidelink positioning is not sufficiently accurate. The direct communication between a reference UE and one or more assistant UEs and/or one or more other reference UEs, and between a target UE and one or more assistant UEs and/or one ore more other reference UE are respectively supported, so that the assistant UEs or other reference UEs can help on measurement and data transmission.

This key issue addresses how to enhance the 5GS to support ranging service operation procedures with the assistance of other UEs (e.g., assistant UEs, additional reference UEs).

At least the following aspects need to be considered in potential solutions:

-	How to discover and determine an assistant UE or additional reference UE for ranging service?

-	What parameters need to be exchanged between different UEs (reference UE, assistant UE, target UE) during ranging service operation procedures?

-	How the reference UE or target UE gets the ranging result with the assistance of one or more assistant UEs or other reference UEs during the ranging service operation procedures?

NOTE:	The direct discovery procedure between reference UEs and assistant UEs, between reference UEs, and between target UE and assistant UEs will be developed under the key issue related to ranging direct discovery.

## 5.4	Key issue #4: Control of Operations for Ranging/Sidelink positioning

### 5.4.1	General description

Ranging and sidelink positioning supports a large variety of use cases, including automotive, public safety, and commercial use. These different uses cases may utilize different access technologies for the communication between the UEs, e.g. LTE based V2X, NR based sidelinks and, in later releases, other RAT technologies may be supported over PC5, e.g. newer version of sidelink and non-3GPP RATs. Therefore, it is beneficial to design a common framework and system to support all these types of operations.

The control signalling part of the Ranging and Sidelink Positioning operation may occur between the UEs before and after the transmission of the Ranging/Sidelink positioning signals. The control signalling includes (but is not limited to):

-	coordination and configuration of the UEs to participate in the ranging/sidelink positioning, e.g. including discovery of the devices, signalling control and configuration information between devices;

-	exchange of measurement results of Ranging/Sidelink positioning signals  between the UEs for ranging/Sidelink positioning calculation.

This key issue is focusing on developing the control signalling for Ranging and Sidelink Positioning operations. Specifically, the key issue addresses:

- What coordination and configuration information are exchanged between the UEs?

- What are the control signalling procedure to exchange the coordination & configuration information and the Ranging/Sidelink positioning signals measurement results between the UEs?

NOTE 1	Common solution is preferred to be developed if the same control signalling procedure can be supported when the signalling is carried over different RATs in different use cases.

NOTE 2:	Common control signalling protocol is preferred to support the different Ranging/Sidelink positioning signals operation in different spectrums.

NOTE 3:	The solution development may also consider signalling efficiency, overhead, energy efficiency and the robustness.

NOTE 4:	Coordination with RAN WGs is needed to develop an aligned solution.

NOTE 5:	It is desirable to avoid any AS layer impacts to support the control signalling.

-	What are the functionality of upper layers, and what parameters need to be exchanged between the upper layers and AS layer for Ranging/ Sidelink positioning?

NOTE 6: Multiple reference UEs may be involved in ranging/sidelink positioning for a more accurate estimation.

************************** END OF CHANGE ***********************