SA WG2 Meeting #151e	S2-2204707r02

# May 16 – 20, 2022; Elbonia                		       (revision of S2-220XXX)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	 KI#4: Updates to Solution 19

Document for: 	Approval

Agenda Item:         9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract of the contribution: Update to Solution #19 on Influencing UPF and EAS (re)location for collections of UEs

1	Discussion

This pCR provides updates to solutions#19 in the TR 23.700-48, and removes existing FFs.

2 Proposal

It is proposed to add below text to EC TR 23.700-48

# * * * * Start of changes * * * *

6.19	Solution 19 ( KI#4 ): Influencing UPF and EAS (re)location for collections of UEs

6.19.1	Introduction

This solution aims to address the technical requirements related to key issues #4. In particular it proposes for certain Edge Application use case e.g. all users playing a certain online game and registered to a particular EAS, or all UEs in a platoon, how these UEs collectively form an ad hoc and dynamic group, how to identify such group, how to handle coordination of the UPF(s) and EAS (re)location for the UEs belonging to the same group.

6.19.2	Functional Description

The following are the main principles of the solution:

- Application Function creates an ad hoc group of collection of UEs specific to specific use cases and based on its respective criteria, for example: all Users served by the same EAS that hosts an online gaming application, and ad hoc group of UE of a Platoon.

- Application Function creates an ad hoc group of collection of UEs, and provides to 5GC, either directly or via NEF, the necessary group specific information, together with (external) group ID.

- NEF performs AF request authorization and provides necessary mappings, and configures UDM with the ad hoc group information.

6.19.3	Procedure

The below figure 6.19.3-1 provides a detail call flow on ad hoc group and group specific attributes provisioning by AF.

Figure 6.19.3-1: AF Provisioning Ad hoc Group Information

Step 1: Application Function configures an ad hoc dynamic group of a collection of UEs. These collection of UEs are based on common use case or application e.g. all users registered on a particular gaming server (EAS) or all UEs in a platoon. AF assigns an (external) Group ID to this collection of UEs. AF provisions this group information to 5GC either directly or via NEF and provides required details including: (external) group ID, Group Type (this is to differentiate with static groups based on user subscriptions), collection of UE IDs forming the group i.e. list of group members,  and group property or attributes such as all users to be served by the same EAS.

Alternatively, AF may use new API e.g. Nnef_GroupParameterProvision_Create/update/delete to provision these information.

Group Type indicates specific ad hoc group e.g. an ad hoc group specific to platooning or an ad hoc group of UEs playing online game and registered to a particular gaming server/EAS.

Editor’s Note: Use of group type is FFS

Step 2: NEF after authorizing the AF request, creates or updates these information in UDM. It translates AF provided information into corresponding 5GC internal information such as: external Group ID into an internal group ID. The Group Attributes are group specific information such as indication to use same EAS for all group members, or same PSA UPF.

NEF stores (or updates) these in UDM.

As specified in 4.15.6.2 in TS 23.502, and 6.5.6.2.6 in TS 29.503, UDM/UDR assigns/allocate an unique Internal Group ID.

Step 3-4: UDM uses Nudr_DM_Query/Update to UDR. UDR stores the data as part of the Group subscription data and responds with Nudr_DM_Create/Update/Delete Response message.

Table 6.19.3-1: Ad hoc group data

| Parameters | Description |
| --- | --- |
| S-NSSAI | (Optional) S-NSSAI for the ad hoc group |
| DNN | (Optional) DNN for the ad hoc group |
| Group Type | indicates specific ad hoc group e.g. platooning, online gaming, etc. |
| EAS ID | (optional) Edge Application Server ID e.g. application server instance where users of the online gaming application are registered |
| Group Attributes | Group type specific properties |

Table 6.19.3-2: Ad hoc group membership management parameters

| Parameters | Description |
| --- | --- |
| External Group ID | An identifier of the Ad Hoc group |
| List of GPSI | List of Ad Hoc group members, each member is identified by GPSI |

Step 5: UDM responds with Nudm_ParameterProvision_Create/Update/Delete Response.

Step6:  NEF responds to AF with Nnef_ParameterProvision_Create/Update/Delete Response

Step 7: UDM notifies to the subscribed Network Function e.g. AMF and SMF, of the Group subscription data via Nudm_SDM_Notification Notify message The SMF stores the received parameters and associates them with a PDU Session based on the DNN and S-NSSAI included in the message from UDM.

The group information is used e.g. by SMF to ensure collective and common behaviour and treatment to the member UEs of the ad hoc group. For example, all the UEs of a particular ad hoc group are (re-)located same EAS instance as in solution 6.X in S2-2204706.

6.19.3	Impacts on services, entities and interfaces

Editor's note:	This clause lists impacts to services and interfaces.

AF:  Provisions ad hoc group information to the 5GC

NEF: Authorizes, translates and stores ad hoc group information to UDM/UDR

SMF: Subscribes to the ad hoc group subscription data

# * * * * End of changes * * * *