SA WG2 Meeting #151e	S2-2204711r03

16 – 20 May 2022, e-Meeting	(revision of S2-220xxxx)

Source:	Qualcomm Incorporated

Title:	New Solution for KI#1, KI#2: Provision of Coverage Data to a UE

Document for:	Approval

Agenda Item:	9.24

Work Item / Release:	FS_5GSAT_Ph2 / Rel-18

Abstract of the contribution: Describes a new solution for providing coverage data (e.g. for satellites) to a UE.

# 1. Introduction

The SID agreed for Study on satellite access Phase 2 in SP-211651 includes the following objective:

The study item aims at investigating on further 5GC/EPC enhancements to support satellite access based on the R17 5GSAT_ARCH achievements with the following 5GC/EPC areas for study: - WT#1: Discontinuous coverage WT#1.1: Architectural enhancements to support discontinuous coverage for mobility enhancement (e.g. paging enhancement) WT#1.2: Architectural enhancements considering prediction, awareness & notification of UE wake-up time, power saving optimizations.

TR 23.700-28 contains the following objectives for KI#1:

- Identify gaps in rel.17 solution designed in EPS (e.g. concerning minimizing a period of no coverage and/or minimizing power consumption), considering at least below aspects: a) Study how UE determines that it has to remain with no service or it has to attempt to register on available different RAT's/ PLMNs to receive the normal service during discontinuous coverage in current NTN RAT. NOTE: Consider dependencies with RAN2 and CT1 since inter-RAT selection and PLMN selection related specifications are in control of RAN2 and CT1. b) Study how to reduce the impact to target RAT or system due to large number of UEs triggering signalling load on the target RAT or system to receive normal service. - Propose solution to resolve these gaps.

TR 23.700-28 contains the following objectives for KI#2:

At least the following aspects need to be investigated: - Based on the coverage information of the UE: - whether and how to enhance the power saving mechanisms, e.g. PSM, MICO mode and eDRX in CM-IDLE state, in order to make sure that the UE: - does not attempt PLMN access when there is no network coverage; and - when there is network coverage the UE attempts PLMN access as needed e.g. to transfer signalling, transfer data or receive paging, etc. NOTE: Network coverage can be provided by any RAT supported by the UE.

The solution described here addresses the objectives highlighted above and enables a UE to obtain data on satellite coverage (and possibly TN coverage) for a serving PLMN. The coverage data is provided by a server over user plane which allows greater flexibility in the quantity and type of the coverage data, greater security and the possibility of charging. For example, in Release 17, the SIB32 defined in TS 36.331 provides orbital data to a UE for up to 4 satellites. These would most likely support the same satellite RAT as the satellite broadcasting the SIB32 and might only provide coverage data for a few hours (given that each satellite could only be visible for around 5-15 minutes at any one location). This data is not secure (e.g. could be spoofed by a fake base station to mislead UEs to go into an inactive state) and cannot be billed. These limitations are all overcome with the proposed solution.

# 2. Text Proposal

The following text is proposed to be applied to TR 23.700-28.

*** First Change ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[7]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[8]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[aa]	3GPP TS 36.331: “Radio Resource Control (RRC); Protocol specification”.

[x]	3GPP TS 26.531: " Data Collection and Reporting; General Description and Architecture".

[y]	3GPP TS 23.288: " Architecture enhancements for 5G System (5GS) to support network data analytics services".

*** Next Change ***

## 6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 |  |  |
| X | x | x |  |  |

## *** Next Change (all new text) ***

## 6.X	Solution #X: Solution to support Provision of Coverage Data to a UE

### 6.X.1	Description

This solution supports portions of KI#1 and KI#2 and consists of procedures for providing this coverage data to a UE that supports IP PDU Sessions.

The content of coverage data can include a coverage map which shows the expected coverage by one or more satellite RATs one or more locations and for a particular time in the future. A set of coverage maps can then be provided for each of a sequence of times occurring at fixed periodic intervals, such as at intervals of one minute. The locations supported by a coverage map can correspond to grid points in a rectangular (or possibly hexagonal) array, as illustrated by the rectangular array shown in Figure 6.X.1-1.

Figure 6.X.1-1: Coverage Data Map for a set of Grid Point Locations

In Figure 6.X.1-1, each grid point (dot) represents one location. These locations (i.e. grid points) can be spaced apart by a fixed distance (e.g. 10 to 100 kms). The absolute location of each grid point can then be known by specifying an absolute (global) location for just one grid point in the array (e.g. a centre grid point or a grid point at one corner). The expected satellite coverage at the location of each grid point.

A coverage map (or set of maps) can be provided to a UE by a server via user plane. Two alternative solutions can be used for this part.

The first solution is based on an HTTPS query from a UE to a server. The UE also provides a temporary ID to the server that is pre-configured in the server and verifies permission for the UE (e.g. a subscription) to receive the coverage data.

With the second solution, the DCAF/AF functionality and architecture as defined in TS 26.531 [x] and TS 23.288 [y] could be extended to expose network data analytics to a UE.

Editor’s Note: Whether and how DCAF/AF functionality is FFS.

### 6.X.2	Procedures

#### 6.X.2.1	Obtaining Coverage Data using an HTTPS Query to a Server

The procedure is in two parts. First the UE obtains a URI for the server and a temporary ID that is already pre-configured in the server. Then the UE sends an HTTPS request to the server (identified by the URI) and includes the temporary ID.

### Editor’s note: Applicability of an HTTPS query to 3GPP specs and 3GPP WGs is FFS.

### Figure 6.X.2.1-1 shows the first procedure for 5GS. A procedure for EPS can be analogous using an Attach or TAU. The procedure is based on NAS but could use some other protocol layer instead – e.g. RRC.

Figure 6.X.2.1-1: Procedure to Request a URI and temporary ID

1.	The UE sends a Registration Request to the serving AMF (e.g. for an initial registration, periodic registration or registration update) and includes a request for a URI and Temporary ID to enable provision of coverage data. Some additional parameters might be included to indicate applicable location(s) in case different servers are used to provide coverage data for different locations (e.g. for different countries).

2.	The serving AMF may verify the UE subscription to receive coverage data.

Editor’s Note: It is FFS why the AMF needs to verify the location requested or whether the AMF only provides information for areas it serves.

3.	The AMF determines a server applicable to the  location(s), a URI for the server and a temporary ID for the UE.

Editor’s Note: The generation, sharing and configuration of the security information are FFS and requires coordination with SA3.

4.	The AMF returns the UE and temporary ID in the Registration Accept. The AMF may also indicate a limitation on usage of the URI and temporary ID - e.g. a maximum number of requests to the server or a maximum time duration of usage.

Editor’s Note: If is FFS whether and how the AMF determines the additional information in the response.

### Figure 6.X.2.1-2 shows the second procedure which can be the same for both 5GS and EPS.

Figure 6.X.2.1-2: Procedure to Request Coverage Data from a Server

- The UE sends an HTTPS Request (e.g. an HTTPS GET) to the server indicated by the URI obtained in the first procedure. The HTTPS Request includes the temporary URI (if applicable) and parameters indicating the location of coverage data, an area of coverage data (e.g. a size of a rectangular or circular area), a time period of coverage data, one or more satellite RATs, an applicable serving PLMN.

- The server verifies that the temporary ID is valid.

- The server returns the requested coverage data to the UE according to the request at step 1.

Editor’s Note: The generation, sharing and configuration of the security information are FFS and requires coordination with SA3.

#### 6.X.2.2	Obtaining Coverage Data using an DCAF and NWDAF

Editor’s Note: The details of these procedures are FFS.

This procedure is an alternative to the procedure in clause 6.X.2.1 and is more focused on PLMN provision of coverage data rather than provision by a third party like a satellite operator.

Since Rel-17, the DCAF functionality which is part of the AF was introduced to enable the NWDAF to perform data collection from an Application. Given the communication path between NWDAF and the DCAF has been introduced for data collection, it is extend the existing DCAF/AF functionality and architecture as defined in TS 26.531 [x] to expose network data analytics related to the UE to assist the UE’s determination for coverage information.

Editor's note:	The feasibility of this clause needs further discussion and to be checked with eNA_Ph3 and SA4.

The following assumptions are made:

-	The UE can Subscribe/Request to NWDAF via DCAF to request the information from 5GC (e.g. the Analytics result from the NWDAF).

-	The communication services and functional architecture between UE and DCAF is in the scope of SA4 and shall be compatible with the existing architecture defined in TS 26.531 [x].

A user plane data collection from UE application client is in clause 6.2.8 in TS 23.288 [y] in Rel-17. A UE Application is configured the address of DCAF by the Application server. The UE establishes a connection to the DCAF over user plane via a PDU session. The DCAF communicates with the UE and collects data from the UE Application to NWDAF.

Editor’s Note: it is FFS how the Application server determines the address.

The same procedure for user plane connection between the UE and DCAF (R2 interface in TS 26.531 [x]) is proposed to be used by the UE to request data exposure from NWDAF.

The following information can be exposed to the UE via the support of the DCAF based on network consent acquired by UE:

- Satellite ephemeris from any satellite operators the UE may be able to access

- Other RAT information the UE may be able to access

Editor’s note: How to extend the interaction between UE and DCAF as defined in TS 26.531 [7] to enable UE to subscribe/request information to/from 5GC NF (e.g., NWDAF) via DCAF is in SA4 scope.

The SLA between the operator and an External Provider of the related information to NWDAF further include the Supported Analytics ID list.

The Supported Analytics ID indicates the Analytics outputs that allowed by the operator to expose to the UE. The Supported Analytics ID list will be stored in DCAF.

The External Provider will provision the Supported Analytics IDs to UE with means out of scope of 3GPP.

The UE requested data exposure procedure is described in Figure 6.X.2.2-1.

Figure 6.X.2.2-1 UE requested data exposure procedure

- The UEs Direct Data Collection Client to send the Analytics Request to DCAF via signalling defined in TS 26.531 [7]. The DCAF address is provided to UE using a PCO when the UE establishes the PDU session to access DCAF. The requested Analytics ID(s) are included in the signalling to DCAF.

DCAF can retrieve the UE IP address from the source IP address of the received packet.

2.	UE ID retrieval procedure is described in clause 6.x.2.2.

3.	DCAF discovers the NWDAF that supports the Analytics ID received in step 1.

For DCAF in trusted domain, DCAF sends Nnrf_nwdafdiscovery _request to NRF to discovery the NWDAF. The request message shall include the Analytics ID and the S-NSSAI.

For DCAF in untrusted domain, DCAF sends Nnef_EventExposure_subscribe with AF specific Identifier to NEF. NEF determines the S-NSSAI for the AF specific and sends Nnrf_nwdafdiscovery _request to NRF to discovery the NWDAF. The request message shall include the Analytics ID and the S-NSSAI

4. NRF sends the Nnrf_nwdafdiscovery_response with the discovered NWDAF identity.

5. DCAF subscribes to NWDAF for the analytics request with UE Identity and Analytics ID(s). Step 5a is for DCAF in trusted domain and step 5b is for DCAF in untrusted domain.

Before step 5b, the AF asks the NEF for authorization information to check if the UE is allowed to obtain analytics ID from the network. The NEF determines the authorization information for the UE based on local policy and the UE subscription data from the UDM about whether the UE has subscribed to the service that obtaining some specific analytics ID from network. Then the NEF sends the authorization information to the AF.

6. If NWDAF does not have available analytics result for the requested analytics ID, the NWDAF will trigger the analytics procedures for the analytics ID(s). NWDAF performs user consent check to UDM to determine if the analytics procedure is allowed or not.

7. Analytics procedure is performed as described in TS 23.288 [y]. NWDAF may combine data it received from various sources e.g. OAM, external satellite providers etc in order to determine the analytics results.

8. NWDAF sends analytics result to DCAF. Step 8a is for DCAF in trusted domain and step 8b is for DCAF in untrusted domain.

NOTE:	The security aspects of exposure of network data must be evaluated by SA3.

9. DCAF sends the analytics result to UE application client via application layer signalling.

10-11. If a Subscribe/Notify service operation is invoked in step 5, the NWDAF may further notify the analytics to UE e.g. periodically). NWDAF notifies the analytics result to DCAF. Step 11a is for DCAF in trusted domain and step 11b is for DCAF in untrusted domain.

- DCAF exposes the requested analytics to UE application client.

For DCAF in trusted domain, Since the supported pairs of S-NSSAI+DNN for the PDU session established for UE to DCAF are configured in the DCAF, DCAF may also retrieve the S-NSSAI+DNN for the PDU sessions that used by the UE sent the Analytics request. DCAF provides the UE IP address and the S-NSSAI to request UE ID from BSF as described in Figure 6.X.2.2-2.

Figure 6.X.2.2-2	UE ID Retrieval by DCAF in trusted domain

1. DCAF sends the Nbsf_Management_Discovery request to BSF with UE IP address, DNN and S-NSSAI to retrieve the session binding information of the UE.

2. BSF provides SUPI to DCAF via Nbsf_Management_Discovery response message.

For DCAF in untrusted domain, the supported pairs of S-NSSAI+DNN are configured in NEF. DCAF provides UE IP address to NEF and NEF retrieves the UE ID from BSF by reusing the procedure as described in clause 4.15.10 in TS 23.502 [4]. Figure 6.X.2.2-3 is the UE ID Retrieval procedure by DCAF in untrusted domain.

Figure 6.X.2.2-3 UE ID Retrieval by DCAF in untrusted domain

1. DCAF requests to retrieve UE ID via the Nnef_UEID_Get service operation. The request message shall include UE address and AF Identifier.

NOTE:	The case where UE IP address provided by the AF to the NEF corresponds to an IP address that has been NATed (Network and Port Address Translation) is not supported in this release.

2. The NEF authorizes the DCAF request.

3-4. The NEF may use the Nbsf_Management_Discovery service operation with UE address to retrieve the session binding information of the UE. BSF provides the SUPI / GPSI, S-NSSAI and DNN to NEF.

NEF stores the SUPI / GPSI, S-NSSAI and DNN for the UE address.

5-6. NEF interacts with UDM to retrieve the AF specific Identifier from UDM and further responds to DCAF.

### 6.X.3	Impacts on existing nodes and functionalities

#### 6.X.3.1	Obtaining Coverage Data using an HTTPS Query to a Server

UE:

-	Request and obtain a URI and temporary ID from an AMF or MME using NAS

-	Request and obtain coverage data from a server using HTTPS

AMF/MME:

-	Verify a UE subscription to receive coverage data and provide a URI to a server and temporary ID to the UE using NAS

Server:

-	Receive a request for coverage data from a UE using HTTPS, verify the temporary UE ID and provide coverage data

#### 6.X.2.2	Obtaining Coverage Data using an DCAF and NWDAF

DCAF:

- receives analytics request from UE via HTTP signalling to be defined in TS 26.531 [x].

- discovers NWDAF for the received analytics request from UE.

- sends analytics request to NWDAF.

- receives analytics response from NWDAF.

- sends the received analytics result to UE with signalling to be defined in TS 26.531 [x].

- checks with NEF about authorization information for the UE if it is allowed to obtain analytics ID from the network

NEF:

- Checks about authorization information for the UE based on local policy, the UE subscription data about whether the UE has subscribed to the service that obtaining some specific analytics ID from network.

NWDAF:

- Support coverage data encoded as a binary coverage map

- Provide coverage data in response to a request from a DCAF

UE:

- Send an analytics request to a DCAF for coverage data and receive coverage data in a response

- Support coverage data encoded as a binary coverage map

### 6.X.4	Solution evaluation

Editor's note:	This clause captures how each solution solves KIs and what other properties it may have.

*** End of Changes ***