SA WG2 Meeting #S2-151E	S2-2204728

16 - 20 May, 2022, Electronic meeting

# Title:	Reply to LS handling of packet filters when the supported number is exceeded

# Response to:	(S2-2203627/C3-221503) LS on handling of packet filters when the supported number is exceeded

# Release:	Rel-17

# Work Item:	TEI17, 5GS_Ph1-CT

Source:	SA2

To:	CT3

Cc:	-

Contact Person:

Name:	Judy Gan Juying

Tel. Number:

E-mail Address:	Judy.Gan.Juying@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: 23.502 CR3430, 23.503 CR0712

1. Overall Description:

SA2 thanks CT3 for the LS on handling of packet filters when the supported number is exceeded. SA2 would like to clarify that the limit of 16 packet filters applies to per EPS bearer and the total number of supported packet filters can reach 240 if 15 EPS bearers are supported for a PDN connection.

SA2 discussed and agreed on the solution in the attached CRs, and would like to provide the following feedback:

CT3 Question 1: For a UE with an established PDU session in 5GS with dynamic PCC deployment, how does the SMF+PGW-C know what packet filters will be part of the TFT allocation based on the PCC rules obtained from the PCF if the number of packet filters for signalled QoS rules exceeds the number supported in EPS? Is there any precedence criteria for the SMF+PGW-C to consider the pre-defined PCC rules (known or unknown to the PCF) in the TFT allocation?

[SA2 answer] For dynamic PCC rule, PCF indicates the order of the PCC rule in TFT packet filter allocation. For pre-defined PCC rules (known or unknown to the PCF), if there is a need to allocate TFT filter according to the pre-defined PCCs, it has to be the SMF+PGW-C that makes decision (e.g., based on operator policy which is usually static or implementation).

CT3 Question 2: For a UE with an established PDU session in 5GS without dynamic PCC deployment, how does the SMF+PGW-C know what packet filters will be part of the allocated TFT based on the local policy if the number of packet filters for signalled QoS rules exceeds the number supported in EPS?

[SA2 answer] When dynamic PCC is not deployed, the situation that number of TFT filters reaching the limit is unlikely to happen. In case it happens, the SMF+PGW-C makes the decision (e.g., based on operator policy which is usually static or implementation).

2. Actions:

To CT3

ACTION: 	SA2 kindly asks CT3 to take the above into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #152E		August 17 - 26, 2022	Elbonia

TSG-SA2 Meeting #153E		October 10 - 14, 2022	Elbonia