3GPP TSG-WG SA2 Meeting #150E meeting	S2-2204796

Elbonia, May 16 – 20, 2022	(revision of S2-2204281r03)

Source:	Lenovo

Title:	KI #6, New Sol: UE provided reason for registration with S-NSSAI

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3/Rel-18

Abstract of the contribution: This solution proposes that the UE provides to the AMF the reason for registration with an S-NSSAI.

# 1. Discussion

The TR 23.700-41 state for the KI#6:

This Key Issue will study how to enable network-controlled behaviour and ensure the proper utilization of Slices in the system (e.g. what the network can request to the UE, and how, and what additional policies the network can provide to the UE) taking into account the above aspects (e.g. actual slice usage, UE activity, etc.)

The main issue is that the network has to assure proper utilization of the network slices in the 5GS. The network-controlled behaviour of the UE, i.e. providing policies to the UE, is aleady a solution to the issue. However, it is not clear why the policies need to be provided to the UE, when the nerwork can internally apply the policies, assuming that the UE provides assistance information to the Requested NSSAI, e.g. the assistance information being the reason for registration with the S-NSSAI.

This paper proposes that the UE is configured to send the reason for registration with an S-NSSAI. The reasons for registration with S-NSSAI can be for example: (a) for immediate use; (b) for default applications or (c) proactive registration.  Then the network (AMF or NSSF), based on the reason for registration with S-NSSAI determines which S-NSSAI of the Requested NSSAI are included in the Allowed NSSAI.

# 2. Proposal

It is proposed to add a new solution to 3GPP TR 23.700-41.

* * * First Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| Solution #1: Additional S-NSSAI associated with the PDU session | X |  |  |  |  |  |
| Solution #2: Slice Re-mapping Capabilities for Network Slice Service Continuity | X |  |  |  |  |  |
| Solution #3: Support of Network Slice Service continuity using SSC mode 3 | X |  |  |  |  |  |
| Solution #4: PDU Session on compatible network slice | X |  |  |  |  |  |
| Solution #5: PDU session handover to a target CN with an alternative S-NSSAI support | X |  |  |  |  |  |
| Solution #6: Extended SoR VPLMN Slice Information transfer to UEs |  | X |  |  |  |  |
| Solution #7: Enabling awareness of Network Slice availability in VPLMNs |  | X |  |  |  |  |
| Solution #8: Gracefully network slice termination |  |  | X |  |  |  |
| Solution #9: Support of a Network Slice with an AoS not matching existing TA boundaries |  |  | X |  |  |  |
| Solution #10: Associating a validity timer with a temporary slice |  |  | X |  |  |  |
| Solution #11: Enabling UEs to Request S-NSSAIs not uniformly available |  |  | X |  | X |  |
| Solution #12: Solution for Centralized Counting for Multiple Service Areas and 5GS-EPS Interworking |  |  |  | X |  |  |
| Solution #13: Hierarchical NSACF Architecture for Maximum UE/PDU Session number control |  |  |  | X |  |  |
| Solution #14: Maximum Number Distribution in multiple NSACFs |  |  |  | X |  |  |
| Solution #X: UE provided reason for registration to S-NSSAI |  |  |  |  |  | X |

* * * Second Change * * * *  ALL text is new

## 6.X	Solution #X: UE provided reason for registration to S-NSSAI

### 6.X.1	Introduction

This solution targets KI#6 on "Improved network control of the UE behaviour".  Particularly the issue is that the network has to assure proper utilization of the network slices in the 5GS.

It is assumed that the network is aware about the policies and conditions how a UE is allowed to register with the requested Network Slices.

### 6.X.2	Functional Description

It is proposed that during Registration procedure the UE provides assistance information associated with the S-NSSAIs of the Requested NSSAI. This solution can be used during the Registration procedure and work in addition to other solution which monitor the usage of a Network Slice after the Registration procedure. The assistance information is basically the reason for registration with the S-NSSAI. The reason for registration with the S-NSSAI can be one of the following:

a)	The S-NSSAI is requested due to need for 'immediate use': an application in the UE requests connectivity and a matching URSP rule or UE Local Policy determined to use an S-NSSAI (e.g. as part of the URSP rule's RSD). In this sense, the UE needs the slice registration for (immediate) usage. In another example, if there are already established PDU Sessions (with active or inactive UP resources) in the S-NSSAI, the UE can use this reason for registration.

b)	The S-NSSAI is requested due to a match to 'default applications': based on local configuration, the UE is aware about the default applications and can determine the corresponding S-NSSAI (which can be identified as “always on” S-NSSAIs). For example, a voice-centric UE knows that IMS voice application is needed at any time, i.e. the IMS voice application is a default application. The corresponding S-NSSAI falls into this category.

c)	The S-NSSAI is requested due to a 'proactive registration': based on UE implementation, the UE may want to register to an S-NSSAI in order to be able to immediately establish a PDU Session in this network slice when needed at some point of time.

The network (e.g. the AMF) may configure the UE whether to include the reason for registration with the S-NSSAI.

When the network (e.g. AMF or NSSF) receives a reason for registration with S-NSSAI, the network determines, based on internal policies and the received reason for registration with each S-NSSAI of the Requested NSSAIs, which S-NSSAI of the Requested NSSAI are included in the Allowed NSSAI.

The AMF can optionally include a requested S-NSSAI with reason for registration 'proactive registration' in the list of rejected S-NSSAIs with an appropriate reject cause (e.g. rejected due to 'proactive registration'). The UE may request the S-NSSAI at any time later if there is application which needs this S-NSSAI.

Regarding the UE behaviour, if an application request data connectivity and the matching URSP rule includes an S-NSSAI which is not in the Allowed NSSAI (but also not rejected due unavailability in the PLMN, RA, or NSAC), the UE can first sends a request to register with the S-NSSAI before processing further RSDs or before the UE concludes that the RSD is invalid.

### 6.X.3	Procedures

The Figure 6.X.3-1 shows the procedure how the UE provides the reason for registration with S-NSSAI and how the AMF (or NSSF) determines the Allowed NSSAI by considering the information received from the UE and UE Subscription Data from the UDM.

Figure 6.X.3-1:  Procedure for UE registration with S-NSSAI having different reasons

The detailed description of the steps is provided as follows:

0.	The network (e.g. the AMF) may configure the UE whether to indicate the reason for registration with S-NSSAI. This step is shown as optional because indicating the reason for registration with S-NSSAI may be a default/mandatory behaviour and the configuration may be not needed.

1.	The UE determines the reason for registration for each S-NSSAI which is to be included in the Requested NSSAI.

The details for the reason for registration for each S-NSSAI are described in clause 6.X.2.

2.	The UE sends NAS Registration Request message. The Requested NSSAI includes information about the reason for registration for each S-NSSAI.

3.	The network continues with the registration procedure, e.g. steps 2-19 from Figure 4.2.2.2.2-1 in TS 23.502 [5].

4a.	During the UE Subscription Data retrieval from the UDM, the AMF may receive a list of Subscribed S-NSSAIs indicating whether a UE is allowed to register with the S-NSSAI when the S-NSSAI is needed for use.

4b.	The AMF (or the NSSF) determines the Allowed NSSAI by considering the following information: (1) the reason for S-NSSAI registration from step 1; (2) the S-NSSAI type as per step 4a; and (3) current network conditions for each S-NSSAI from the Requested NSSAI (e.g. slice load, whether the S-NSSAI is subject to NSAC for maximum number of UEs, etc.).

For example, if due to configuration or load conditions the AMF determines to allow only S-NSSAI registrations when the S-NSSAI is to be used or already in use, the AMF includes in the Allowed NSSAI only S-NSSAIs having a reason for registration 'immediate use' or 'default applications'. When the AMF is not configured to limit the registrations for an S-NSSAI, the AMF can include the S-NSSAI in the Allowed NSSAI independent on the reason for registration sent from the UE.

The AMF can optionally include an S-NSSAI with reason for registration 'proactive registration' in the list of rejected S-NSSAIs in order to avoid the UE to continue requesting this S-NSSAI.

5.	The AMF sends Registration Accept message to the UE including the Allowed NSSAI as determined in step 4b.

6.	The UE processes the Registration Accept message as known (e.g. step 21 from Figure 4.2.2.2.2-1 in TS 23.502 [5]).

Additionally, if an application request data connectivity and the corresponding S-NSSAI is not in the Allowed NSSAI, the UE first sends a request to register with the S-NSSAI before processing further RSDs or before the UE concludes that the RSD is invalid.

### 6.X.4	Impacts on services, entities and interfaces

Impacts to the UE:

-	ability to determine the reason for registration with an S-NSSAI.

-	sending the reason for registration for each S-NSSAI part of the Requested NSSAI.

-	receiving an S-NSSAI reject cause (e.g. rejected due to 'proactive registration').

-	triggering a registration with an S-NSSAI which is outside the Allowed NSSAI (and also not in the rejected S-NSSAIs different from rejected due to 'proactive registration') based on the URSP rules evaluation.

Impacts to the AMF:

-	Configuring the UE whether to send the reason for registration with an S-NSSAI.

-	Determining the Allowed NSSAI based on internal policies and the received reason for registration with each S-NSSAI of the Requested NSSAIs.

-	Providing to the UE an S-NSSAI reject cause (e.g. rejected due to 'proactive registration').