SA WG2 Meeting #151E	S2-2204803

16 - 20 May, 2022, Elbonia

Source:	Nokia, Nokia Shanghai Bell

Title:	solution proposal for key issue 1 (inactive reception)

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_MBS2 / Rel-18

Abstract of the contribution: this paper proposes a solution for key issue 1 (inactive reception).

# 1		Discussion

Key issue 1 is defined as follows:

## 5.1	Key Issue #1: Multicast MBS data reception in RRC Inactive state

### 5.1.1	Description

In order to provide MBS service to more UEs in a cell, NG RAN could enable UEs within an MBS multicast session to receive MBS session data while in CM-CONNECTED with RRC Inactive state.

The following aspect will be studied for multicast:

-	Whether, how and what MBS assistance information to provide from 5GC to RAN for an MBS session allowing UEs in CM-CONNECTED with RRC Inactive state to receive MBS content, including the aspect which 5GC NF(s) determine the MBS assistance information and how they do so;

-	Whether and how to enhance the current procedures (including mobility related procedures) for MBS session with member UEs in RRC Inactive state.

NOTE 1:	During the study of this key issue, coordination with RAN WGs is needed before final conclusion.

NOTE 2:	RAN WG will determine how the switching for the UEs belonging to MBS session from CM-CONNECTED state to CM-CONNECTED with RRC Inactive state (and vice versa) is performed by the RAN node.

This contribution is based on S2-2203972 which has been objected by one company at SA2#150 as too RAN centric. However, this contribution has system wide impacts and the related possibilities will need to be considered in the evaluation phase:

- This contribution contains proposal for the activation of MBS sessions with inactive UEs. This topic is also touched by solutions 4 and 17

- As inactive UEs can move freely between RAN nodes within the RAN paging area, additional RAN nodes may need to be made aware of the MBS session and may need to be distribute MBS data. This can be accomplished with more and less core network impact, and different possibilities need to be compared.

# 2		Proposal

It is proposed to include the following changes in TR 23.700-47.

*** 1st Change ***

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 MBS session reception in RRC Inactive | 2 MOCN network sharing | 3 On demand multicast MBS session | 4 Group Message Delivery | 5 Coexistence with existing power saving mechanisms for capability-limited devices | X Improvement on performance issues for public safety UEs |
| 1 | X |  |  |  |  |  |
| 2 |  | X |  |  |  |  |
| 3 | X |  |  |  |  | X |
| 4 | X |  |  |  |  |  |
| 5 | X |  |  |  |  |  |
| 6 | X |  |  |  |  |  |
| 7 |  | X |  |  |  |  |
| 8 |  | X |  |  |  |  |
| 9 |  | X |  |  |  |  |
| 10 |  |  | X |  |  |  |
| 11 |  |  | X |  |  |  |
| 12 |  |  |  | X |  |  |
| 13 |  |  |  | X |  |  |
| 14 |  |  |  |  | X |  |
| 15 |  |  |  |  | X |  |
| 16 |  |  |  |  |  | X |
| 17 |  |  |  |  |  | X |
| x | x |  |  |  |  |  |

*** 2nd Change (all new)***

## 6.X	Solution #X: Procedures for Transmission mode tor inactive data reception

### 6.X.1	Introduction

This solution addresses Key Issue #1.

### 6.X.2	Functional description

This solution builds on top of solution 1.

UEs that transition to RRC inactive mode can move freely in their RAN notification area without notifying the RAN nodes. Their location is thus not known at cell level. At the moment only the RAN node serving a UE is aware that it is participating in an MBS multicast session. Additional RAN nodes not handling UEs within the multicast session themselves but within RAN notification areas of UEs in the multicast session may need to be made aware of the multicast session and perform procedures for the multicast session such as transmitting data of the multicast session and also manage the related the shared delivery path.

Editor's note:	Related procedures need to be defined by RAN WGs. It also neeeds to be studied by RAN WGs whether it is possible to determine the presence of inavtive UEs that desire to receive a multicast session in a cell.

RAN nodes not serving any connected UEs in the MBS sessions thus need to be aware of the MBS session. This solution describes two option to accomplish that with CN impacts:

Option A: RAN nodes inform their peers serving parts of RAN notification areas of UEs in an MBS session about the MBS session. The peer RAN nodes request assistance information from the CN via shared delivery establishment.

Option B: The MB-SMF informs all RAN nodes in a service area of an MBS session via the AMF about the MBS session

Editor's Note: The RAN-centric option A may exist, but is in scope of RAN WGs to a large extent. The Evaluation of this proposal thus requires RAN feedback.

As service areas are optional for MBS sessions, all RAN nodes in a PLMN are possible affected by Option B.

Editor's Note: It may be preferable to only inform RAN nodes within RAN paging areas, but related procedures need to be investigated by RAN WGs

RRC_inactive UEs can move to cells where the transmission mode for RRC connected state is applied and then need to transition to the RRC connected state to receive MBS data.

The current MBS session activation procedures use paging for the MBS session and trigger RRC-inactive UEs to transition to RRC-connected state. For the transmission mode for RRC_inactive reception, RRC-inactive UEs should be made aware that the MBS multicast session is activated but remain in RRC-inactive state.

Editor's note:	Related procedures need to be defined by RAN WGs.

Editor's note:	Procedures to support UEs that move between cells with different transmission mode need to be agreed by RAN WGs.

### 6.X.3	Procedures

Editor's Notes: It is not assumed all the RAN node within the same RNA area have the same capability. It needs to be verified whether there are gaps in the procedures for inhomogeneous deployment

#### 6.X.3.1	Moving a UE to RRC Inactive state and providing assistance information to additional RAN nodes (Option A)

Figure 6.x.3.1-1: NG-RAN node moves a UE in 5MBS session to RRC Inactive state

- UE joined an MBS session and receives MBS data while in RRC connected state

- NG-RAN 1 uses assistance information to decide whether to apply the transmission mode for RRC-inactive receptionfor an MBS session , and whether to move one or several UEs in MBS session to RRC inactive state..

- RAN internal procedures.

3.	NG-RAN 1 determines that part of the RAN notification areas of the UEs is served by RAN 2. It informs RAN 2 that handling of the MBS session for inactive UEs is required in that area,

Editor's note:	Related procedures need be determined by RAN WGs. Depending on RAN decision, assistance information for MBS might be transferred in that step

4.	The NG-RAN 2 establishes shared delivery with MB-SMF. The MB-SMF transmits assistance information as defined in solution 1for inactive reception in the shared delivery response.

5.	RAN internal procedures at NG-RAN 2 to handle MBS session.

Editor's note:	The procedures in this clause need be determined by RAN WGs. However, transfer of assistance information and shared delivery to additional RAN nodes impacts SA2

#### 6.X.3.2	MBS service activation

Figure 6.x.3.2-1: MBS service activation

- When triggered according to step 11 of of Figure 7.2.5.2-1 of TS 23.247 [4], AMF sends NGAP activation request message to NG-RAN nodes.

- Related RAN procedures. RAN nodes decides whether to transmit in mode for RRC inactive reception, and otherwise will perform RAN paging. For the transmission mode for RRC_inactive reception, RRC-inactive UEs should be made aware that the MBS multicast session is activated but remain in RRC-inactive state.

- NG-RAN 1 determines that part of the RAN notification areas of UEs in MBS session is served by RAN 2. It requests RAN 2 to perform RAN paging for MBS session,

Editor's note:	Depending on RAN decision, assistance information for MBS might be transferred in that step.

- The NG-RAN 2 establishes shared delivery with MB-SMF. The MB-SMF transmits assistance information (as defined in solution 1) for inactive reception in the shared delivery response.

- RAN internal procedures at NG-RAN 2 similar to step2.

Editor's note:	The procedures in this clause need be determined by RAN WGs.. However, transfer of assistance information and shared delivery impacts SA2.

#### 6.X.3.3	Triggering MBS service announcement by MB-SMF (Option B)

Editor's note:	Differences to broadcast MBS Session are FFS.

Figure 6.x.3.3-1: Triggering MBS service announcement by MB-SMF (Option B)

- MBS multicast session suitable for transmission mode for inactive reception is created is created at MB-SMF.

2.-5.	MB-SMF informs RAN nodes in service area of multicast session via AMFs of multicast session and asks them to announce multicast session

6. RAN decides transmission mode for MBS session

- RAN related procedures

Editor's note:	The steps 6 to 7 need to be decided by RAN WGs.

### 6.1.4	Impacts on services, entities and interfaces.

Editor's note:	This clause describes impacts to existing services, entities and interfaces.

Functional entities defined in clause 5.3.2 of TS 23.247 [4] are reused.

In addition to impacts for solution 1:

NG-RAN:

-	Decide transmission mode for a multicast session in a cell

-	Receive assistance information either a spart of request for service announcement or within shared delivery Establishment

UE:

-	Receive MBS data in indicated transmission mode while MBS data is active.

MB-SMF:

-	inform RAN nodes in service area of multicast session (Option B)

-	provide assistance infirmation as psrt of shared delivery establishment