3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204816

Elbonia, May 16 – 20, 2022	(revision of S2-2204267r03)

Source:	Huawei, HiSilicon

Title:	KI #4, Sol #13: Update of Solution 13 of 23700-47

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract: It is proposed to clarify the reliable delivery aspect for group message delivery.

1. Introduction

1.1	Addressing the Editor's NOTEs

This solution was introduced in SA2#150E meeting, and the following EN was added to allow further clarification with respect to reliable group message delivery.

Editor's note: How to ensure reliable delivery in transport-only mode (without AL-FEC) over MBS broadcast delivery is FFS.

Group message delivery using MBMS defined in TS 23.682 [6] is possible for SCEF to use MB2 and xMB. Object Distribution Method defined by SA4 in TS 26.502 [11] is comparable to File delivery and Streaming method for eMBMS, and it can accommodate the reliable message delivery by supporting AL-FEC in network. It is beneficial for the case when the reliability is necessary, and/or AF does not support the FEC feature. As such, the group message application using NEF would be more flexible and provide greater functional support.

In fact the proposal of this solution is not aiming at fixing group message delivery for any specific interface, i.e., MB2 or xMB. Therefore, in this document it is proposed to modify the procedure to clarify such aspect.

Moreover, it seems that the solution does not provide sufficient justification for NEF enhancement, and one EN was left to leave more space to exploit/clarify.

Editor's note: Further evidence that the outlined problem and solution is specific to group message delivery would be desirable.

In study phase of GROUPE, it is recorded in the conclusion clause 6.1 of 23.769 that:

Group message delivery using MBMS is more suitable to deliver group messages to large numbers of group members in a particular geographical area. This solution has limited applicability and does not support all the scenarios, e.g. UEs not supporting MBMS, UEs located on areas where MBMS is not deployed. For such scenarios where the UE cannot receive messages via MBMS, message delivery may occur via unicast communication, e.g. via user plane or via the methods using SMS as described in TS 23.682 [3].

And in TS 23.682, features of group message delivery are highlighted as follows:

The specific procedure handling for group message delivery using MBMS is described in clause 5.5.1. The group message delivery using MBMS has limited applicability and does not support all the scenarios, e.g. UEs not supporting MBMS, UEs located in areas where MBMS is not deployed. The SCS/AS may recall or replace a previously submitted MBMS message; this is described in clause 5.5.2.

This solution proposes how to use MBS for group message delivery for the case when the UEs locate in the area where MBS is not deployed. This architecture can be re-used for general group message delivery purposes (not limited to MTC devices).

As analyzed above, it is proposed to clarify the aspects highlighted by the two ENs.

1.2	Determining the PUSH/PULL mode

Moreover, for Object Distribution, since 5GC provides the delivery IP address and port number to the AF, or receives SSM from AF, meaning that the PUSH mode is used. However, the ingestion can be either PUSH mode or PULL mode, if PUSH mode is used between AF and NEF but PULL mode is used between NEF and MBSF/MBSTF, several extra (and might be not necessary) issues needs to be addressed:

-	Payload has not been pushed by AF to the NEF but MBSF/MBSTF requests to pull;

-	Payload has been pushed by AF to the NEF but MBSF/MBSTF has not requested to pull.

To simplify, it's proposed to clarify that only PUSH mode is used.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-47.

# * * * * First change * * * *

## 6.13	Solution #13: Group message delivery for broadcast

### 6.13.1	Introduction

This solution addresses Key Issue #4.

### 6.13.2	Functional description

Editor's note:	This clause outlines solution principles and documents any assumptions made.

It is assumed to reuse the current architecture and TMGI definition in Rel-17 MBS work (see TS 23.247 [4]). In other words, MB-SMF is used to handle MBS session-level management while SMF performs per-UE MBS session management, e.g. authorization, multicast session information provisioning, managing 5GC Individual MBS traffic delivery.

For group message delivery, it mainly targeted for the Machine type communication services. The group message delivery has limited applicability and does not support all the scenarios, e.g. UEs not supporting MBS, UEs located in areas where MBS is not deployed.

In TS 23.682 [6], features of group message delivery are highlighted as follows:

-	"The specific procedure handling for group message delivery using MBMS is described in clause 5.5.1. The group message delivery using MBMS has limited applicability and does not support all the scenarios, e.g. UEs not supporting MBMS, UEs located in areas where MBMS is not deployed. The SCS/AS may recall or replace a previously submitted MBMS message; this is described in clause 5.5.2."

This solution proposes how to use MBS for group message delivery for the case when the UEs locate in the area where MBS is not deployed. This architecture can be re-used for general group message delivery purposes (not limited to MTC devices).

Compared with Rel-17 defined mechanism, the case that UEs located in areas where MBS is not deployed needs to be considered for broadcast.

NOTE: This solution assumes that the AF is aware of the exact location of UEs

### 6.13.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

#### 6.13.3.1	General

NOTE:	The message names in the procedures below are descriptive. It is assumed that the names are updated with corresponding SBI based names where applicable during the normative phase.

#### 6.13.3.2	Broadcast Session Establishment

4. Nmbsmf_MBSSession_Create Request (TMGI, supported area)

UE

NG-RAN

MB-UPF

AF

MBSF

NEF

1. Group message delivery request (TMGI, requested area)

13. Group message delivery response (TMGI, area not supporting MBS)

14. Service announcement

MBSTF

MB-SMF

5. Step 12-15, 17 of TS 23.247 [4] when PCC is not used, or Step 12-28, 30 of TS 23.247 [4] when PCC is used.

6. Nmbsmf_MBSSession_Create Response ()

Transport Only Mode

0a. TMGI allocation procedure (see step 1-6 of TS 23.247 [4])

2. Authorization

3. NEF figures out the area does not support MBS

Service Mode

7a. Nmbsf_MBSUserService_Create Request ([TMGI], supported area)

9. Same as step 5

8. Nmbsmf_MBSSession_Create Request ([TMGI], supported area)

10. Nmbsmf_MBSSession_Create Response ()

15. AF uses unicast for the UEs not appear in the supported area.

7b. Nmbsf_MBSUserDataIngestSession_Create Request ()

11. Nmbstf_MBSDistributionSession_Create

12a. Nmbsf_MBSUserService_Create Response

12b. Nmbsf_MBSUserDataIngestSession_Create Response

0b. Service announcement

7a. Nmbsmf_TMGI allocation

16. Group message delivery

Figure 6.13.3.2-1: Broadcast Session Establishment for Group message delivery

The procedure is based on clause 7.3.1 and 7.1.1.1/7.1.1.2 of TS 23.247 [2], and clause 5 of TS 26.502 [11], the delta parts is highlighted at following:

0.	AF optionally requests the 5GC to allocate the TMGI, details see step 1-6 of TS 23.247 [4]. AF may send service announcement to the UEs.

1.	The AF later further request to establish the MBS session, with including the requested MBS service area in Group message delivery request sent to NEF. If the TMGI is available (step 0 is executed), AF also includes the TMGI in the message.

2.	NEF authorizes the request sent by AF.

3.	NEF figures out the area which does not support MBS.

If Service Mode is used, or MBSF is not involved, step 4-6 are skipped:

4.	NEF invokes Nmbsmf_MBSSession_Create Request, and uses the area supporting MBS as the MBS service area in the message.

5.	MB-SMF continues the MBS Session Creation procedure as defined in TS 23.247 [2].

6.	MB-SMF sends the response message to NEF. If MBSF is involved, the message will go through MBSF.

NOTE:	For Transport Only Mode, it is assumed 5GC will not processing the ingested data therefore the AF is assumed to provide the functionalities e.g., reliability if needed.

If Transport Only Mode is used, step 7-12 are skipped:

7.	NEF invokes Nmbsf_MBSUserService_Create Request, and uses the area supporting MBS as the MBS service area in the message. NEF also sends Nmbsf_MBSUserDataIngestSession_Create Request message to the MBSF in which the MBS service area is the area supporting MBS. If Object Distribution Method is used, the "PUSH" mode is supposed to be used.

Editor's note:	It is FFS whether “PULL” mode in Object Distribution Method is applicable or not. The PUSH requires the NEF to push the object to the MBSTF, while the PULL requires the MBSTF to get the object to the NEF.

8.	MBSF invokes Nmbsmf_MBSSession_Create Request.

9.	Same as step 5.

10. MB-SMF sends Nmbsmf_MBSSession_Create Response message to MBSF.

11.	MBS Distribution Session is created as defined in TS 26.502 [11].

12. MBSF responses the messages from NEF in step 7.

13.	NEF responses AF with Group message delivery response. NEF includes the areas in the requested MBS service area that do not support MBS. NEF may also send the address to the AF.

14.	AF may send service announcement.

15.	For the UEs locating out of the supported area, AF uses unicast for sending the data to the UE.

16.	If Group Message Payload was included in step 1, then the NEF delivers to MBSTF/MB-UPF the Group Message Payload(s). If Group Message Payload was not included in step 1, the AF transfers the content to be delivered to the group to the NEF using the address received at step 13.

Editor's note:	It is to be clarified in detail about in payload delivery: If Group Message Payload included in step 1, how NEF handles the payload and deliver what to MBSTF or MB-UPF. If Group Message Payload not included in step 1, how AF transfer the content to NEF, and how NEF transfer the content to which NF.

### 6.13.4	Impacts on services, entities and interfaces

Editor's note:	This clause describes impacts to existing services, entities and interfaces.

Functional entities defined in clause 5.3.2 of TS 23.247 [4] is reused exception for the following additions:

NEF:

-	Support differentiating the area with supporting MBS and non-supporting MBS.

-	Support responding AF with the information that not supporting MBS.

-	User plane handling of the group communication message.

# * * * * End of changes * * * *