3GPP TSG SA2 Meeting #151 (e-meeting) 	S2-2204901

16 – 20 May, 2022, Elbonia 	(revision of S2-2204041r03)

# Source:	LG Electronics

# Title:	New WID: MBS support for V2X services (TEI18_MBS4V2X)

# Document for:	Approval

Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	MBS support for V2X services

Acronym:	TEI18_MBS4V2X

Unique identifier:

Potential target Release:	Rel-18

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes | X | X | X | X |  |
| No |  |  |  |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 720011 | Architecture enhancements for LTE support of V2X services | Rel-14 Stage 2 work item to specify EPS architecture enhancements for V2X services |
| 820018 | Architecture enhancements for 3GPP support of advanced V2X services | Rel-16 Stage 2 work item to specify 5GS architecture enhancements for V2X services |
| 900009 | Architectural enhancements for 5G multicast-broadcast services | Rel-17 Stage 2 work item to specify 5GS architecture enhancements for MBS |

3	Justification

MBS (Multicast/Broadcast Service) can support V2X services and V2X applications such as V2V (Vehicle-to-Vehicle), V2I (Vehicle-to-Infrastructure), V2N (Vehicle-to-Network) and V2P (Vehicle-to-Pedestrian).

For example, MBS Broadcast can be used to distribute V2V messages and V2P messages to UEs. MBS Multicast can be used for V2N such as navigation map update or software update by V2X application server to UEs.

MBMS (Multimedia Broadcast/Multicast Service) is supported for V2X communication over LTE-Uu reference point as specified in TS 23.285. On the other hand, there is no description about MBS support for V2X communication over Uu reference point in TS 23.287. TS 23.287 describes that V2X communication over Uu reference point is only unicast.

According to TS 23.501, 5QI = 75 was reserved for V2X message delivery via MBS as the following NOTE reads.

NOTE 14:	This 5QI is not supported in this Release of the specification as it is only used for transmission of V2X messages over MBMS bearers as defined in TS 23.285 [72] but the value is reserved for future use.

Because MBS for 5GS was specified in Rel-17, aspects of V2X communication via MBS need to be described in TS 23.287 by referring to aspects of V2X communication via MBMS specified in TS 23.285.

In addition, QoS characteristics values for 5QI = 75 need to be defined by referring to QCI = 75 specified in TS 23.203.

4	Objective

The objective of this work item is to:

-	Add description and procedures related to MBS support for V2X services including:

-	General concept related to V2X communication via MBS

-	MBS for Uu based V2X architecture reference model

-	Functionalities for UE, PCF and V2X Application Server related to V2X communication via MBS

-	Policy/Parameters related to V2X communication via MBS

-	V2X message transmission and reception via MBS

-	V2X Application Server discovery via MBS broadcast session

-	MBS User Service Description for V2X use

-	Define QoS characteristics values for 5QI = 75 that could be used for V2X message delivery via MBS.

NOTE:	AN impact in this work item is only due to new 5QI = 75.

This work item will require 1.0 TU(s) to discuss and agree on the corresponding CRs.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.287 | Add description and procedures related to MBS support for V2X services | SA#99 (Mar. 2023) |  |
| TS 23.501 | Define QoS characteristics values for 5QI = 75 | SA#99 (Mar. 2023) |  |

6	Work item Rapporteur(s)

LaeYoung Kim (LG Electronics), laeyoung.kim@lge.com

7	Work item leadership

SA2

8	Aspects that involve other WGs

None

9	Supporting Individual Members

Supporting IM name

LG Electronics

KT Corp.

LG Uplus

ETRI

AT&T

Tencent

Tencent Cloud

SyncTechno Inc.

Xiaomi

ZTE

Hansung University

China Mobile

TOYOTA MOTOR CORPORATION

vivo

CBN

CATT

Huawei

Hisilicon

Volkswagen AG

OPPO

Qualcomm Incorporated

InterDigital