3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204190

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1, Sol#3: Update for no downlink data before the first PTW

Document for:	Approval

Agenda Item:	9.8

Work Item / Release:	FS_REDCAP_Ph2 / Rel-18

Abstract: Update of Solution l#3 to handle the cases when there is no DL data before the first PTW and when the time between PTWs after the first might be different.

# 1. Introduction

For Sol#3, the SMF/UPF enables the CN buffering based on the buffer timer from RAN. When the buffer timer expires, the SMF/UPF transmits the buffered data to trigger RAN paging. However, when there is no DL data buffered in the UPF and no MO data/signalling triggered by UE, the UE still stays in RRC_Inactive state. Then SMF/UPF needs to re-enable the CN buffering functions again. As the time between when the UE enters RRC_Inactive and the first PTW after that, and the time between PTWs can be different an initial buffer time and a re-buffering timer values are needed.

Therefore, Solution #3 is updated to restart the buffering time in the above case.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-68.

# * * * * First change * * * *

## 6.3	Solution #3: MT signalling and data handling for UE with long eDRX cycle in RRC_Inactive state and UPF buffering

### 6.3.1	Description

This solution resolves Key Issue #1 for MT signalling and data handling for UE with long eDRX cycle in RRC_Inactive state.

As specified in TS 23.501 [2], NG-RAN decides the eDRX in RRC_Inactive state based on the eDRX parameters in IDLE mode. NG-RAN also determines the buffer timer which is used to indicate the time needed for SMF/UPF to buffer the MT data. Before transitioning the UE into RRC_Inactive state, NG-RAN sends the buffer indicator and buffer timer to the AMF to enable data buffering in Core Network.

Then when MT data arrives, the SMF/UPF will buffer the MT data and forward the buffered data to NG-RAN to trigger RAN based paging when the buffer timer expires. If the UE moves out of the original NG-RAN, the data is transfer between the original and new NG-RAN nodes as needed.

If there is a MO data when the UE is during the eDRX in RRC_Inactive state, the NG-RAN forwards the MO data to UPF to disable the data buffering and any MT buffered data is sent to RAN. If the UE enters RRC_CONNECTED without MO data then then RAN can inform the AMF that the UE is no longer in RRC_Inactive and that can trigger delivery of buffered MT to RAN or delivery of MT signalling.

The AMF stores an indication of when the UE is in RRC_Inactive with a long eDRX cycle and uses that indication to determine how to handle paging, how to respond to reachability and event exposure requests, enabling the use of HLCom and SMS over NAS with long eDRX cycles in RRC_Inactive.

### 6.3.2	Procedures

Figure 6.3.2-1: high level procedure for MT data handling of UE with long eDRX in RRC_Inactive state

1.	The UE initiates the Registration Procedure and the eDRX parameters in CM-IDLE state are negotiated.

2.	The AMF sends the Inactive Assistance Information to the NG-RAN. The eDRX parameters in CM-IDLE state are included as defined in clause 5.3.3.2.5 of TS 23.501 [2].

3.	Before transitioning the UE into RRC_Inactive state, NG-RAN determines the eDRX parameters for UE in RRC_Inactive state.

4.	If the eDRX cycle in RRC_Inactive state is larger than 10.24s, NG-RAN may send the N2 message to inform the AMF that UE is entering RRC_Inactive state to enable CN buffering. The buffer indicator and the buffer timer are included to indicate to the AMF to enable data buffering until the buffer timer expires. The CM state of the UE is not update by the indication from NG-RAN, i.e. the UE remains in CM-CONNECTED.

5.	If the AMF receives the notification from NG-RAN that the UE will enter RRC_Inactive state and need to buffer downlink data in the CN, AMF sends the buffer indication and buffer timer to SMF to enable data buffering. The AMF stores an indication that the UE is in RRC_Inactive state with a long eDRX cycle and is therefore not reachable until buffer timer expires. Optionally, the buffer timer contains an initial buffer timer (i.e. time until the start of the first PTW) and re-buffer timer (i.e. the eDRX cycle length time between the start of subsequent PTWs) for the case when the UE remains in RRC_Inactive.

If the AMF receives a Namf_EventExposure_Subscribe request for UE Reachability or Availability after DDN failure and if the AMF has stored the indication that the UE is in RRC_Inactive with a long eDRX cycle then the AMF treats the request as if the UE was in CM_IDLE and sends any responses or events for when UE can be paged (i.e. buffer timer expires).

For MT signalling the AMF takes into account the indication that the UE is in RRC_Inactive state with a long eDRX cycle and triggers the MT signalling for when UE can be paged (i.e. buffer timer expires).

If the AMF receives a Namf_MT_EnableUEReachability Request while the AMF has an indication that the UE is in RRC_Inactive with a long eDRX cycle, then the Namf_MT_EnableUEReachability Response is sent back to the requesting NF that the UE is not reachable until when UE can be paged (i.e. buffer timer expires).

6.	The SMF may configure UPF with the new Buffering Action Rule and buffer timer to enable downlink data buffering for the UE until the buffer timer expires.

7.	Based on the new BAR and buffer timer, the UPF starts the buffer timer with the initial buffer timer and enables data buffering for the UE in RRC_Inactive state.

NOTE:	When the initial buffer timer expires and there is no DL buffered data, the buffer timer is always restarted with the re-buffer timer value, unless there is a new value from RAN.

8.	The UE may be moved to RRC_Inactive state. This step may occur in parallel with steps 4 - 7. In case the UE was moved to RRC_Inactive state before the UPF was reconfigured, the RAN node may buffer DL data that is forwarded to the RAN node for the duration of the eDRX cycle or discard the data based on RAN configuration.

9.	When MT data arrives at the UPF, the UPF buffers it until the buffer timer expires.

10.	The UPF may forward the buffered data to the NG-RAN when the buffer timer expires, so NG-RAN can trigger paging for the UE in time. The NG-RAN node buffers the data from the UPF during the paging procedure.

11.	The NG-RAN sends the RAN Paging message to the UE.

12.	The UE responds to paging from RAN.

13.	The response to paging may not be in the same RAN node which moved the UE to RRC_Inactive state. If the UE has moved away from of the RAN node that moved the UE to RRC_Inactive state, the UE will initiate the RNAU procedure and the context will be transferred between the original RAN node and the new RAN node.

If there is no Xn interface between the original and the new RAN nodes, the Core Network can be used to transfer the UE context or buffered data from the original RAN node to the new RAN node. For the UE context in the original RAN node, the AMF can help to retrieve the UE context from the original RAN node and send it to the new RAN node in a container. Similarly, an indirect data forwarding path is built from the original RAN node, the anchor UPF and the new RAN node to enable the buffered data to be transferred.

14.	The MT data is sent to UE.

15.	If there is MO data/signalling triggered connection resume between UE and NG-RAN, the UE enters CM-CONNECTED state.

16.	The NG-RAN notifies the AMF that the UE's RRC state has been changed from RRC_Inactive to RRC_CONNECTED state.

17.	The AMF may also initiate the Nsmf_PDUSession_UpdateSMContext service to notify SMF about the state change.

18.	Accordingly, the SMF decides disables the data buffering in the UPF and the UPF sends the any buffered data to RAN.

19. If the UE initiated connection resume is triggered by UL data traffic, the NG-RAN sends the UL data to the UPF, which triggers the UPF to disable the data buffering and forward any DL traffic.

### 6.3.3	Impacts on services, entities and interfaces

RAN:

-	Derive the buffer timer and send it to AMF together with the buffer indicator.

-	Proactively inform AMF about the RRC states of UE.

-	Support CN assisted AS UE context retrieval and data forwarding.

AMF:

-	Store the indication that the UE is in RRC_Inactive with a long eDRX cycle until the buffer timer expires and replies to requests or sends events if relevant requests were received while it was set.

-	Support CN assisted AS UE context retrieval and data forwarding.

UPF:

-	Disable data buffering and forward buffered data when buffer timer expires or there is any UL data traffic.

-	Restart buffer timer if there is no DL data buffered or new buffer timer from RAN.

SMF:

-	Disable data buffering and forward buffered data when buffer timer expires or there is any UL data traffic.

# * * * * Second change * * * *

## 7.1	General

There are four solutions proposed to address the objective to enable long eDRX when the UE is in RRC-Inactive. The table below is to provide an overview of these solution. The solutions being:

Solution 1: Power saving enhancement with long eDRX cycle support for UE in CM-CONNECTED with RRC_INACTIVE state.

Solution 2: Long eDRX with RAN buffering.

Solution 3: Solution #3: MT signalling and data handling for UE with long eDRX cycle in RRC_Inactive state and UPF buffering.

Solution 4: RRC Inactive with CM-IDLE for eDRX>10.24s.

|  | Solution #1 | Solution #2 | Solution #3 | Solution #4 |
| --- | --- | --- | --- | --- |
| State Combination(s) | RRC Inactive/CM-Connected | RRC Inactive/CM-Connected | RRC Inactive/CM-Connected | RRC Inactive/CM-Idle |
| Node that configure the UE eDRX in RRC-Inactive | RAN (at RRC Release) | RAN (at RRC Release) | RAN (at RRC Release) | RAN (at RRC Release) |
| AMF awareness of RRC-Inactive state needed | Yes | No | Yes | Yes |
| User data Buffering | UPF/SMF | RAN If RAN fails to buffer according to configured eDRX, RAN will initiate AN release procedure towards the CN. UE is moved to CM-Idle in the CN and the state in the UE remains as CM-Connected. | UPF | UPF/SMF |
| MT Signalling | AMF is aware of eDRX information | AMF -> RAN forward NAS msg. If the UE is not reachable then: RAN -> AMF releases UE context with Notification that UE is in eDRX cycle. UE is moved to CM-Idle. AMF may try again later based on eDRX information | AMF is aware of buffer timer (initial and re-buffer timers) | AMF is aware of eDRX information |
| UE RRC Release timing. | UE may be released before AMF is notified | N/A | UE may be released before AMF is notified | UE is released after AMF responds to the N2 suspend as specified in clause 4.8.1 of TS 23.502 [3] |
| Signalling when UE moves from RRC Connected to RRC Inactive | RAN-> AMF Notification (eDRX config) AMF-> SMF -> UPF PDU session modification | None | RAN-> AMF Notification (buffer timer) AMF-> SMF -> UPF PDU session modification (buffer timer) | RAN-> AMF Suspend message (eDRX config) AMF-> SMF -> UPF PDU session modification |
| Signalling when UE moves from RRC Inactive to RRC Connected | RAN-> AMF Notification AMF-> SMF -> UPF PDU session modification | None | RAN-> AMF Notification and then AMF-> SMF -> UPF PDU session modification for MO triggered connection resume. | RAN-> AMF Resume message AMF-> SMF -> UPF PDU session modification |
| Information exchanged among NFs | AMF communicates with other NFs on the UE reachability the same way as in CM-IDLE modes deducing UE reachability information based on gNB provided eDRX value. | None EN: It is FFS whether and how the AMF may support UE reachability notification based on IDLE mode eDRX information also in CM-Connected. | AMF communicates with other NFs on the UE reachability based on the buffering information provided by gNB. | AMF communicates with other NFs on the UE reachability the same way as in CM-IDLE modes deducing UE reachability information based on gNB provided eDRX value. |
| Paging | AMF trigger UPF to forward buffered data based on "stored extended DRX information" to trigger RAN paging If UE does not respond data will be dropped | RAN trigger paging based on UE eDRX configuration If UE does not respond data will be dropped. Optional: RAN may escalate to AMF for paging within RA. | UPF forwards buffered data based on "buffer timer" to trigger RAN paging. If UE does not respond data will be dropped. | AMF send a paging request with new indication to trigger RAN paging. If UE does not respond AMF can escalate CN paging within RA. |
| Mobility within RNA | UE trigger RNAU New RAN node fetch UE context from anchor RAN node and perform path switch | UE trigger RNAU New RAN node fetch UE context from anchor RAN node and perform path switch | UE trigger RNAU New RAN node fetch UE context from anchor RAN node and perform path switch | UE trigger RNAU New RAN node fetch UE context from anchor RAN node and perform path switch |
| Mobility outside RNA | UE trigger RNAU RAN fetch UE context if Xn. If RAN is unable to fetch the UE context, RAN release UE to RRC Idle. No UE state change notification to CN If UE has MO data, the UE will perform SR from Idle | UE trigger RNAU a) RAN fetch UE context if Xn. b) RAN fetch UE context via AMF (buffered data will be forwarded) If RAN is unable to fetch the UE context, RAN release UE to RRC Idle. No UE state change notification to CN If UE has MO data the UE will perform SR from Idle | UE trigger RNAU a) RAN fetch UE context if Xn. b) RAN fetch UE context via AMF (buffered data will be forwarded) If RAN is unable to fetch the UE context, RAN release UE to RRC Idle. No UE state change notification to CN If UE has MO data the UE will perform SR from Idle | UE trigger RNAU RAN fetch UE context if Xn. If RAN is unable to fetch the UE context, RAN release UE to RRC Idle. No UE state change notification to CN If UE has MO data the UE will perform SR from Idle |
| UE signalling in RRC-Inactive | UE performs only periodic update signalling at AS. | UE performs only periodic update signalling at AS. | UE performs only periodic update signalling at AS. | UE performs periodic update signalling at both AS and NAS. |

# * * * * End of changes * * * *