SA WG2 Meeting #151e	S2-2205106

# May 16th – May 20th; Elbonia               		(revision of S2-2203944r03)

Source: 	Intel, Vivo

Title: 	KI#8: New Solution to support FL training updates to NWDAF containing AnLF from NWDAF containing MTLF

Document for: 	Approval

Agenda Item: 	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract of the contribution:

# 2 Proposal

It is proposed to update TR 23.700-81 as follows.

Start of change

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| 1 | X |  |  |  |  |  |  |  |  |  |
| 2 | X |  |  |  |  |  |  |  |  |  |
| 3 | X |  |  |  |  |  |  |  |  |  |
| 4 | X |  |  |  |  |  |  |  |  |  |
| 5 | X |  |  |  |  |  |  |  |  |  |
| 6 | X |  |  |  |  |  |  |  |  |  |
| 7 | X |  |  |  |  |  |  |  |  |  |
| 8 |  | X |  |  |  |  |  |  |  |  |
| 9 |  | X |  |  |  |  |  |  |  |  |
| 10 |  |  | X |  |  |  |  |  |  |  |
| 11 |  |  | X |  |  |  |  |  |  |  |
| 12 |  |  |  | X |  |  |  |  |  |  |
| 13 |  |  |  |  | X |  |  |  |  |  |
| 14 |  |  |  |  | X |  |  |  |  |  |
| 15 |  |  |  |  | X |  |  |  |  |  |
| 16 |  |  |  |  |  | X |  |  |  |  |
| 17 |  |  | X |  |  | X |  |  |  |  |
| 18 |  |  |  |  |  |  | X |  |  |  |
| 19 |  |  |  |  |  |  | X |  |  |  |
| 20 |  |  |  |  |  |  | X |  |  |  |
| 21 |  |  |  |  |  |  |  | X |  |  |
| 22 |  |  |  |  |  |  |  | X |  |  |
| 23 |  |  |  |  |  |  |  | X |  |  |
| 24 |  |  |  |  |  |  |  | X |  |  |
| 25 |  |  |  |  |  |  |  |  | X |  |
| 26 |  |  |  |  |  |  |  |  | X |  |
| 27 |  |  |  |  |  |  |  |  | X |  |
| X |  |  |  |  |  |  |  | X |  |  |

Next Change (all new text)

## 6.X	Solution #X: FL training update to NWDAF containing AnLF from NWDAF containing MTLF

### 6.X.1	Description

This solution is proposed for KI#8 to support the Federated Learning procedure between different NWDAFs containing MTLF and provide Federated Learning training updates from NWDAF containing MTLF (with FL aggregation capability/ in the role of FL server) to NWDAF containing AnLF.

### 6.X.2	Procedures

Procedure for Federated Learning training updates from NWDAF containing MTLF to NWDAF containing AnLF is described in Figure 6.X.2-1.

Figure 6.X.2-1: Procedure for FL training update to NWDAF containing AnLF from NWDAF containing MTLF

Editor’s Note: The aspects related to trained ML model(s) sharing should be aligned with Key Issue #5.

- NWDAF containing AnLF discovers one or more instances of NWDAF containing MTLF via the NRF as illustrated in steps 0a, 0b, 0c.

- FL training between NWDAF containing MTLF (with Federated aggregation capability) and NWDAF containing MTLF (with Federated participation capability) is performed as described in Solution 21 Figure 6.21.2.3-1 or Solution 23 Figure 6.23.2-1.

- The NWDAF containing MTLF sends Nnwdaf_MLmodelProvision_Notify message with Analytics ID(s), ML model ID(s), ML model file address, ML model serialization format, Training Accuracy metric per ML model ID. The training accuracy metrics indicates the ML model accuracy when the NWDAF containing MTLF performs training using training dataset.

- [conditional] NWDAF containing AnLF sends Nnwdaf_MLModelTrainingUpdate_Subscribe message to the NWDAF containing MTLF with Analytics ID(s), ML model ID(s), Base Accuracy metric, Notification Correlation ID. The same ML model ID(s) as provided in step 2 are included. The Base Accuracy metric is an accuracy metric determined by the NWDAF containing AnLF using the dataset from live network. The Base Accuracy metric is provided by the NWDAF containing AnLF to notify the NWDAF containing MTLF, when the same ML model as in step 2 or a new ML model (for a given Analytics ID) is available with training accuracy higher than Base Accuracy metric.

Editor's note: The Accuracy metric used depends on the conclusion of KI#1.

- [conditional] NWDAF containing MTLF sends Nnwdaf_MLModelTrainingUpdate_Notify message with Analytics ID(s), ML model ID(s), Training Accuracy metric. The ML model ID(s) included in the message may be the same re-trained ML model as provided in step 2 with new training accuracy higher than the Base Accuracy metric provided in step 3 or the ML model ID(s) may be a new trained ML model available for the Analytics ID(s) provided in step 3 with training accuracy level higher than the Base Accuracy metric.

- [conditional] NWDAF containing AnLF decides if it wants to use the ML model ID provided in step 4.

- [conditional] NWDAF containing AnLF sends Nnwdaf_MLModelProvision_Request with Analytics ID(s), ML model ID(s) provided in step 4.

- [conditional] NWDAF containing MTLF sends Nnwdaf_MLModelProvision_Response with Analytics ID(s), ML model ID(s), ML model file address, Training Accuracy metric.

End Change