SA WG2 Meeting #S2-151E	          S2-2205135

16 - 20 May, 2022, Electronic meeting	(revision of S2-2203948r02)

Source:	CableLabs, Charter

Title:	Access SNPN via 3GPP and N3GPP AN using same credentials and credential holder

Document for:	Approval

Agenda Item:	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: Proposes a solution for access to SNPN via 3GPP and N3GPP AN using same credentials and credential holder.

# 1	Background

Proposes a solution for inclusion in TR 23.700-08.

* * * * Start of Change * * * *

## 6.X	Solution X: Access SNPN via 3GPP and N3GPP AN using same credentials and credential holder

### 6.X.1	Introduction

This solution addresses KI#2.

The solution defines how the same credentials from a credential holder external to the SNPN can be leveraged for devices accessing SNPN via 3GPP and non-3GPP access network (both connected to 5GC).

Many enterprise networks have existing deployments with non-3GPP network infrastructure (WLAN or Wireline access) using the AAA server to authenticate the end-devices. The addition of SNPN deployments could leverage the already provisioned identities and credentials to authenticate devices accessing SNPN via 3GPP and non-3GPP access networks both connecting to 5GC via a credential holder (AAA server) external to the SNPN.

### 6.X.2	Functional Description

- The same AAA Server (external credential holder) is used for primary authentication of a UE accessing the SNPN via a 3GPP or non-3GPP access network (e.g., WLAN or Wireline access network) re-using the architecture defined for "Credentials Holder for primary authentication and authorization" as defined in clause 5.30.2.9.2 of TS 23.501 [1]. A N3GPP device accessing the SNPN via non-3GPP access network (e.g., WLAN or Wireline access network) can also leverage the same external credential holder for authentication.

-	The UE connecting via the 3GPP access or Trusted and Untrusted Non-3GPP access network, the 5G_RG and FN-RG connected via Wireline access to the SNPN uses the same permanent identity and credentials. The existing identity and credentials used for WLAN and Wireline access authentication can be re-used for SNPN access using a SUPI in NAI format as defined in clause 28.7.2 of TS 23.003 [2].

-	The SNPN access via NG-RAN and the non-3GPP network (e.g., WLAN or Wireline access network) provides access to the same Data Network e.g., internet or enterprise network.

-    Seamless mobility between the NG-RAN and the Trusted and Untrusted Non-3GPP access network, the 5G_RG and FN-RG connected via Wireline access for accessing the SNPN is supported by this architecture.

Editor’s Note: The scenario of UE and N3GPP device connected to 5GC via 5G-RG/FN-RG, is FFS. This scenario shall take into account the conclusion of 5WWC SID on the support of the device behind an RG.

AAA Server

External Credential Holder

NSSAAF

AUSF

AF

PCF

NRF

NEF

UDM

Nsmf

Namf

Nnssaf

NSSF

AMF

SMF

Uu

N2

N3

N6

N3

N2

N4

Nausf

Naf

Npcf

Nnrf

Nnef

Nnssf

Nudm

Trusted / Untrusted

Access

Network

Wireline

Access

Network

UPF

DN

N11

SNPN

(R)AN

N3

N9

N3IWF/TNGF

W-AGF

N2

N1

FN-RG

5G-RG

N1

3GPP Device

N3GPP Device

Figure 6.X.2.1: Access to SNPN via 3GPP and non-3GPP access networks (both connected to 5GC) using the same credentials from an external credential holder

### 6.X.3	Procedures

0 	If the SNPN supports the architecture as in Figure 6.X.2-1 and wants the UE to use the same credentials for accessing SNPN via 3GPP or Trusted and Untrusted Non-3GPP  access network and wireline access network , then it is configured with the same permanent identity and credentials for primary authentication with the NG-RAN and the non-3GPP access network (Trusted and Untrusted Non-3GPP  access network, and wireline access network).

1) A connection is established between a UE towards 5GC via the untrusted non-3GPP access using the procedures as defined in clause 4.12 of TS 23.502 [4] or a connection is established between a 3GPP capable device towards 5GC via the trusted non-3GPP access using the procedures as defined in clause 4.12a of TS 23.502 [4] or a connection is established between the UE behind 5G_RG / FN-RG towards 5GC via the wireline access using the procedures as defined in clause 4.10 of TS 23.316 [5].

2) A connection is established between a UE and the SNPN network via NG-RAN and 5GC as defined in clause 5.30.2 of TS 23.501 [1].

The same credentials are used for authentication by an external credential holder (AAA server) for primary authentication of a device accessing the SNPN via 3GPP or the non-3GPP access network (e.g., WLAN or Wireline access network), re-uses the architecture defined for "Credentials Holder for primary authentication and authorization" as defined in clause 5.30.2.9.2 of TS 23.501 [1]

A UE may establish connection towards SNPN 5GC using either or both 3GPP and non-3GPP access network (Trusted and Untrusted Non-3GPP access network, and wireline access network) simultaneously.

### 6.X.4	Impacts on services, entities, and interfaces

UE:

-	Use the same permanent identity and credentials for primary authentication for accessing the SNPN via NG-RAN and the non-3GPP network (Trusted and Untrusted Non-3GPP access network, and wireline access network).

No other impacts in the UE, NG-RAN, and 5GC are identified.

* * * * End of Change * * * *