SA WG2 Meeting #S2-151E	S2-2205137

16 – 20 May , 2022, E-meeting                               rev of S2-2204629r05

Source:	Xiaomi

Title:	Hosting network to provide localized service

Document for:	Approval

Agenda Item:	9.4

Work Item / Release:	FS_eNPN_ph2/Rel-18

Abstract of the contribution: This paper proposes a solution for KI#3 on how to enable hosting network to provide localized service for the UE

Discussion

5.3 Key Issue #3: Enabling NPN as hosting network for providing access to localized services 5.3.1 Description Providing access to local services refers to the capability to provide access to a hosting network and a set of services offered by the hosting network provider, and 3rd party service providers including other network operators and 3rd party application providers. The services may be localized (i.e. provided at specific/limited area) and may be bounded in time. The user may become aware of the available access to local services, and the process to gain and terminate access to the hosting network and local services. This process should be efficient, and convenient from a user experience standpoint. Providing access to local services creates new opportunities for users and service providers. For example, access can be provided in areas where there is no coverage provided by other networks (for example, on a cargo ship out at sea), or the access and local services can be established as needed (on a short-term basis), without the need for long term business relationships, permanently installed equipment, etc. The type of local services and access for localized services via a hosting network can be promoted and arranged through different channels. Principally the Localized service provider (e.g. brick and mortar businesses, construction contractors, first responder agencies, etc.) will provide information and proper incentive or instructions to potential users so that they will seek to access the local services via hosting networks. The 5G network as hosting network offering access to such localized services can be either a PNI-NPN or an SNPN. It is assumed that hosting network and the localized services can be operated by different entities. Localized services may provide more than just data connectivity to end users, e.g. additional information/incentive/instructions in order to seek access to the localized services in. This key issue aims at addressing how to enable a NPN (i.e. a SNPN or a PNI-NPN) as a hosting network for providing access to localized services with the following aspects: - Define hosting NPN and identify the difference(s) between hosting NPN and NPN defined in both Rel-16 and Rel-17. - Define localized services and identify the difference(s) between localized services and regular services. - Define where and when localized services are available based on localized service agreements (i.e. a service agreement for a localized service). NOTE 1: SA WG2 works under the assumption that the relationships and the localized service agreements are already available by means outside SA2 scope i.e. SA2 is to agree where the information is made available. - What is required to enable communication between a network operator deploying a hosting network and a localized services provider: - Investigate which type of interaction (e.g., configuration of the hosting network, information reporting) is needed, in such relation to enable the localized services provider for making the best use of the hosting network; and NOTE 2: Collaboration with SA WG5 and SA WG3 may be needed.

Proposal

It is proposed to add following new solution into the TR 23.700-08 "Study on enhanced support of Non-Public Networks; Phase 2"

*** Start of changes ***

6.X	Hosting network to provide localized service based on default credentials

6.X.1	Introduction

The solution addresses key issue # 3 Enabling SNPN as hosting network for providing access to localized services.

The solution aims to enable hosting network to provide the localized service for the UE.

6.X.2	Functional Description

In this solution, it is assumed that UE has default credentials that can be used in the hosting network.

Hosting network broadcasts a dedicated Hosting networking service indication to show that this NPN can provide hosting networking for localized services.

The localized services information are pre-configured in AMF of hosting network.

For the network selection aspect, there are following assumptions:

- In case there is some service agreement between hosting network and home network, so that the UE can be successfully authenticated by the hosting network by using credentials of home network. Automatic network selection is performedThe manual network selection is performed.

- In case there is no service agreement between hosting network and home network, the hosting network performs authentication by using the UE’s default credentials. After successful authentication, hosting network can allow the UE to  initiate restricted PDU session to the localized service, and may perform secondary authentication during the PDU session establishment.  Manual network selection is performed.

Editor’s Note: Details of the restricted PDU session are FFS

Editor’s NOTE: the nature of the default credentials and how to perform the authentication by using the default credentials in the hosting network is FFS. How to pre-configure the default credentials is FFS. Where the subscription corresponding to the default credentials is stored is FFS. Whether further security is needed should depend on SA3.

6.X.3	Procedures

Figure 6.x.3-1 Hosting network providing localized service

- SNPN network broadcasts a Hosting Network Service Indication, to say that the hosting network can support the UE to access to localized service

- UE performs network selection and selects the hosting network, based on the pre-configuration and broadcasting information from hosting network.

In case there is no service agreement between hosting network and home network, manual network selection is performed.

- UE initiates the registration request with UE’s information and Hosting Network Service Indication to gNB

- gNB selects the AMF that supporting Hosting Network Service based on the indication in step 2

- AMF performs the authentication and authorization by using the credentials from home network if there is some service agreement between hosting network and home network. 
AMF performs authentication and authorization by using the default credentials pre-configured in the UE, when there is no service agreement between hosting network and home network.

Editor’s Note: What exactly are the “hosting network default credentials”, how they are used and who/how they are configured in the UE is FFS

- AMF sends UE the localized services information that can be provided by the hosting network when AMF determines to accept UE’s request. The localized information may include IP address/FQDN/DNN of the localized service.

6-7, AMF sends the registration accept to UE via gNB, including localized service information

- UE initiates the PDU Session to dedicated DNN that can provide expected localized service, if the expected localized service is in the received localized list if the list is available.

6.X.4	Impacts on services, entities, and interfaces

network impact:

- AMF configured with localized service information

- gNB broadcasts the Hosting Network Service Indication.

Editor’s Note: Default credential aspects, restricted PDU session aspects

*** End of changes ***