SA WG2 Meeting #S2-151E	S2-2205145

16 - 20 May, 2022, E-Meeting

Source: 	Ericsson

Title: 	KI #4,KI #6, New Sol: Temporary network reselection for localized service support

Document for: 	Agreement

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This paper proposes a solution to enable a UE to enjoy localized services through temporary network reselection for Key Issue #4 and #6.

# Discussion

For enabling PALS, the hosting network can be either a regular PLMN or an NPN (i.e. PNI NPN or SNPN) which provides localized services to the UE. Either, separate credentials/subscriptions are needed for the hosting network, or the hosting network can have an onboarding mechanism by which it allows any UE to configure access (e.g., using a credentials server). Alternatively, a hosting network and localized services can be accessed using the same subscription credentials as for accessing a home network. If a UE is already registered to a network that is not the hosting network at the time / location when the localized services is about to start, then a mechanism is required for how the UE switches to the hosting network offering the localized services.

This paper provides a solution for the scenario when the UE has subscribed to a service in a hosting network and a network reselection from a home/or other network to a hosting network is needed. The existing network selection procedures are described in 3GPP TS 23.122. This procedure needs to be adapted to support temporary localized services in the UE without interruption, either leading to re-prioritization of networks or temporarily avoid triggering of the current automatic network selection procedure. The issue with the current procedure is that it causes to the UE to periodically re-select a higher priority network, like the home network, when available. A procedure is, therefore, required that lets the UE to enjoy localized services from a hosting network, without the need of manual network selection towards the hosting network or without returning to the higher priority network. Such a solution would also enable the network to control the procedure for returning, e.g., to the home network, after the termination of the localized service.

This solution was proposed by S2-2202168 in SA2#150E, but not handled during the meeting. '

The editor's note regarding the authorized for the temporary network reselection is addressed by ultilizing the SOR procedure, or by pre-authorization via UE configuration.

Editor's note:	It is FFS whether and how the home network would authorize the temporary network reselection.

The editor's note regarding the new network selection after UE deactivates the temporary network reselection is addressed by storing the previous registered network before UE activates the temporary network reselection, and leave to CT1 to decide how to use the stored information.

Editors' note:	It is FFS how UE determines the previous registered network when performing normal network selection after the temporary network reselection is deactivated.

Regarding the criteria that enable UE to activate the termporary network reselection, it is proposed to use time and location as the criteria.

Editor's note:	It is FFS what criteria can include, except a start and a stop time.

But home network/service connectivity are also part of the SA1 requirements (listed below) for UE to select hosting network. So, the previous editor's note is replaced with a new one which focus on the how to address home network related criteria.

If the UE is able to obtain services from two networks simultaneously, it may additionally select the hosting network.

If the UE cannot maintain the connection to the home network while selecting the hosting network, the selection shall only be done on request by the user, i.e., using manual selection.

# Proposal

Add the following solution to TR 23.700-08.

*** BEGIN CHANGES ***

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | X |  |  |  |  |  |
| 2 |  | X |  |  |  |  |
| 3 |  | X |  |  |  |  |
| 4 |  | X |  |  |  |  |
| 5 |  | X |  |  |  |  |
| 6 |  | X |  |  |  |  |
| 7 |  |  | X | X | X | X |
| 8 |  |  |  |  |  | X |
| 9 |  |  |  |  |  | X |
| 10 |  |  |  | X |  | X |
| 11 |  |  |  |  | X |  |
| 12 |  |  |  | X |  |  |
| 13 |  |  |  | X | X |  |
| 14 |  |  |  | X |  |  |
| 15 |  |  |  | X | X |  |
| 16 |  | X |  |  |  |  |
| 17 |  |  |  |  |  | X |
| 18 |  |  |  |  | X |  |
| X |  |  |  | X |  | X |

*** Next CHANGES ***

## 6.x	Solution #x: Temporary network reselection for localized service support

### 6.X.1	Introduction

This solution addresses the scenario of when the UE has subscribed to a localized service in a hosting network and a temporary network reselection procedure from a home/or other network to a hosting network is needed. It describes a temporary network reselection procedure that enables the UE to enjoy localized services and covers Key Issue #4 and #6.

This solution details the step H5 and H8 in solution #7.

### 6.X.2	Functional Description

The solution assumes that the UE is aware of which hosting network to use for the desired localized service. This solution facilitates a temporary change in the network priorities for network selection or a temporary change of SNPN access mode. Additionally, it enables the home network (i.e. HPLMN, subscribed SNPN) to authorize the UE to use this reselection procedure. There are criteria associated with the temporary reselection procedure (e.g., the start times and end times), so that the temporary reselection procedure is only applicable when the criteria are met, otherwise the existing network selection procedures defined in TS 23.122 [6] apply.

It would enable:

-	the UE to perform a network reselection to the hosting network from a network which was selected based on the existing network selection procedures as described in TS 23.122 [6] before the beginning of the localized service.

-	the UE to leave the hosting network and select a new network using existing network selection procedures as described in TS 23.122 [6] after the localized service is stopped.

-	the UE to switch the SNPN access mode.

### 6.X.3	Procedures

#### 6.X.3.1	Overview of Temporary Network Reselection (TNR) to enable localized services

Figure 6.X.3.1-1: Temporary Network Reselection (TNR) procedure to enable localized services

The following steps describe how to use Temporary Network Reselection (TNR) procedure to enable localized services as shown in Figure 6.X.3.1-1:

1.	The UE is already registered in a network, which was selected based on existing network selection procedures defined in TS 23.122 [6].

2.	The UE selects a localized service (e.g., the user selects a Localized service, the application requests a Localized service) and is aware of which hosting network to use for this localized service.

NOTE 1:	How the UE gets aware of which hosting network to use for the localized service is assumed to be described in other solutions.

3.	The UE and the home network (i.e. HPLMN or subscribed SNPN) decides to enable TNR procedure.

The home network can authorize its subscriber to use the TNR, e.g.:

i.	via pre-authorization for the UE based on UE configuration, or

ii.	via UE request as described in clause 6.X.3.2.

4.	The UE stores the current registered network, and activates TNR when the criteria associated with the TNR are met.

The TNR can be achieved, in one way, by reprioritizing (or reordering) the list of networks that are made available to the UE, with the hosting network as the top priority.

Another way is to avoid the automatic network selection to the higher priority networks, e.g., by avoiding triggering of periodic checks for higher prioritized networks when the UE is in a VPLMN (e.g. hosting network is a PNI-NPN which is different than the serving PLMN and has a lower priority than the serving PLMN).

The TNR can also trigger the UE to switch the SNPN access mode in case it is needed (e.g. hosting network is an SNPN while the UE is currently registered in a PLMN).

The criteria associated with the authorized TNR can include, e.g. start/stop time, location.

Editor's note:	It is FFS how simultaneous home network connectivity and home network service availability can be used as the criteria for TNR.

5.	The UE starts and stops using the localized service.

6.	The UE deactivates the TNR when the criteria are no longer met, and selects a new network based on the existing network selection procedures defined in TS 23.122 [6].

NOTE 2:	It is up to CT1 to determine whether and how the previous registered network stored in step 4 is used in the network selection after UE deactivates the TNR.

#### 6.X.3.2	Home network authorization via Steering of Roaming (SOR) procedure

Figure 6.X.3.2-1: Home network authorizes Temporary Network Reselection (TNR)

The temporary network reselection (TNR) can be requested by the UE and be authorized by the home network, reusing Steering of Roaming (SOR) procedure defined in TS 23.122 [6] with additions listed below.

Figure 6.X.3.2-1 shows PLMN as an example of the home network. The same principle applies if home network is SNPN.

0.	The UE is already registered via an AMF in a VPLMN or in the HPLMN. This implies that the AMF has already registered in UDM, has obtained subscription data for the UE and subscribed to subscription data updates including possible updates of SOR information.

1.	The UE requests an authorization of TNR procedure by including a TNR information within a NAS message, e.g. a Registration Request. The TNR information can be sent as a separate IE or in a new transparent container for the HPLMN UDM (i.e. all AMFs would simply forward the transparent container).

NOTE:	Whether and how the TNR information is protected by UE before sending to AMF is up to SA WG3.

The TNR information in the UE request includes, e.g.:

i.	Requested start and stop time for permission to use TNR procedure.

ii.	The selected localized service identity.

iii.	The selected hosting network identity.

2.	Based on the TNR information received from the UE, the AMF decides to update its registration in UDM to provide the UE requested TNR information to UDM.

3.	Based on the TNR information received from the AMF, the UDM decides to request the SOR-AF for authorizing the TNR requested by the UE (step 3a and 3b). The UDM does this even if the UE is registered in the HPLMN.

The SOR-AF updates the SOR information for the UE if needed according to SOR-AF decision (step 3c) and protects the SOR info via AUSF (step 3d). The SOR information provided by the SOR-AF includes the TNR authorization in the SOR-AF. This can be in the form of e.g.

i.	An indication that the UE is authorized for TNR. Based on this indication, UE can temporarily suspend the automatic network selection mechanism which is according to the available SOR list stored in the UE, or temporarily switch SNPN access mode.

ii.	An updated or new list of prioritized PLMNs or SNPNs including the hosting network selected by the UE with the highest priority.

iii.	Conditions for UE to activate TNR, e.g. authorized start/stop time, location, etc.

In order to avoid overload of the home network when UE leaves the hosting network(Key Issue #6), the UDM/SOR-AF can apply an offset to the authorized star/stop time in the authorized TNR sent to UE.

4.	The UDM provides the authorization decision to the UE via the AMF after protection via AUSF.

Steps 6-10 are as defined in TS 23.122 [6] Annex C.2.

11.	The UE applies the TNR as authorized by the SOR-AF when conditions are met.

If the SOR-AF is used and the SOR-AF keeps track of the conditions associated with the authorized TNR, then SOR-AF can issue an update of the SOR information to the UE as described by Figure C.3.1 and Figure C.6.1 in TS 23.122 [6].

### 6.X.4	Impacts on services, entities, and interfaces

UE:

-	Request temporary network reselection via NAS message

-	Temporarily apply temporary network reselection, using updated or new list of prioritized networks with hosting network as top priority

-	Temporarily avoid periodic check of higher prioritized networks

-	Temporarily switch SNPN access mode.

UDM/SOR-AF:

-	Authorize the UE request for temporary network reselection

-	Update the authorized temporary network selection with UE

-	Pre-authorize the UE to use temporary network reselection, based on UE configuration.

AMF:

-	Forward the UE request for temporary network reselection to UDM.

*** END CHANGES ***