SA WG2 Meeting #151E (e-meeting)	S2-2205158

May 16 – 20, 2022, Elbonia	(revision of S2-2204003r03)

Source:	LG Electronics

Title:	KI #6, New Sol: Sequential deregistration by hosting network

Document for:	Approval

Agenda Item:	9.4 Study on enhanced support of Non-Public Networks phase 2

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This paper proposes a new solution to Key Issue #6.

Discussion

Proposal

It is proposed to agree the following changes into TR 23.700-08 on FS_eNPN_Ph2.

* * * * Start of 1st Change * * * *

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | X |  |  |  |  |  |
| 2 |  | X |  |  |  |  |
| 3 |  | X |  |  |  |  |
| 4 |  | X |  |  |  |  |
| 5 |  | X |  |  |  |  |
| 6 |  | X |  |  |  |  |
| 7 |  |  | X | X | X | X |
| 8 |  |  |  |  |  | X |
| 9 |  |  |  |  |  | X |
| 10 |  |  |  | X |  | X |
| 11 |  |  |  |  | X |  |
| 12 |  |  |  | X |  |  |
| 13 |  |  |  | X | X |  |
| 14 |  |  |  | X |  |  |
| 15 |  |  |  | X | X |  |
| 16 |  | X |  |  |  |  |
| 17 |  |  |  |  |  | X |
| 18 |  |  |  |  | X |  |
| X |  |  |  |  |  | X |

* * * * Start of 2nd Change (All new text) * * * *

6.x	Solution #X: Sequential deregistration by hosting network

6.X.1	Introduction

The solution addresses the KI#6: Support for returning to the home network. The congestion of home network is unavoidable if large number of UEs return to the home network as soon as Localized service is terminated. This solution proposes to control the number of returning UE based on service level agreement between the home network and hosting network.

6.X.2	Functional Description

This solution assumes that

-	There is service level agreement between the home network and hosting network.

-	The hosting network configures the AMF with maximum number of returning UEs for the given units of time for each of home network, e.g. 10 UEs per second for PLMN A. The home network operator can determine the maximum number of returning UEs e.g. considering the network processing capability and notifies it to the hosting network via service level agreement.

-	The AMF of hosting network knows termination time of Localized services.

When the Localized service is terminated, the AMF of hosting network triggers deregistration procedure. The AMF selects UEs from different home networks considering maximum number of returning UEs configured in the AMF. The AMF triggers deregistration for the selected UEs in sequence and ensures that number of returning UEs for each home network does not exceed the configured maximum number of returning UEs for the given units of time.

NOTE 1:	If many UEs trigger deregistration without waiting for network initiated deregistration, the AMF cannot control the number of returning UEs. In order to give some time to the AMF to trigger deregistration in a sequence manner, service termination time provided to the UE can be later than the actual service termination time. The time gap can be determined considering the expected number of returning UEs and maximum number of returning UEs for the given units of time for the home network.

NOTE 2:	How the AMF enforces maximum number of returning UE is not specified by 3GPP specification and it is up to implementation.

6.X.3	Procedures

Figure 6.X.3-1: Overall procedure

1.	The AMF is configured with the maximum number of returning UEs for the given units of time for each of home network.

2.	The UE registers to the hosting network for localized service. The AMF stores the home network of the UE.

Editor's note:	How the AMF knows the UE's home network is FFS.

3.	After Localized service is terminated, the AMF triggers deregistration in a sequence manner so that the number of returning UEs does not exceed the configured maximum number of returning UEs. The AMF indicates that Localized service is terminated in the Deregistration Request message. The UE returns to the home network. The UE does not re-register to the hosting network again based on received indication from the AMF.

NOTE:	The AMF can also take into account UE-initiated deregistered UEs when the AMF enforces maximum number of returning UE.

6.X.4	Impacts on services, entities, and interfaces

UE:

-	If the AMF indicates that Localized service is terminated in the Deregistration Request message, the UE finishes deregistration procedure and returns to the home network.

AMF:

-	The AMF is configured with maximum number of returning UEs for the given units of time for each of home network. The AMF triggers deregistration in a sequence manner so that number of returning UEs does not exceed configured maximum number of returning UEs.

-	The AMF indicates that Localized service is terminated in the Deregistration Request message.

* * * * End of Changes * * * *