SA WG2 Meeting #151e	S2-2205196

# May 16th – 20th, 2022; Elbonia               	              (revision of S2-2204093R03)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	new solution (KI3) for supporting offload policies to match more granular sets of UE(s)

Document for: 	Approval

Agenda Item: 	9.11

Work Item / Release:	FS_EDGE_Ph2/ Rel-18

Abstract of the contribution: new solution (KI3) for supporting offload policies to match more granular sets of UE(s)

# Discussion

The FS_EDGE_Ph2 SID contains

WT5)	Investigate the potential need and solutions for supporting offload policies to match more granular sets of UE(s) without exposing operator-internal configurations to 3rd party AFs.

TS 23.503 defines that a PCF can use information on Subscriber category and on allowed services for the user to define user related policies. These policies may then be sent to AMF and /or to SMF (for PDU Session related policies) but do not contain information on Subscriber category and on allowed services for the user any more.

TS 23.503 (from R15 on) defines

Subscriber category: is a means to group the subscribers into different classes, e.g. gold user, silver user and bronze user.

The policy control subscription profile information provided by the UDR during the UE Policy Association Establishment procedure using Nudr service for Data Set "Policy Data" and Data Subset "UE context policy control data" is described in Table 6.2-1:

Table 6.2-1: UE context policy control subscription information

| Information name | Description | Category |
| --- | --- | --- |
| Subscriber categories | List of category identifiers associated with the subscriber | Optional |

Table 6.2-2: PDU Session policy control subscription information

| Information name | Description | Category |
| --- | --- | --- |
| Allowed services | List of subscriber's allowed service identifiers | Optional |
| Subscriber categories | List of category identifiers associated with the subscriber | Optional |

Thus, PCF may from R15 on read information on Subscriber category and on allowed services in policy control subscription stored in UDR to determine suitable policies for a UE.

Currently AF can influence traffic routing on a PDU Session (as defined in TS 23.501 § 5.6.7 and in 23.502 for Nnef_TrafficInfluence_Create/Update) by issuing rules (via a NEF API) that may target a (list of) single UE, a group of UE identified by a Group ID, or any UE, as defined in TS 29.522 Table 5.4.3.3.2-1: Definition of type TrafficInfluSub

NOTE 2:        One of individual UE identifier (i.e. "gpsi", "ipv4Addr" or "ipv6Addr"), External Group Identifier (i.e. "externalGroupId") or any UE indication "anyUeInd" shall be included.

It is not possible for an AF to request specific traffic routing (as defined for Nnef_TrafficInfluence_Create/Update in TS 23.502 § 5.2.6.7) for a category of users or for users having subscribed to specific services (basically for users having subscribed to specific tariff plans)

Use case for specific traffic routing would be: very local traffic offload is expensive and would apply only for users with specific tariff plans  (e.g. users having subscribed to a game boost  offer).

A subscriber buys a subscription package for a specific service that requires traffic offload (e.g. summer package to watch internet tv or house security package where pictures of the surveillance cameras at home is to be processed as locally as possible)  but the operator wants a single DNN for Internet access (possibly the UE supports a single PDU Session) (OR operator avoiding the burden of different (DNN, S-NSSAI) per service package);

For users having subscribed such a package it is needed to allow offload of the corresponding traffic at the closest PSA UPF (User Plane Function). The operator may want to only do insertion of a very local PSA UPF only for users who have bought such a package.

# 2 Proposal

It is proposed to update TR 23.700-48 as follows

## 6.0	Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 |  |  |  |  |
| Solution X: Policies referring to “Allowed services” and/or “Subscriber categories” |  |  | X |  |  |  |  |  |  |

NEXT CHANGE (2) All text is new

## 6.X	Solution #X: Policies referring to “Allowed services” and/or “Subscriber categories”

### 6.X.1	Description

This solution targets KI3

The solution aims at supporting AF request for specific traffic routing (as defined for Nnef_TrafficInfluence_Create / Update in TS 23.501 [2] § 5.6.7) targeting a category of users or users having subscribed to specific services (basically for users having subscribed to specific tariff plans)

NOTE: 	Very local traffic offload can be expensive (the more locally CPU is deployed, e.g. to support EAS, the more expensive this CPU is) and it may be useful to apply very local offload only for users with specific tariff plans (e.g. users having subscribed to a “game boost” offer). The goal is also to avoid the burden to define dedicated DNN and/or slices for these cases.

AF guidance is needed as it is the AF that knows the relative cost of deploying very local EAS capabilities.

The solution adds a new kind of targets (beyond a UE, a group of UE, Any UE) for AF requests for traffic routing Nnef_TrafficInfluence_Create/Update where this new kind of targets may correspond to one of “Allowed services” and/or “Subscriber categories” as defined in TS 23.503 Table 6.2-2: PDU Session policy control subscription information.

The AF requests is provided to the PCF as defined in TS 23.502 [3] clause 4.3.6.2. When the SMF invokes an SM Policy Association Establishment as defined in TS 23.502 [3] clause 4.16.4, the PCF looks up the UDR to check whether the corresponding user subscription maps to one of “Allowed services” and/or “Subscriber categories” of the AF policies received by the PCF (from the UDR as defined in TS 23.502 [3] clause 4.3.6.2 ).

Allowed services and Subscriber categories as defined in TS 23.503 Table 6.2-2 cannot be sent on roaming interfaces as they refer to subscription plans of a given operator; this is not an issue as Nnef_TrafficInfluence_Create/Update and NWDAF services are not defined on roaming interfaces;

An external AF (an AF from a third party with no specific agreement with the 5GC operator) cannot use “Allowed services” and/or “Subscriber categories” targets whose set of values are not defined by 3GPP and left for deployment definition (values defined by the 5GC operator). The NEF may have local policies allowing it to map the information included in an AF request from an external AF to identify the more granular sets of UE(s) into operator defined “Allowed services” and/or “Subscriber categories” values. The information included in an AF request to identify the more granular sets of UE(s) can include  AF identifier and/or MTC Provider Information (identifying the MTC/non-MTC Service Provider and/or MTC/non-MTC Application).

NOTE:	NOTE 5 in 23.502 clause 4.15.6.10 states “The MTC Provider Information can be used by any type of Service Providers (MTC or non-MTC) or Corporate or External Parties for, e.g. to distinguish their different customers.” So, the MTC Provider Information can also be used to carry information for non-MTC service.

NOTE: 	Different values of the allowed services parameter (in PCF subscription data stored in UDR) can based on local operator configuration to distinguish between subscription allowing to benefit from edge computing or even from deep edge computing (using more expensive compute resources at the RAN site) from subscription not allowing not to benefit from edge computing for some services. For example, the operator may associate some “game boost” tariff plan with values of the allowed services parameter in PCF subscription data (stored in UDR) allowing the users to have a better gaming experience as the gaming application they would use would be very local.

### 6.X.2	Procedures

TS 23.502 [3] clause 4.3.6.2 “Processing AF requests to influence traffic routing for Sessions not identified by an UE address” is modified as described below

Figure 4.3.6.2-1: Processing AF requests to influence traffic routing for Sessions not identified by an UE address

- In step 1, the target of the AF request may refer a set of UEs as described in 6.x.1

- In step 3 and 4, the data stored in UDR and the information sent to the PCF may refer to one of “Allowed services” and/or “Subscriber categories” as defined in TS 23.503 Table 6.2-2

TS 23.502 clause 4.3.6.2 “SM Policy Association Establishment” is modified as follows

Figure 4.16.4-1: SM Policy Association Establishment

-	in step 2, the PCF looks up the UDR PDU Session policy control subscription information as defined in TS 23.503 Table 6.2-2 to check whether the corresponding user subscription maps to one of “Allowed services” and/or “Subscriber categories” of the AF policies received by the PCF as defined in TS 23.502 clause 4.3.6.2. If there is a match the PCF uses this information of the AF policies for its policy rules determination

### 6.X.3	Impacts on Existing Nodes and Functionality

- AF and NEF need to support the new target (“Allowed services” and/or “Subscriber categories”) in Nnef_TrafficInfluence_Create / Update in TS 23.501 [2] § 5.6.7

- UDR for the storage of the new target type of “Allowed services” and/or “Subscriber categories” in data set “Application data” and Data Subset “AF traffic influence request information”

- PCF to look up the UDR PDU Session policy control subscription information defined in TS 23.503 Table 6.2-2 to check whether the user subscription corresponding to a SM Policy Association maps to one of “Allowed services” and/or “Subscriber categories” received from the “AF traffic influence request information” policies in UDR