3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2205282

Elbonia, May 16 – 20, 2022	(revision of S2-2204371r03)

Source:	Huawei, HiSilicon, Tencent, Tencent Cloud, vivo

Title:	KI#8, Sol#33: Update on CDRX traffic pattern

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM/ Rel-18

Abstract: The CDRX traffic pattern is updated as number of periodic bursts per second instead of the periodicity.

# 1. Introduction

In Sol#33, the UL/DL traffic pattern can be used to assist RAN for CDRX configuration and optimization. Whether the existing TSCAI can be used to deliver the UL/DL traffic pattern is still under discussion.

Generally, the periodic traffic pattern is because there may be fixed frame rate for media services, e.g. 30 fps, 60 fps and 90 fps. Then we can get the periodicity as 1/FPS per traffic burst. However, for common frame rates (i.e. 60), the periodicity is 16.67ms which is not so accurate as 1/60 s. When gNB gets this periodicity and takes it into account, the inaccuracy could be further increased. Therefore, it is not appropriate to use the periodicity to reflect the traffic patterns for UL/DL services like TSCAI.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60.

# * * * * First change * * * *

## 6.33	Solution #33: support of CDRX enhancement for power saving handling

### 6.33.1	Key Issue mapping

This solution is for Key Issue #8, which address the assistance to RAN for CDRX configuration and potential enhancement for power saving handling, including:

-  What information (if any) can be used to assist the CDRX enhancement?

-	Where does such information come from?

### 6.33.2	Description

In order to support the CDRX enhancement for power saving handling, the CN can send the UL/DL traffic patterns to assist RAN for CDRX setting and configuration.

- The UL/DL traffic patterns may include the number of periodic bursts per second or traffic periodicity, etc. which can be provisioned by the third-party AF or derived by UPF as the statistic results. The AF may update the traffic pattern to the network when it changes.

NOTE:	Traffic periodicity may exist for some XRM applications but not for all. It is assumed that the media application layer is aware whether traffic periodicity exist or not.

NOTE:	In case the UPF uses statistic approach to derive traffic periodicity and thus trigger CDRX related optimization, AF indication of DL/UL traffic pattern is not needed.

The traffic pattern may applies to one QoS Flow (in case all PDUs of the traffic are delivered within a QoS Flow) or to a QoS Flow group (in case the different kinds of PDU Sets of the traffic are bound to different QoS Flows).

2.	During PDU Session Establishment/Modification procedure, SMF receives the new or updated UL/DL traffic patterns from PCF or UPF and delivers the UL/DL traffic patterns to RAN, which can be taken into consideration when setting CDRX.

3.	Optionally, the UPF may detect the traffic bursts and add marks in the GTP-U layer of DL packets to assist RAN with identifying the end of the traffic bursts.

4.	With the awareness of the UL/DL traffic patterns, the RAN can configure and enable the CDRX accordingly.

NOTE: How to use the UL/DL traffic patterns for CDRX setting/configuration depends on RAN WG.

### 6.33.3	Procedures

Figure 6.33.3-1 shows how 5GS acquires and distributes the UL/DL traffic patterns to RAN for support of power saving handling.

Figure 6.33.3-1 UL/DL traffic pattern provisioning from AF

1a. Optionally, the AF may provision or update the UL/DL traffic patterns to PCF. The traffic patterns may include number of UL/DL periodic bursts per second or traffic periodicity, traffic description to localize the target application traffic, etc.

1b.	During PDU Session Establishment/Modification, PCF may send the UL/DL traffic patterns to SMF via the PCC rule.

1c.	Besides, the UPF may optionally derive the UL/DL traffic patterns based on the statistical analysis, e.g. via the identification mechanisms detailed in Sol#12, up to UPF implementations. The UL/DL traffic patterns may include the number of UL/DL periodic bursts per second or traffic periodicity, etc.

NOTE:	UPF derivation of UL/DL traffic pattern is considered as an alternative of 1a i.e. AF provisioning of UL/DL traffic patterns, which can be applied when AF doesn’t support provisioning of the traffic pattern info.

2.	SMF generates or update the UL/DL traffic patterns and sends that to RAN. Optionally, the burst detection instruction for the UL/DL traffic is also sent to RAN by SMF. The number of UL/DL periodic bursts per second or traffic periodicity may be used for CDRX cycle determination.

3.	Optionally, SMF may send the N4 rule to UPF. The N4 rule may include the burst marking rule, which can help the UPF to mark the DL traffic burst accordingly. If Step 3 is not performed, the N4 rule may also include the burst detection rule to assist UPF for burst detection.

4.	Complete the remaining steps of PDU Session Establishment/Modification procedure.

5.	After the target service started, the UPF may detect the periodic burst and mark the DL burst end in the GTP-U header based on the DL traffic rule. Then the DL packets with marks in the GTP-U header can help RAN to identify the end of the DL traffic bursts.

6.	Based on the UL/DL traffic pattern, the RAN can determine the end of the traffic bursts. For example, based on the end marks in the GTP-U header, the RAN can determine the current burst transmission is over and enable the CDRX, e.g. sending DRX Command MAC CE to trigger the start of CDRX.

7.	Based on the UL/DL traffic patterns, RAN enables the CDRX setting/configuration accordingly.

### 6.33.4	Impacts on services, entities and interfaces

AF:

-	Provisioning the new or updated UL/DL traffic patterns to NEF/PCF.

NEF：

-	Receive the new or updated UL/DL traffic patterns and send to PCF.

PCF:

-	Receive the new or updated traffic patterns from AF and send it to SMF.

SMF:

-	Receive the UL/DL traffic patterns from PCF or UPF.

-	Generate or update the UL/DL traffic patterns and send it to RAN.

-	Send N4 rule to assist UPF for burst detection.

UPF:

-	Statistically collect and report the UL/DL traffic patterns.

-	Report the statistical UL/DL traffic patterns to SMF.

-	Detect and mark the DL traffic burst as instructed by SMF.

RAN:

-	Receive the UL/DL traffic patterns.

-	Detect the end of DL traffic burst as instructed by SMF.

# * * * * End of changes * * * *