SA WG2 Meeting #151E (e-meeting)	S2-2205374

May 16 – 20, 2022, Elbonia	(revision of S2-2204007r01)

Source:	LG Electronics

Title:	Evaluation

Document for:	Approval

Agenda Item:	9.27 Study on Seamless UE context recovery

Work Item / Release:	FS_SUECR / Rel-18

Abstract of the contribution: This paper proposes evaluation for Key Issue #1.

Discussion

Proposal

It is proposed to agree the following changes into TR 23.700-61 on FS_SUECR.

* * * * Start of 1st Change * * * * (All New Text)

7	Overall Evaluation

|  | Summary | System impacts | AF interaction |
| --- | --- | --- | --- |
| Solution #1: Usage of MICO mode for UE unavailability period management | UE provides unavailability period via Registration UE requests MICO and release all PDU Sessions UE stores MM context AMF sets periodic registration timer considering unavailability period | UE provides unavailability period in Registration | No interaction |
| Solution #2: UE provided Unavailability Period | UE provides unavailability period via Registration Capability negotiation between UE and AMF AMF sets periodic registration timer considering unavailability period HLCom for DL data | UE provides unavailability period in Registration Capability negotiation | No interaction |
| Solution #3: Determination of unavailability period in 5GS | Capability negotiation between UE and AMF AMF may reject UE request to become unavailable and optionally provide a back off timer If AMF accepts unavailability period, UE triggers deregistration. Alternatively, MUSIM connection release and paging restriction can be used instead of performing deregistration. | UE provides unavailability period in Registration Capability negotiation AMF may reject unavailability period and provide back off timer UE needs to support unavailability back off timer | No interaction |
| Solution #4: Exposing UE provided unavailability period via Loss of Connectivity event subscription | UE provides unavailability period If Loss Of Connectivity event exists, the AMF sends notification. | UE provides unavailability period in Registration or Deregistration If Loss of Connectivity Event subscription is exist, the AMF reports notification when the UE provided unavailability period | Report unavailability period to the AF via Loss of Connectivity event subscription |
| Solution #5: Determination of unavailability period in 5GS for a specific UE | UE reports unavailability period to the AF via application layer signalling. AF provides unavailability period to the UDR / NWDAF For critical service, the AF negotiates BDT | AF provisions unavailability period provisioning via external parameter provisioning procedure Critical indication in BDT negotiation | Negotiate BDT time window considering unavailability period |
| Solution #6: Determination of unavailability period in 5GS for a specific UE | AF provides unavailability period to the UDM/UDR. UDM/UDR provides unavailability period to the AMF via subscription or AM Policy association UDM/PCF provides unavailability period to the UE via UPU / Policy delivery AMF / UE uses the unavailability period to determine whether deregistration can be executed | AF provisions unavailability period provisioning via external parameter provisioning procedure AMF, UE use the unavailability period whether to trigger deregistration | AF determines unavailability period |

* * * * End of Changes * * * *