SA WG2 Meeting #152e	S2-2205472

# August 17th – 26th, 2022; Elbonia               	  			  		(revision of S2-220)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | DRAFT | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Secondary DN authentication and authorization in EPS IWK case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_SDNAEPC |  |  |  |  |  | Date: |  |  | 2022-07-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Secondary authentication/authorization by a DN-AAA server during the establishment of a PDU Session has been defined since R15 in TS 23.501 clause 5.6.6 (see also discussion document S2-220xxxx). No such mechanism has been defined for EPC; this means that if an operator has negotiated with a 3rd party owning a DN that access to this DN (data connectivity to a DNN corresponding to this DN) is to be controlled by UE authentication / authorization run by a 3rd party DN AAA, UE(s) cannot establish a data connectivity to the DN when the UE is served by EPC. In the “Reply LS on Secondary AUTH for 5GS interworking with EPS”, S2-2101305, SA2 wrote: “EAP based secondary authorization/ authentication has only been defined for 5GS and is thus not applicable to EPS in existing releases. SA2 expects that in case EAP based secondary authorization/ authentication is to be introduced in EPS it would require a new work item in SA2.” A similar issue has been discussed as part of R17 for UTM based authentication and authorization of UAS. TS 23.256 supports UTM based authentication and authorization regardless of whether the UE (UAS) is served by 5GC or by EPC (via MME and SGW). This is defined in TS 23.256 clause 5.2.3.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support Secondary authentication/authorization by a DN-AAA server during the establishment of a PDN connection over EPC, reusing the principles of the solution defined in TS 23.256 clause 5.2.3.3: - A combo SMF+PGW-c is used to serve DNN(s) requiring Secondary authentication/authorization by a DN-AAA server. - For Secondary authentication/authorization by a DN-AAA server, the SMF+PGW-c runs the same procedures with PCF, UDM and DN-AAA and uses the same corresponding interfaces regardless of whether the UE is served by EPC or 5GC - Only the interface towards the UE is different (usage of 4G NAS instead of 5G NAS) between the EPC and 5GC cases. - The MME and SGW are not impacted by the procedure. Specific exchanges between the UE and the SMF+PGW-c for Secondary authentication/authorization by a DN-AAA server are carried via PCO. This includes the support of EAP exchanges between the UE and the DN AAA server - As it is not possible to exchange PCO between the UE and the PGW without first establishing the PDN connection, the PDN connection is established before Secondary authentication/authorization by a DN-AAA server has taken place. - When Secondary authentication/authorization by a DN-AAA server has successfully taken place, the SMF+PGW-c allows traffic exchange at the UPF and indicates to the UE that User plane traffic is now possible; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | if an operator has negotiated with a 3rd party owning a DN that access to this DN (data connectivity to a DNN corresponding to this DN) is to be controlled by UE authentication / authorization run by a 3rd party DN AAA, UE(s) cannot establish a data connectivity to the DN when the UE is served by EPC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.17.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE all text is new

#### 5.17.2.X	Secondary DN authentication and authorization in EPS Interworking case

Secondary authentication/authorization by a DN-AAA server during the establishment of a PDN connection over EPC, is supported based on following principles

- A combo SMF+PGW-c shall be used to serve DNN(s) requiring Secondary authentication/authorization by a DN-AAA server,

- For Secondary authentication/authorization by a DN-AAA server, the SMF+PGW-c runs the same procedures with PCF, UDM and DN-AAA and uses the same corresponding interfaces regardless of whether the UE is served by EPS or 5GS,

- Only the interface towards the UE is different (usage of 4G NAS instead of 5G NAS) between the EPS and 5GS cases.

This is further specified in TS 23.502 [2].

NEXT CHANGE (2)

NEXT CHANGE (3)

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES