SA WG2 Meeting #152e	S2-2205487

# August 17th – 26th, 2022; Elbonia          	  		(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Conclusion for KI 5

Document for: 	Approval

Agenda Item: 	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract of the contribution: Conclusion for KI 5

# 1	Discussion

Void.

# 2 Proposal

It is proposed to update TR 23700-74 as follows

First CHANGE

## 7.5	Key Issue #5: Allowing UE to simultaneously send data to different groups with different QoS policy

There are 3 solutions (sol#6, sol#7, sol#17) addressing key issue #5,

Solution 7 relates to a rare case: UE/application is not capable to replicate multiple copies of a packet but requires usage of multiple groups.

This solution 7 assumes the UE sends to 5GC only one copy of the traffic to be sent, and then UPF(s) in the 5GC are responsible for data duplication and further distribution to different groups each with a different QoS.

This nevertheless induce extensive and complex system impacts.

Quoting the solution

“UE1, which belongs to Group1 and Group2, sends PDU Session Request with Group1 and Group2 to the SMF. In this step, the AMF needs to select the SMF based on the information including Group1 and Group2. UE should include multiple DNNs/S-NSSAIs for a PDU Session”.

This means that the solution 7 requires to be able to associate a PDU Session with multiple groups and multiple DNNs / S-NSSAIs which breaks a fundamental 5GC rule that a PDU Session is associated with a single DNN and slice. This would have significant impacts throughout all the 5GC: in AMF, SMF, PCF, UDM, UDR, CHF.

Much simpler solutions are possible such as

- The UE establishes one PDU Session per 5G VN group (thus each PDU session relates to a single DNN  + S-NSSAI) and then the UE sends one copy of the data on each PDU session targeting possibly different Ethernet or IP multicast destinations.

- There is a unique group and a unique PDU session per UE; the sending UE sends one copy of a packet destinated to the group (using e.g. IP multicast) and the packet is replicated by routers with IGMP/MLD capabilities (could be UPF(s)). QoS rules associated with the destination UE(s) may associate different QoS for the delivery of different replicates sent to the PDU Session of the different UE(s)

For example, UE 0 sends a packet with an IP multicast address that is to be delivered to UE1 and UE2 with higher QoS and to be delivered to UE3 and UE4 with lower QoS. PCC rules can associate traffic towards that IP multicast address

-	with higher QoS for the PDU Sessions of UE1 and UE2

-	with lower QoS for the PDU Sessions of UE3 and UE4

NEXT CHANGE

## 8.5	Key Issue #5: Allowing UE to simultaneously send data to different groups with different QoS policy

Solution 6 and 17 are endorsed as output of the FS_GMEC study in TR 23700-74. Solution 7 will not be considered during normative specifications.

NEXT CHANGE