SA WG2 Meeting #152E	S2-2205569

# 17 – 26 August 2022, e-meeting

Source:	Lenovo

Title:	KI#2: Update of Solution 30 to address editor's notes

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract of the contribution:

# 1		Discussion

The update addresses the following editor's notes:

Editor's note:	It is FFS how such a detection could work? What are the criteria for the UPF to recognize "non matching traffic". Typically default PCC rules with wildcarded filters let let unexpected traffic pass. It is ffs how high the load for detection and reporting at the UPF would be and whether thus is acceptable.

The update clarifies how the NWDAF subscribes from the UPF to be notified if a UE routes traffic that is not part of the allowed traffic as described by the URSP rule configured in the UE. More specifically the following are propsoed:

-	The NWDAF subscribes to the UPF using a a new Nupf event based subscription to report "non-matching traffic" for a PDU session established to a specific S-NSSAI/DNN. The Nupf_EventExposure_Subscribe requests includes the following:

-	A new event ID: e.g. "Non-matching traffic"

-	Traffic Filters indicating the allowed traffic. This can be a 5-tuple or an application id

-	S-NSSAI or DNN of the PDU session to be inspected.

-	Target of Event Reporting (e.g. a single UE, a group of UE or any UE)

-	Notification Target Address (+ Notification Correlation ID) and

-	Event Reporting Information defined in Table 4.15.1-1 of 3GPP TS 23.502 [x]

Coordinatiopn with FS_UPEAS work is required to support the new Nupf_EventExposure SBI for reporting "non-matching" traffic.

Editor's note:	It is FFS whether supplying changed URSP rules to rogue or erroneuos UEs that do not handle URSP rules according to standards can improve the situation or whether other reactions are more appropriate.

-	An additional option is added proposing the SM-PCF to install policy rules to block any non-allowed traffic

Further details are described in the solution update.

# 2		Proposal

The following solution is proposed.

******************************** First change  *******************************

## 6.30	Solution #30: Leveraging NWDAF to determine UEs that enforce URSP rules incorrectly.

### 6.30.1	Description

This solution is related with KI#6 of the eNA study and also KI#2 of FS_eUEPO on how the 5GC can be aware that the UE enforces a URSP rules correctly.

The solution proposes to leverage the NWDAF to identify cases where a UE routes traffic to a PDU session (of a specific S-NSSAI/DNN) that is not according to the routing policies configured to the UE based on a provisioned URSP rule.

The consumer ( i.e.) PCF is aware of the correct application traffic that needs to be routed over a specific S-NSSAI/DNN by inspecting the URSP rule that needs to be determined if it is enforced correctly by UEs. The Traffic Descriptor of the URSP rule will identify the application traffic and the Route Selection Descriptor component will denote the correct S-NSSAI/DNN to route the application traffic.

The PCF determines to request analytics to identify if UEs enforce URSP rules correctly to a PDU session of a specific S-NSSAI/DNN based on local configuration.

The request to the NWDAF to provide analytics to detect UEs that do not enforce URSP correctly include the following:

-	An analytic ID denoting a request for URSP enforcement analytics.

-	Analytic Filters

-	The S-NSSAI/DNN of the PDU session established by a UE

-	The Allowed application traffic towards the PDU session according to the URSP rule. Allowed application traffic may include the Traffic Descriptor of the URSP rule (e.g. Application ID, FQDN) or may include allowed service data flows (i.e. the PCF may determine the allowed service data flows based on the Traffic Descriptor)

-	Area of interest

-	Any UE or list of UEs

The NWDAF subscribes to the UPF using a a new Nupf event based subscription to report "non-matching traffic" for a PDU session established to a specific S-NSSAI/DNN. The Nupf_EventExposure_Subscribe requests includes the following:

-	A new event ID: e.g. "Non-matching traffic"

-	Traffic Filters indicating the allowed traffic. This can be a 5-tuple or an application id

-	S-NSSAI or DNN of the PDU session to be inspected.

-	Target of Event Reporting (e.g. a single UE, a group of UE or any UE)

-	Notification Target Address (+ Notification Correlation ID) and

-	Event Reporting Information defined in Table 4.15.1-1 of 3GPP TS 23.502 [x]

The NWDAF determines the UPF that support the requested S-NSSAI/DNN either by interfacing with the NRF or the UDM (if the request includes a list of UE(s).

When the UPF detect non-matching traffic the UPF reports to the NWDAF, traffic filters indicating the traffic detected and a UE identifier (UE ID or UE IP address) of the UE that routed non-matching traffic to a PDU session established to a specific S-NSSAI/DNN.

NOTE X:	The UPF can use the usage reporting capabilities as described in clause 5.4.6 of 3GPP TS 29.244 [x] to determine and report non-matching traffic.

NOTE Y:	It is assumed that the UPF has received S-NSSAI/DNN information from the SMF when the SMF establishes an N4 session for a requested PDU session.

NOTE Z:	To control the signaling load of the UPF the UPF can be configured to report usage periodically to the NWDAF by re-using the event reporting information defined in Table 4.15.1-1 of 3GPP TS 23.502 [x].

Based on the information collected the NWDAF reports to the PCF a list of UEs that route traffic incorrectly and application traffic that is routed incorrectly over the S-NSSAI/DNN. The PCF uses the information to determine if such UEs require updated URSP rules (e.g. route specific application traffic over a default DNN) (if the consumer is the UE PCF) or block traffic by providing updated PCC rules for the established PDU session (if the consumer is the SM PCF).

### 6.30.2	Input Data

The following input data are required by the NWDAF to identify UEs that enforce URSP rules incorrectly.

Table 6.30.2-1: Input data for URSP enforcement analytics

| Information | Source | Description |
| --- | --- | --- |
| S-NSSAI | UPF /SMF | The S-NSSAI that the UE has established a PDU session |
| DNN | UPF /SMF | The DNN that the UE has established a PDU session |
| UE ID | UPF /SMF | The identity of the UE that has established a PDU session to a specific S-NSSAI/DNN |
| non-matching traffic | UPF | The traffic detected that is not matched against the allowed traffic over a specific S-NSSAI/DNN |

### 6.30.3	Output Analytics

The NWDAF only provides to the PCF statistics on UEs that do not enforce URSP correctly as shown in the table below. For this scenario the NWDAF does not provide any predictions.

Table 6.30.3-1: "URSP enforcement" statistics

| Information | Description |
| --- | --- |
| List of URSP enforcement Analytics (1..max) |  |
| > Non-matching traffic | The traffic detected that is not matched against the allowed traffic |
| >Applicable DNN | The DNN that the UE has established a PDU session |
| >Applicable S-NSSAI | The S-NSSAI that the UE has established a PDU session |
| > List of SUPIs | A list of UE that route the same non-matching traffic over a specific S-NSSAI/DNN |

### 6.30.4	Procedures

The procedure is shown in the figure below.

Figure 6.30.4-1: NWDAF providing analytics for URSP rules enforcement

The procedure is as follows:

1.	The consumer (i.e. PCF) needs to identify if a URSP rule is enforced correctly by UEs. The PCF is locally configured to determine which URSP rule needs to be checked if enforced correctly by UEs.

2.	The consumer request analytics for URSP enforcement by the NWDAF. The consumer includes a specific Analytic ID (that is associated to the URSP enforcement analytics) and includes as analytics filters the following information:

-	An analytic ID denoting a request for URSP enforcement analytics.

-	Analytic Filters:

-	The S-NSSAI/DNN of the PDU session established by a UE.

-	The Allowed application traffic towards the PDU session according to the URSP rule. Allowed application traffic may include the Traffic Descriptor of the URSP rule (e.g. Application ID, FQDN) or may include allowed service data flows (i.e. the consumer may determine the allowed service data flows based on the Traffic Descriptor).

-	Area of interest.

-	Any UE or list of UEs.

3.

If the request includes Αny UE:

-	The NWDAF subscribes from all SMFs supporting the S-NSSAI/DNN requested, to be notified when a UE establishes a PDU session to the S-NSSAI/DNN requested in step 2. The SMF provides the UE ID and UE IP address to the NWDAF.

If the request includes a list of UEs

-	The NWDAF finds the UPF serving the UE for the requested S-NSSAI/DNN by querying the UDM to identify SMF serving UE for the S-NSSAI/DNN and then querying the SMF to identify the UPF serving the UE.

-	Alternatively the NWDAF may query the UDM to find the UPF serving the UE for an S-NSSAI/DNN

4.	The NWDAF subscribes from the UPF to report non-matching traffic. The NWDAF subscribes to the UPF using a a new Nupf event based subscription to report "non-matching traffic" for a PDU session established to a specific S-NSSAI/DNN. The Nupf_EventExposure_Subscribe requests includes the following:

-	A new event ID: e.g. "Non-matching traffic"

-	Traffic Filters indicating the allowed traffic. This can be a 5-tuple or an application id

-	S-NSSAI or DNN of the PDU session to be inspected.

-	Target of Event Reporting (e.g. a single UE, a group of UE or any UE)

-	Notification Target Address (+ Notification Correlation ID) and

-	Event Reporting Information defined in Table 4.15.1-1 of 3GPP TS 23.502 [x]

5.	A UE determines to request a PDU session based on application traffic matching a URSP rule.

6.	The UE requests establishment of a PDU session to the SMF as per TS 23.502 [3].

7.	When the UPF receives traffic (either in downlink or uplink) for a UE the UPF reports non-matching traffic and includes the service data flows of the traffic detected.

8.	The NWDAF compiles a list of UEs where application traffic is sent to the wrong S-NSSAI/DNN.

9.	The NWDAF provides a list of UE IDs and a list of non-matching traffic that is wrongly sent to this S-NSSAI/DNN

10.	The PCF may construct a new URSP rule to route traffic detected via a different S-NSSAI/DNN, e.g. via a default DNN.

### 6.30.5	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

-	NWDAF supporting new analytic id to identify UEs that do not enforce URSP rule correctly.

-	UPF reporting to NWDAF application traffic that is not matched against allowed traffic to a specific S-NSSAI/DNN.

******************************** End of change *******************************