3GPP TSG-WG SA2 Meeting #152E	S2-2205623

E-meeting, 2022‑08‑17 -- 2022‑08‑26	(revision of S2-220xxxx)

Source:	Ericsson

Title:	KI #5, Sol #22: Update misleading terminology

Document for:	Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract: This contribution proposes updates to solution 22.

# Introduction

Clause 3.3.5 of the GSMA OPG.02 requirement document introduces the Edge Node Sharing scenario. The Edge Node sharing scenario is a possible deployment scenario where an operator may choose to partner with a 3rd party or another operator for the deployment of cloud resources. This would be a bilateral agreement between 2 (or more) parties and is not referred to as “federation” in GSMA OPG. The text in Solution 22 should refer to Edge Node Sharing scenario and not federation.

# Proposal

It’s proposed to update solution #22 in 23.700-48 as follows:

## ************** 1st change **************

## 6.0	Solution-Key issue matrix

The solutions in clause 6 can apply to one or more key issues described in clause 5 of this report. Table 6.0-1 describes the relationship between solutions and key issues.

Editor's note:	The table below will be updated with actual content when generating the TR with approved contributions. Page number is automatically updated to ease reference (ctrl-left click to reach the solution).

Table 6.0-1: Solution-Key issue matrix

| Solution |  | Key issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title | (page) | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 | KI#7 |
| 01: EAS discovery in Home Routed roaming scenario | 22 | X |  |  |  |  |  |  |
| 02: Session Breakout in Visited PLMN | 26 | X |  |  |  |  |  |  |
| 03: EAS (re)discovery procedure in roaming scenario | 29 | X |  |  |  |  |  |  |
| 04: Support EAS (re-)discovery in VPLMN via HR PDU Session | 32 | X |  |  |  |  |  |  |
| 05: Accessing V-EHE via HR PDU Session | 38 | X |  |  |  |  |  |  |
| 06: URSP solution to support roamers access to EHE in a VPLMN | 40 | X |  |  |  |  |  |  |
| 07: Using URSP Rules to Establish an LBO PDU Session | 42 | X |  |  |  |  |  |  |
| 08: V-ECS Discovery during Steering of Roaming | 43 | X |  |  |  |  |  |  |
| 09: PDU Session configuration from EASDF | 45 | X |  |  |  |  |  |  |
| 10: LBO PDU Session establishment using PLMN criteria in RSD | 47 | X |  |  |  |  |  |  |
| 11: Exposure of Network Congestion | 49 |  | X |  |  |  |  |  |
| 12: Efficient exposure of RAN information | 52 |  | X |  |  |  |  |  |
| 13: Fast and efficient network exposure improvements | 56 |  | X |  |  |  |  |  |
| 14: Group Management | 58 |  |  |  | X |  |  |  |
| 15: Selection of common DNAI | 62 |  |  |  | X |  |  |  |
| 16: Selecting the same EAS/DNAI for collection of UEs | 70 |  |  |  | X |  |  |  |
| 17: Application layer EAS selection for collections of UEs | 73 |  |  |  | X |  |  |  |
| 18: Discovery of the same EAS for collections of UEs | 75 |  |  |  | X |  |  |  |
| 19: Influencing UPF and EAS (re)location for collections of UEs | 79 |  |  |  | X |  |  |  |
| 20: Global EASDF | 81 |  |  |  |  | X |  |  |
| 21: EAS Deployment information differentiated by PLMN ID | 83 |  |  |  |  | X |  |  |
| 22: EAS discovery for Edge Node Sharing | 84 |  |  |  |  | X |  |  |
| 23: Improvements for EHE operated by separate party | 90 |  |  |  |  | X |  |  |
| 24: Reuse Option D after UL-CL insertion | 92 | X |  |  |  |  |  |  |
| 25: EAS discovery in VPLMN via V-EASDF for a HR PDU Session | 94 | X |  |  |  |  |  |  |
| 26: SM Policy for HR Session Breakout in VPLMN | 97 | X |  |  |  |  |  |  |
| 27: EAS discovery with dynamic setup of a LBO PDU Session | 100 | X |  |  |  |  |  |  |
| 28: Support edge computing in Roaming | 102 | X |  |  |  |  |  |  |
| 29: Use of Internal Group ID and constraints in EDI | 107 |  |  | X |  |  |  |  |
| 30: Policies referring to "Allowed services" and/or "Subscriber categories" | 108 |  |  | X |  |  |  |  |
| 31: Providing traffic offload policy for a set of UEs with service information | 111 |  |  | X |  |  |  |  |
| 32: Offload policy for finer granular set of UEs | 114 |  |  | X |  |  |  |  |
| 33: AF requests offload policy for sets of UEs | 115 |  |  | X |  |  |  |  |
| 34: Selecting the same EAS/DNAI for collection of UEs | 116 |  |  |  | X |  |  |  |
| 35: Providing dedicated (re)location information as traffic routing information | 119 |  |  |  | X |  |  |  |
| 36: Providing dedicated (re)location information as EAS Deployment information | 123 |  |  |  | X |  |  |  |
| 37: (Re)location of same EAS and coordination across UEs | 126 |  |  |  | X |  |  |  |
| 38: EAS Discovery for EHE shared with other PLMN | 128 |  |  |  |  | X |  |  |
| 39: Support EAS relocation of inter-PLMN | 129 | X |  |  |  | X |  |  |
| 40: EAS discovery for shared EHE | 133 |  |  |  |  | X |  |  |
| 41: Controlling non-3GPP access of EC traffic via URSP and ATSSS | 136 |  |  |  |  |  | X |  |
| 42: Network-guided EC traffic switching | 139 |  |  |  |  |  | X |  |
| 43: Network-based solution for keeping EC traffic on 3GPP Access | 141 |  |  |  |  |  | X |  |
| 44: EAS traffic switching avoidance | 144 |  |  |  |  |  | X |  |
| 45: Application selected PDU Session | 146 | X |  |  |  |  | X |  |
| 46: Avoid UE switching on-going EC traffic away from 3GPP access | 147 |  |  |  |  |  | X |  |
| 47: Avoiding Switch Away Based on an SMF Indication | 149 |  |  |  |  |  | X |  |
| 48: Avoiding Switch Away Based on an Indication in the URSP | 150 |  |  |  |  |  | X |  |
| 49: URSP based solution to avoid UE to switch away from Edge PDU Session | 151 |  |  |  |  |  | X |  |
| 50: Obtain and maintain mapping table between IP address/IP range with DNAI | 152 |  |  |  |  |  |  | X |
| 51: EDI holding the IP address to DNAI mapping | 154 |  |  |  |  |  |  | X |
| 52: AF obtaining target DNAI provided by NEF | 155 |  |  |  |  |  |  | X |

## ************** 2nd change **************

## 6.22	Solution 22 (KI#5): EAS discovery for Edge Node SharingOPs

### 6.22.1	Introduction

This solution addresses Key Issue #5 and allows the discovery of an Edge Application Server (EAS) in case of Edge Node Sharing. In this solution, it is assumed that the EAS is hosted by a different PLMN than the PLMN that is serving the UE.

### 6.22.2	Functional description

#### 6.22.2.0	Option 0: SMF configuration

This solution is based on the Rel-17 EAS based discovery procedures with the assumption that the SMFs of each PLMN Sharing Edge Node need to be configured with the DNAI values associated to the EAS' hosted by the other Operators. More in details:

-	the SMF needs to know the EAS deployment information of the EAS running on other PLMN's edge infrastructure e.g. IP address range(s)/FQDN(s).

For example, if the EAS is running in MNO 2's PLMN#2, when the EAS discovery is triggered the EASDF in MNO 1's PLMN#1 will receive the DNS response from the DNS server knowing the EAS' address and will forward the EAS' address to the SMF in PLMN#1.

-	At this point, after the SMF gets the IP address of EAS running in PLMN#2, in order to know that the EAS is instantiated in PLMN#2's edge infrastructure, it is required that the SMF knows the mapping between the PLMN ID of the PLMN hosting the EAS and the corresponding IP address received as a result of the DNS query so that the SMF can steer the user plane path towards the PLMN#2's domain. This may require an update of the SMF each time an EAS is added or removed.

Editor's note:	How to support the low latency requirement for edge computing when the traffic routing from MNO1's UPF to MNO2's DN is FFS.

#### 6.22.2.1	Option 1: Shared EASDF

This option is based on the concept of shared EAS Discovery Function (shared EASDF), which is a new network entity shared among multiple operators and used to support the discovery of EAS for shared Edge Nodes.

Figure 6.22.2.1-1: Architecture with deployed Shared EASDF

The Shared EASDF is a new network entity deployed in the core network and shared among the PLMNs of the MNOs that support Edge Node Sharing. The shared EASDF is hosted by one PLMN. The PLMN hosting the shared EASDF is the anchor PLMN. The communication between other PLMNs and the shared EASDF takes place via the serving EASDF (sEASDF) and the serving SMF (sSMF), that is via the EASDF and the SMF of the PLMN serving the UE. The sSMF handles the DNS context on the shared EASDF in a similar way as for the sEASDF, i.e. via the Neasdf_DNSContext_Create service.

Editor's note:	Whether the shared EASDF is feasible for network operator is FFS. Checking with GSMA may be needed in the future.

Editor's note:	How sSMF creates DNS context on shared EASDF is FFS.

The shared EASDF stores EAS deployment information such as EAS address hosted by other PLMNs. An operator needs to update its EAS information in the shared EASDF any time a change is applied (e.g. an EAS is added/modified/removed, or an MNO joins or leaves sharing of Edge Node).

#### 6.22.2.2	Option 2: Per-PLMN EASDFs

This solution option is based on the communication between the EASDFs of the different PLMNs of the MNOs that support sharing of Edge Nodes. The EASDF of a PLMN manages information for EAS' that are hosted by the Edge Data Network of that PLMN, and the EASDF of a PLMN can interact with other PLMNs' EASDFs to support Edge Node Sharing.

Architecture

Figure 6.22.2.2-1: Architecture with per-PLMN EASDF

In the figure above:

-	serving EASDF (sEASDF) and the serving SMF (sSMF) are, respectively, the EASDF and the SMF of the PLMN that is currently serving the UE;

-	partner EASDF (pEASDF) is the EASDF of the partner PLMN which hosts the Edge Application Server whose service is desired.

Editor's note:	How to support the low latency requirement for edge computing when the traffic routing from MNO1's UPF to MNO2's DN is FFS.

Editor's note:	How sSMF determines the proper pEASDF is FFS.

EAS Deployment Information extension

By using the Nnef_EASDeployment APIs, the AF provides to the CN the EAS Deployment Information (see table 6.2.3.4-1 of TS 23.548 [3]). Such information needs to be associated to the additional PLMN ID(s) and FQDN filter/DNS server address filter(s) to help the SMF identify, during EAS discovery, where the EAS is located. Table 6.22.2.2-1 shows the filtering information used to select the proper EAS and table 6.22.2.2-2 shows some examples of FQDN/DNS server address filter(s).

Table 6.22.2.2-1: NEW: Filtering table for EAS Deployment Information

| Parameters | Description |
| --- | --- |
| PLMN ID(s) – NEW | Used to identify in which PLMN the EAS is located [optional] |
| Filter(s) – NEW | Used to identify where the EAS is located [optional] |

Table 6.22.2.2-2: NEW: Examples of Filters for EAS Deployment Information

| DNS query content or DNS resolution | DNS server address or FQDN filter | Result |
| --- | --- | --- |
| MNO1.xrgaming.edge | MNO1.* | Matched, select EASDF of MNO1 |
| Battle.gamingX.edge.com | *.gamingX.edge.* | Matched, serving MNO knows in which MNO 'gamingX' is running, so select the MNO's EASDF |
| 11.123.45.22 | 11.123.* | Matched, selected the EASDF of MNO1, which is corresponding to 11.123.* |

EAS Discovery

The EAS discovery is based on the existing EAS discovery procedures described in TS 23.548 [3], with the following changes:

-	provisioning of EAS deployment information to 5GS:

-	For each of its EASs, the application service provider (via the AF) provides each PLMN Sharing Edge Nodes with the EAS deployment information with PLMN ID, DNAI, and (list of) FQDNs necessary to discover the EAS. Such information is stored in the UDR following the EAS Deployment Information Provision from AF via NEF procedure (clause 6.2.3.4.2 of TS 23.548 [3]).

-	The SMF retrieves the EAS deployment information from the UDR (clause 6.2.3.4.3 of TS 23.548 [3]) and provides the following pieces of information to the EASDF (clause 6.2.3.4.4 of TS 23.548 [3]):

a)	FQDN filter: the list of filters regarding FQDN in the DNS query from the UE;

b)	DNS server address filter: the list of filters regarding DNS server address in the DNS query from the UE;

c)	PLMN ID associated with the FQDN filter or DNS server address;

d)	Action: DNS query forwarding to the target pEASDF, reporting to the SMF.

-	EAS discovery:

-	At EAS discovery (clause 6.2.3.2.2 of TS 23.548 [3]), the sEASDF determines the target pEASDF associated with the PLMN ID, if the received DNS query (from the UE) meets the following conditions:

-	the FQDN in the DNS query matches with the configured FQDN filter associated with the DNAI/PLMN ID;

-	the DNS server address in the query matches with the configured DNS address filter with the DNAI/PLMN ID.

-	Based on the indication by the sSMF, the sEASDF forwards the query to target pEASDF.

-	Target pEASDF replies with the specific DNAI/PLMN ID result.

-	The result is reported to the sSMF.

### 6.22.3	Procedures

#### 6.22.3.0	Option 0: SMF configuration

This option re-uses the EAS discovery procedure defined in clause 6.2.3.2.2 of TS 23.548 [3].

#### 6.22.3.1	Option 1: Shared EASDF

Figure 6.22.3.1-1 describes a modified EAS discovery procedure based on the Rel-17 procedure described in TS 23.548 [3].

Figure 6.22.3.1-1: Modified EAS discovery (clause 6.2.3.2.2 of TS 23.548 [3]) for Option 1

The procedure is based on the EAS discovery procedure defined in clause 6.2.3.2.2 of TS 23.548 [3], with the following changes:

-	new step 9a: based on the DNS message report sent by the sEASDF in steps 8/9, the SMF selects the shared EASDF.

-	new step 9b/c: the sSMF creates the DNS context in the shared EASDF.

NOTE 1:	In case of IPv4, it is assumed that the SMF knows the UE's public IP address. This can be done based on interaction with the NAT server and/or on implementation specific ways.

-	steps 10/11: the sSMF instructs the sEASDF to forward the DNS query to the shared EASDF. The sSMF provides to the serving EASDF the FQDN filter (e.g. FQDN ranges) to be reported. The FQDN filter indicates that the shared EASDF needs to be contacted to resolve the DNS query.

-	new step 11a: the sEASDF forwards the DNS query to the shared EASDF.

Editor's note:	How the DNS query is matched with the DNS context for the UE is FFS.

-	new step 13a: the shared EASDF resolves the DNS query and sends the DNS reply to the sEASDF.

NOTE 2:	Alternatively to steps 11a/13a, the DNS query can be forwarded by the sEASDF to the shared EASDF via the SMF.

#### 6.22.3.2	Option 2: Per-PLMN EASDFs

Figure 6.22.3.2-1 describes the modified EAS discovery procedure based on the Rel-17 procedure described in TS 23.548 [3].

Figure 6.22.3.2-1: Modified EAS discovery (clause 6.2.3.2.2 of TS 23.548 [3]) for Option 2

The procedure is based on the EAS discovery procedure defined in clause 6.2.3.2.2 of TS 23.548 [3], with the following changes:

-	new step 9a: based on the DNS message report sent by the sEASDF in steps 8/9, the SMF selects the pEASDF.

-	steps 10/11: the sSMF instructs the sEASDF to forward the DNS query to the partner EASDF. The sSMF provides to the serving EASDF the FQDN filter (e.g. FQDN ranges) to be reported. The FQDN filter indicates that the partner EASDF needs to be contacted to resolve the DNS query.

-	new step 11a: the sEASDF forwards the DNS query to the pEASDF.

Editor's note:	How the DNS query is matched with the DNS context for the UE is FFS.

-	new step 13a: the pEASDF resolves the DNS query and sends the DNS reply to the sEASDF.

### 6.22.4	Impact on existing entities and interfaces

#### 6.22.4.0	Option 0: SMF configuration

SMF:

-	needs to know EAS deployment information (IP address range(s)/FQDN(s)) of each EAS running in the PLMNs of the operators supporting Edge Node Sharing;

-	needs to be configured with mapping between EAS deployment information and PLMN ID. If EAS' are added or removed, the configuration information needs to be updated.

#### 6.22.4.1	Option 2: Shared EASDF

EASDF:

-	requires capability to communicate with shared-EASDF forward DNS query to and receive DNS response.

Shared-EASDF:

-	needs to be configured with IP range/FQDN of list of all EAS' running in PLMNs of operators participating in the Edge Node Sharing.

SMF:

-	needs to be configured with shared-EASDF address.

#### 6.22.4.2	Option 3: Per-PLMN EASDFs

EASDF:

-	requires capability to communicate with EASDFs of another PLMN to forward DNS query to and receive DNS response.

SMF:

-	needs to be configured per-PLMN EASDF addresses.

AF:

-	provides CN with EAS Deployment Information with PLMN ID(s) and/or Filters.

**************  End of changes **************