3GPP TSG-WG SA2 Meeting #152E e-meeting  	S2-2205701

Elbonia, August 17 – 26, 2022

Title:	Reply LS on NSSRG restriction on pending NSSAI

Response to:	LS (C1-224073/S2-2205406) on NSSRG restriction on pending NSSAI from CT1

Release:	Rel-17

Work Item:	eNS_Ph2

Source:	SA2

To:	CT1

Cc:

Contact Person:

Name:	Haiyang Sun

Tel. Number:

E-mail Address:	sunhaiyang3@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 would like to thank CT1 for their LS on NSSRG restriction on pending NSSAI.

Specific to the CT1 questions in C1-224073/S2-2205406 SA2 would like to provide the following feedback:

Question: Should the UE take the S-NSSAIs in the Pending NSSAI into account when applying NSSRG restrictions in the Requested NSSAI?

Answer:  SA2’s position is as follows:

If UE does not take the S-NSSAIs in the Pending NSSAI into account when applying NSSRG restrictions in the Requested NSSAI, there are potential following cases:

- Case#1: The S-NSSAIs in the current Requested NSSAI and the S-NSSAIs in the Pending NSSAI do not share a common NSSRG.

  - Because the S-NSSAIs in the Pending NSSAI are the subset of Requested NSSAI for the last Registration, the AMF regards the S-NSSAIs in the current Requested NSSAI as the lasted S-NSSAIs UE attempts to request. When Pending NSSAI does not share a common NSSRG with current Requested NSSAI, the AMF can abort the on-going procedure of NSSAA for those S-NSSAIs or ignore the results of NSSAA procedure.

- Case#2: The S-NSSAIs in the current Requested NSSAI and the S-NSSAIs in the Pending NSSAI share a common NSSRG.

  - In this case, the AMF waits for the completion of NSSAA and updates the Allowed NSSAI accordingly.

If UE takes the S-NSSAIs in the Pending NSSAI into account, then UE shall only include in the Requested NSSAI S-NSSAIs that share a common NSSRG with the S-NSSAIs in the Pending NSSAI as specified in the TS 23.501.

Based on the above analysis, SA2 thinks the UE is not required to take the S-NSSAIs in the Pending NSSAI into account as no potential issues have been foreseen.

2. Actions:

To CT1 group.

ACTION: 	SA2 asks CT1 group to take the above feedback into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #153e   	Oct 10 – 14, 2022 				Elbonia

TSG-SA2 Meeting #154   	Nov 14 – 18, 2022 				Canada, CA